勘测系统服务器 技术说明书

版本 v0.1.3 — 含系统流程图 + 详细使用文档

2026年6月

# 目录

第一章 系统概述

第二章 系统流程与架构图

第三章 详细使用文档（连接→输入→使用→输出查看）

第四章 V4-Local 合规报告

第五章 模块详解

第六章 数据库改造

第七章 部署指南

第八章 API 接口

附录 完整源代码

# 第一章 系统概述

## 1.1 系统定位

勘测系统服务器是一款面向工业勘测场景的数据采集、处理与管理系统。采用 Python 开发，基于 DuckDB 数据库引擎进行数据分片存储，通过 EventBus 事件总线驱动数据处理流程。

系统严格遵循 V4-Local 工业级重构方案（3.docx）、结果输出接口增强版（4.docx）、可执行升级方案（5.docx）完整规范。版本 v0.1.3。

## 1.2 核心能力

多协议接入：TCP/UDP/Serial/MQTT

多格式解析：BESTPOS/NMEA0183/CM510/K803/JSON

A/B/C三组定位比对（E/N/U三轴距离+5cm阈值）

按地区分片存储（杭州HW/绍兴SX/总部ALL）+ 4表标准化

EventBus事件总线（topic/publish/subscribe/replay/pipeline）

PluginSDK插件化解析（BasePlugin + sandbox + version）

CLI测试引擎（11命令）+ GPS回放系统

Result Service结果裁决（1/0/-1 + HTTP API）

数据库4表：device_data/gps_data/result_data/event_log

熔断/重复检测/序列检查/统计API

## 1.3 技术栈

| 组件 | 选型 |
| --- | --- |
| 语言 | Python 3.12+ |
| 数据库 | DuckDB + SQLite + JSON持久化 |
| 事件总线 | EventBus（进程内） |
| 通信 | TCP/UDP/Serial/MQTT |
| 报文 | BESTPOS/NMEA0183/CM510/K803/JSON |
| 插件 | PluginSDK |
| 位置比对 | E/N/U三轴距离，阈值5cm |
| 结果裁决 | ResultEngine（三规则） |
| 存储结构 | device_data/gps_data/result_data/event_log |
| CLI | 11命令（main/replay/stream_view/stress_test等） |
| API | StatsAPI(8080) + ResultAPI(8081) |

# 第二章 系统流程与架构图

## 2.1 总体架构流程图

系统完整数据处理流程：

┌─────────────────────────────────────────────────────────┐

│                   勘测系统服务器                          │

│  版本 v0.1.3 — 工业级数据流测试引擎                       │

└─────────────────────────────────────────────────────────┘

┌──────────────┐

│  设备层       │  ← GPS设备/CM510/K803/模拟器

│  Device Layer │  ← TCP/UDP/Serial/MQTT 协议接入

└──────┬───────┘

↓

┌──────┴───────┐

│  接入层       │  signal_receiver/ (TCP/UDP/Serial/MQTT)

│  Receiver    │  pipeline.py（自动格式检测+BESTPOS/NMEA/JSON）

└──────┬───────┘

↓

┌──────┴───────┐

│  EventBus    │  eventbus/ → publish/subscribe/replay

│  事件总线     │  Topics: device.raw/gps.data/analysis.result 等

└──────┬───────┘

↓

┌──────┴───────┐

│  Processor   │  processor/ → 统一输入/输出格式

│  处理层       │  gps_processor/cm510_parser/k803_parser/rule_engine

└──────┬───────┘

↓

┌──────┴───────┐

│  Plugin SDK  │  plugins/ → 插件化解析+隔离执行

│  插件系统     │  bestpos/nmea/cm510/k803/gps_analyzer

└──────┬───────┘

↓

┌──────┴───────┐

│  Rule Engine │  processor/rule_engine.py → 三规则判定

│  规则引擎     │  坐标范围+精度阈值+错误标记

└──────┬───────┘

↓

┌──────┴───────┐

│  Result Eng. │  result_service/result_engine.py

│  结果裁决     │  输出: 1=正确 / 0=错误 / -1=未知

└──────┬───────┘

↓

┌──────┴───────┐

│  Storage     │  DuckDB 4表: device_data/gps_data/result_data/event_log

│  存储层       │  SQLite (备用) + JSON (ResultStore) + Parquet (导出)

└──────┬───────┘

↓

┌──────┴───────┐

│  CLI / API   │  cli/ (11命令) + ResultAPI(8081) + StatsAPI(8080)

│  用户接口     │  CLI主入口: python app.py

└──────────────┘

## 2.2 勘测业务流程图

一次完整勘测的流程：

勘测员到测量点 → 设备开机获取GPS定位

↓

测量点数据（A组第1次定位）

↓

测量点数据（B组第2次定位）

↓

测量点数据（C组第3次定位）

↓

数据通过通信模块发送至服务器

↓

服务器: compare_position()

计算 E/N/U 三轴距离差

↓

最大差值 < 5cm?

├── 是 → save_survey() 存入4表

│           → 自动识别地区（杭州/绍兴）

│           → 存入对应分片数据库

│           → BatchShardMerger 合并至总部

│           → ResultEngine 返回 1（正确）

│           → 可通过CLI/API查询结果

└── 否 → 返回 0（错误），要求重新勘测

## 2.3 数据流闭环图

Device → Gateway → EventBus → Processor → PluginSDK → RuleEngine → ResultEngine → ResultStore → API/CLI

↕            ↕            ↕           ↕           ↕            ↕            ↕

TCP/UDP    eventbus/   processor/   plugins/   rule_engine  result_eng.  result_store

Serial     Topics      parser       analyzer                result(1/0/-1)

MQTT       replay      pipeline     sandbox

数据存储流向:

save_survey()

├── gps_data ← A/B/C 三组定位数据（各1条）

├── result_data ← E/N/U比对结果（1条）

├── device_data ← 设备信息（1条）

└── event_log ← 操作日志（1条）

查询接口:

├── cli/result_query.py → 终端查询设备结果

├── ResultAPI (8081)    → HTTP GET /result/{device_id}

└── StatsAPI (8080)     → HTTP GET /api/survey/stats

# 第三章 详细使用文档

本章详细说明从连接到使用的完整流程。

## 3.1 环境准备

### 3.1.1 安装Python

需要 Python 3.12 或更高版本。

验证安装：

python --version

应输出: Python 3.12.x

### 3.1.2 安装依赖库

pip install duckdb paho-mqtt fastapi uvicorn websockets aiofiles python-docx

### 3.1.3 项目结构确认

确认以下目录存在：

服务器端/ — 服务器程序（进入此目录操作）

软件端/   — 开发板固件（用于烧录到K803等设备）

## 3.2 启动服务器

### 3.2.1 基本启动

cd 服务器端

python app.py

启动后显示主菜单：

1. 手动录入勘测数据（A/B/C 三组）

2. 模拟定位器自动录入

3. 查看分部服务器数据

4. 查看总部服务器数据

5. 查看总部汇总统计

6. 图表/文本数据导入

7. 网络健康检查区

8. 数据管理（删除/清除缓存）

9. 系统升级功能

A. EventBus 状态与回放

B. CLI 测试引擎

C. GPS 回放系统

D. 插件管理

E. 结果服务（Result Service）

0. 退出

### 3.2.2 启动服务和查看结果

其他独立服务：

Result API（结果查询）

python -c "from result_service.result_api import api_server; api_server.start()"

访问: http://localhost:8081/result/CM510_001

Stats API（统计查询）

python -c "from upgrade.stats_api import run_server; run_server(port=8080)"

访问: http://localhost:8080/api/survey/stats

## 3.3 CLI 命令使用

### 3.3.1 CLI 命令列表

| 命令 | 功能 | 示例 |
| --- | --- | --- |
| python -m cli.main | CLI帮助菜单 | python -m cli.main |
| python -m cli.run_test | 运行测试（发送模拟数据） | python -m cli.run_test --device gps --count 5 |
| python -m cli.test_runner | 测试运行器 | python -m cli.test_runner --device cm510 --count 10 |
| python -m cli.replay | GPS回放 | python -m cli.replay --lat 30.0 --lng 120.5 --count 3 |
| python -m cli.stream_view | 实时流查看 | python -m cli.stream_view --topic gps.data |
| python -m cli.show_stream | 实时流查看（别名） | python -m cli.show_stream |
| python -m cli.device_control | 设备控制 | python -m cli.device_control list |
| python -m cli.device_simulator | 设备模拟器 | python -m cli.device_simulator --type gps --count 5 |
| python -m cli.result_query | 结果查询 | python -m cli.result_query CM510_001 |
| python -m cli.stress_test | 压力测试 | python -m cli.stress_test --count 100 |

### 3.3.2 启动测试（入门示例）

最简单的测试方法：

python -m cli.run_test --device gps --count 3

输出示例：

测试运行: device=gps count=3

[1/3] 发布 -> gps.data

[2/3] 发布 -> gps.data

[3/3] 发布 -> gps.data

完成

### 3.3.3 查看实时流

实时查看 EventBus 中的 GPS 数据流：

python -m cli.stream_view --topic gps.data

输出示例（每秒刷新）：

[10:30:45.123] gps.data              src=replay:geo  id=8cf53608  size=85B

└─ lat=30.0; lng=120.5; alt=50.0; e=0.01; n=0.01; u=0.02

### 3.3.4 GPS回放

回放模拟坐标：

python -m cli.replay --lat 30.0 --lng 120.5 --count 5

从文件回放：

python -m cli.replay --file data.txt

### 3.3.5 查询结果

python -m cli.result_query CM510_001

输出：

OK CM510_001: result=1  reason=全部校验通过

python -m cli.result_query --all

### 3.3.6 压力测试

python -m cli.stress_test --count 500 --device gps

输出：

结果: 成功=500 失败=0 耗时=5.23s RPS=96

## 3.4 从软件端（开发板）接入

### 3.4.1 连接方式

软件端运行在 K803/EK0407 开发板上，通过以下方式与服务器通信：

| 通信方式 | 适用场景 | 配置 |
| --- | --- | --- |
| TCP直连 | 局域网内（开发板→PC） | config.py: TRANSPORT=tcp, SERVER_HOST=PC的IP |
| MQTT | 跨网络/公网 | config.py: TRANSPORT=mqtt, MQTT_BROKER=broker地址 |
| Serial串口 | USB直连开发板 | signal_receiver/serial_receiver.py |

### 4.4.2 K803开发板配置

编辑 软件端/config.py：

# 设备ID

DEVICE_ID = "K803_001"

# 服务器地址

SERVER_HOST = "192.168.1.100"  # 改为PC的IP

SERVER_PORT = 8888

# 通信方式

TRANSPORT = "tcp"        # tcp 或 mqtt

# MQTT配置（transport=mqtt时使用）

MQTT_BROKER = "broker.emqx.io"

MQTT_PORT = 1883

MQTT_TOPIC = "survey/data"

### 3.4.3 烧录并运行

将 软件端/ 目录文件复制到开发板，执行：

python main.py

开发板自动执行：GPS定位 → 读取 3 次（A/B/C）→ 发送到服务器

## 3.5 导入数据

### 3.5.1 从文件导入

通过 app.py 主菜单选项6，支持文件格式：

| 格式 | 说明 | 示例 |
| --- | --- | --- |
| CSV | 逗号分隔值 | a_lat,a_lng,a_alt,a_e,a_n,a_u,... |
| TXT | 文本（每行一条BESTPOS报文） | #BESTPOSA,COM1,0,60.0,... |
| JSON | JSON数组 | [{...batch_id, a_lat, a_lng, ...}] |

操作步骤：

1. 在主菜单输入 6

2. 输入文件路径

3. 选择目标地区（1=杭州 / 2=绍兴）

4. 系统自动导入、检测、存储

### 3.5.2 手动录入

主菜单选项1：手动输入 A/B/C 三组定位数据

输入格式：纬度 经度 高度(m) E精度 N精度 U精度

示例：30.25 120.16 50.0 0.02 0.02 0.03

## 3.6 输出查看

### 3.6.1 查看数据

| 入口 | 查看内容 |
| --- | --- |
| 主菜单3 | 分部服务器数据（按地区杭州/绍兴） |
| 主菜单4 | 总部服务器数据（汇总） |
| 主菜单5 | 总部汇总统计（total/correct/failed） |
| CLI result_query | 设备结果查询 |
| Result API | HTTP GET /result/{device_id} |
| Stats API | HTTP GET /api/survey/stats |

### 3.6.2 导出数据

导出为 CSV 或 JSON：

python app.py --export csv --db hangzhou

python app.py --export json --db shaoxing

导出为 Parquet 格式（高效列式存储）：

python -c "from main_db import MainDatabase; m=MainDatabase(); m.connect(); m.export_parquet(); m.disconnect()"

### 3.6.3 数据管理

主菜单选项8 提供：

1. 删除指定数据（按 batch_id）

2. 清空全部数据

3. 清除上传缓冲区和锁文件

## 3.7 7. 完整使用示例

以下是一个从头到尾的完整使用流程：

场景：勘测员在杭州测量一个点

步骤1: 启动服务器

cd 服务器端

python app.py

步骤2: 选择地区（输入1=杭州）

步骤3: 手动输入A组数据

A组> 30.25 120.16 50.0 0.02 0.02 0.03

步骤4: 手动输入B组数据

B组> 30.25001 120.16001 50.01 0.02 0.02 0.03

步骤5: 手动输入C组数据

C组> 30.25002 120.16002 50.02 0.02 0.02 0.03

步骤6: 查看比对结果

E/N/U 差值均在 5cm 以内 → 勘测成功！

批次ID: HW-1

步骤7: 查看已存储的数据

主菜单3 → 查看分部 → 杭州

步骤8: 查询结果

python -m cli.result_query survey_HW-1

结果: 1 (正确)

# 第四章 V4-Local 合规报告（3.docx 63项）

## 总体重构目标（6/6）

| 要求 | 状态 |
| --- | --- |
| 无Web依赖 | ✅ CLI主入口 |
| 可回放 | ✅ GPS回放+信号回放+文件回放 |
| 可测试 | ✅ 11 CLI命令 |
| 可插件扩展 | ✅ PluginSDK |
| 事件流驱动 | ✅ EventBus |
| 本地工业级信号处理 | ✅ 完整架构 |

## STEP1 删除Web（5/5）

| 要求 | 状态 |
| --- | --- |
| dashboard_v4.py | ✅ |
| dashboard_pages/ | ✅ |
| html/ | ✅ |
| start_web.* | ✅ |
| web_*.py | ✅ |

| 替代CLI | 状态 |
| --- | --- |
| cli/main.py | ✅ |
| cli/test_runner.py | ✅ |
| cli/stream_view.py | ✅ |
| cli/device_simulator.py | ✅ |

## STEP2 EventBus（全部通过）

| 要求 | 状态 |
| --- | --- |
| topic机制 | ✅ 7预定义topic |
| 发布/订阅 | ✅ |
| replay | ✅ |
| pipeline | ✅ |
| 标准事件 | ✅ id/timestamp/topic/device_id/data |
| 标准topic | ✅ device.raw/parsed/gps.data/signal.cm510/signal.k803/analysis.result/system.anomaly |

## STEP3 CLI+回放（全部通过）

| 要求 | 状态 |
| --- | --- |
| 启动测试 | ✅ |
| 模拟设备 | ✅ |
| 查看实时流 | ✅ |
| 触发回放 | ✅ |
| cli/replay.py | ✅ |
| cli/stress_test.py | ✅ |
| 轨迹回放/延迟模拟/信号重放 | ✅ |

## STEP4 插件系统（全部通过）

| 要求 | 状态 |
| --- | --- |
| 可插拔/隔离/标准化/版本 | ✅ |
| on_init/on_event/on_result | ✅ |
| sandbox/schema/目录结构 | ✅ |
| 6个插件 | ✅ bestpos/nmea/cm510/k803/gps_analyzer/gps_plugin |

# 第五章 模块详解

| 模块 | 路径 | 功能 |
| --- | --- | --- |
| 主入口 | app.py | CLI菜单（0-9 + A-E） |
| EventBus | eventbus/ | 事件总线（core/topic/stream） |
| Processor | processor/ | 数据处理（gps/cm510/k803/rule_engine） |
| 位置比对 | position_compare.py | E/N/U三轴距离，5cm阈值 |
| 数据库 | db_config/shard_db/main_db | 4表标准化（device/gps/result/event） |
| CLI | cli/ | 11命令测试引擎 |
| Replay | replay/ | GPS/信号/文件回放 |
| Plugin | plugins/ | 6解析器 + SDK标准 |
| Result | result_service/ | 结果裁决（engine/store/api） |
| Upgrade | upgrade/ | 熔断/重复检测/序列检查/stats_api |
| Receiver | signal_receiver/ | TCP/UDP/Serial/MQTT接收 |
| MQTT | mqtt/ | 桥接服务 |

详细功能说明参见完整版文档 v0.1.2（含全部82文件源代码附录）。

# 第六章 数据库改造（5.docx §9）

依据 5.docx §9，8张旧表 → 4张标准表：

| 新表 | 主键 | 来源 |
| --- | --- | --- |
| device_data | device_id | device_info + gps_position合并 |
| gps_data | id (序列自增) | position_data_rural/urban + gps_position |
| result_data | id (序列自增) | position_compare.py E/N/U比对输出 |
| event_log | id (序列自增) | log_event() 系统调用 |

自动迁移：旧表数据初始化时自动转换，完成后DROP旧表。

| 测试项 | 结果 |
| --- | --- |
| 4表创建 | ✅ |
| 旧表迁移 | ✅ |
| 三组保存 | ✅ GPS3+Result1+Device1+Log1 |
| compare_position | ✅ E/N/U 5cm阈值 |
| batch查询/删除 | ✅ |
| 总部合并 | ✅ |
| 向后兼容 | ✅ |

# 第七章 部署指南

## 7.1 快速启动

pip install duckdb paho-mqtt fastapi uvicorn websockets aiofiles python-docx

cd 服务器端

python app.py

## 7.2 CLI命令速查

python -m cli.run_test --device gps --count 5     # 测试

python -m cli.stream_view --topic gps.data         # 实时流

python -m cli.replay --lat 30.0 --lng 120.5        # 回放

python -m cli.result_query CM510_001               # 结果

python -m cli.stress_test --count 100               # 压力

## 7.3 API服务

# Result API (端口8081)

python -c "from result_service.result_api import api_server; api_server.start()"

# Stats API (端口8080)

python -c "from upgrade.stats_api import run_server; run_server(port=8080)"

# 第八章 API 接口

| 端点 | 方法 | 端口 | 说明 |
| --- | --- | --- | --- |
| /api/survey/stats | GET | 8080 | 系统统计 |
| /api/survey/health | GET | 8080 | 健康检查 |
| /api/survey/upload | POST | 8080 | BESTPOS上传 |
| /result/{device_id} | GET | 8081 | 查询设备结果(1/0/-1) |
| /result/all | GET | 8081 | 全部结果列表 |

# 附录 完整源代码

以下为服务器端全部Python源文件（收录82个文件）：

#### app.py

"""
位置勘测数据管理系统
====================
定位器读取经纬度+高度，获取 H/V/D 位置差分精度，A/B/C 三组不重复数据。
三组为一轮勘测，自动编号（如 HW-1），精度比较 -> 去重 -> 存入分部 -> 上传总部。
分部服务器：Home-HW（杭州）、Home-SX（绍兴）
总部服务器：Home-ALL
数据库引擎：DuckDB（应用层分片）
"""

import json
import re
import time
import random
import sys
import argparse
import logging
from logging_config import setup as setup_logging
setup_logging()
from typing import Dict, Tuple

from db_config import REGION_CONFIG, PRECISION_THRESHOLD
from shard_db import ShardDatabase
from main_db import MainDatabase
from position_compare import compare_position


# ============================================================
# 定位器模拟：读取位置值（经纬度 + 高度），H/V/D 精度
# ============================================================
def simulate_positioner() -> Dict:
    """模拟定位器在 1s 内读取位置数据。
    返回 {"lat": 纬度, "lng": 经度, "alt": 高度(m), "h": 水平精度(cm), "v": 垂直精度(cm), "d": 三维精度(cm)}
    """
    time.sleep(0.1)  # 模拟读数延迟
    return {
        "lat": round(random.uniform(29.0, 31.5), 6),   # 纬度
        "lng": round(random.uniform(118.0, 122.0), 6),  # 经度
        "alt": round(random.uniform(0, 500), 2),         # 高度(m)
        "h":   round(random.uniform(1.0, 8.0), 2),       # 水平精度(cm)
        "v":   round(random.uniform(1.0, 8.0), 2),       # 垂直精度(cm)
        "d":   round(random.uniform(1.0, 8.0), 2),       # 三维精度(cm)
    }


def input_group(label: str) -> Dict:
    """手动输入一组位置数据"""
    print(f"\n--- {label}组数据 ---")
    print("输入格式：纬度 经度 高度(m) H水平精度(cm) V垂直精度(cm) D三维精度(cm)")
    print("示例：30.25 120.16 50.0 3.2 2.8 4.1")
    while True:
        try:
            parts = input(f"{label}组> ").strip().split()
            if len(parts) != 6:
                print("错误：必须输入 6 个值（lat lng alt H V D）")
                continue
            vals = [float(x) for x in parts]
            return {
                "lat": vals[0], "lng": vals[1], "alt": vals[2],
                "h": vals[3], "v": vals[4], "d": vals[5],
            }
        except ValueError:
            print("错误：请输入有效数字")


# ============================================================
# 数据显示
# ============================================================
def print_group(label: str, g: Dict):
    print(f"  {label}: lat={g['lat']}, lng={g['lng']}, alt={g['alt']}m, "
          f"H={g['h']}cm, V={g['v']}cm, D={g['d']}cm")


def print_record(record: Dict):
    """打印一条勘测记录"""
    print(f"  批次ID: {record['batch_id']}")
    print(f"  地区: {record['region_code']}  第{record['round_number']}轮 类型: {record['survey_type']}")
    print_group("A", {"lat": record["a_lat"], "lng": record["a_lng"], "alt": record["a_alt"],
                       "h": record["a_e"], "v": record["a_n"], "d": record["a_u"]})
    print_group("B", {"lat": record["b_lat"], "lng": record["b_lng"], "alt": record["b_alt"],
                       "h": record["b_e"], "v": record["b_n"], "d": record["b_u"]})
    print_group("C", {"lat": record["c_lat"], "lng": record["c_lng"], "alt": record["c_alt"],
                       "h": record["c_e"], "v": record["c_n"], "d": record["c_u"]})
    print(f"  AB: H差{record['ab_h_diff']:.2f} V差{record['ab_v_diff']:.2f} D差{record['ab_d_diff']:.2f}")
    print(f"  AC: H差{record['ac_h_diff']:.2f} V差{record['ac_v_diff']:.2f} D差{record['ac_d_diff']:.2f}")
    print(f"  BC: H差{record['bc_h_diff']:.2f} V差{record['bc_v_diff']:.2f} D差{record['bc_d_diff']:.2f}")
    print(f"  位置正确: {'是' if record['is_correct'] else '否'}  时间: {time.strftime('%Y-%m-%d %H:%M:%S', time.localtime(int(record['survey_time']))) if record.get('survey_time') else 'N/A'}")


# ============================================================
# 主菜单
# ============================================================
def main():
    print("=" * 55)
    print("  勘测系统服务器 v0.1.1 — V4-Local 工业级测试引擎 (5.docx)")
    print("=" * 55)

    while True:
        print("\n请选择操作：")
        print("  1. 手动录入勘测数据（A/B/C 三组）")
        print("  2. 模拟定位器自动录入")
        print("  3. 查看分部服务器数据")
        print("  4. 查看总部服务器数据")
        print("  5. 查看总部汇总统计")
        print("  6. 图表/Excel 数据导入（JSON/Excel 接口）")
        print("  7. 网络健康检查区 (Check Zone)")
        print("  8. 数据管理 (删除/清除缓存)")
        print("  9. 系统升级功能 (重复检测/熔断/统计API)")
        print("  A. V4-Local EventBus 状态与回放")
        print("  B. V4-Local CLI 测试引擎")
        print("  C. V4-Local GPS 回放系统")
        print("  D. V4-Local 插件管理")
        print("  0. 退出")

        choice = input("\n请输入选择: ").strip()

        if choice == "0":
            print("退出系统")
            break

        elif choice == "1":
            _manual_input()

        elif choice == "2":
            _auto_simulate()

        elif choice == "3":
            _view_shard()

        elif choice == "4":
            _view_main()

        elif choice == "5":
            _view_summary()

        elif choice == "6":
            _chart_import()
        elif choice == "7":
            _show_check_zone()
        elif choice == "8":
            _data_management()

        elif choice == "8":
            _data_management()
        elif choice == "9":
            _upgrade_menu()
        elif choice == "A" or choice == "a":
            _eventbus_menu()
        elif choice == "B" or choice == "b":
            _cli_test_menu()

elif choice == "C" or choice == "c":
            _gps_replay_menu()
        elif choice == "D" or choice == "d":
            _plugin_menu()
        elif choice == "E" or choice == "e":
            _result_service_menu()
        else:
            print("无效选择，请重新输入")


# ============================================================
# 操作实现
# ============================================================
def _select_region() -> str:
    """选择地区"""
    print("\n可用地区：")
    keys = list(REGION_CONFIG.keys())
    for i, k in enumerate(keys, 1):
        cfg = REGION_CONFIG[k]
        print(f"  {i}. {cfg['name']} ({cfg['code']}) - 农村高度范围: "
              f"{cfg['rural_alt_min']}~{cfg['rural_alt_max']}m")
    while True:
        try:
            idx = int(input("请选择地区: ").strip()) - 1
            if 0 <= idx < len(keys):
                return keys[idx]
        except ValueError:
            pass
        print("无效选择")


def _do_survey(region_code: str, A: Dict, B: Dict, C: Dict) -> Tuple[bool, str]:
    """执行一次完整勘测流程"""
    cfg = REGION_CONFIG[region_code]
    print(f"\n地区: {cfg['name']} ({cfg['code']})")
    print_group("A", A)
    print_group("B", B)
    print_group("C", C)

    # 精度比较
    h_vals = [A["h"], B["h"], C["h"]]
    v_vals = [A["v"], B["v"], C["v"]]
    d_vals = [A["d"], B["d"], C["d"]]
    passed, details = compare_position(A, B, C, threshold=PRECISION_THRESHOLD)

    print(f"\n精度比较（阈值< {PRECISION_THRESHOLD}cm）：")
    print(f"  AB: H差{abs(A['h']-B['h']):.2f} V差{abs(A['v']-B['v']):.2f} D差{abs(A['d']-B['d']):.2f}")
    print(f"  AC: H差{abs(A['h']-C['h']):.2f} V差{abs(A['v']-C['v']):.2f} D差{abs(A['d']-C['d']):.2f}")
    print(f"  BC: H差{abs(B['h']-C['h']):.2f} V差{abs(B['v']-C['v']):.2f} D差{abs(B['d']-C['d']):.2f}")

    if not passed:
        print("\n[结果] 勘测失败：精度差超过阈值，数据未保存")
        return False, "勘测失败"

    # 存入分部（含去重、上传总部）
    with ShardDatabase(region_code) as db:
        ok, msg = db.insert_survey_data(A, B, C)

    print(f"\n[结果] {msg}")
    return ok, msg


def _manual_input():
    """手动录入三组数据"""
    region_code = _select_region()
    print("\n输入三组不重复的勘测数据：")
    A = input_group("A")
    B = input_group("B")
    C = input_group("C")
    _do_survey(region_code, A, B, C)


def _auto_simulate():
    """模拟定位器自动录入"""
    region_code = _select_region()
    print("\n模拟定位器读取（1s 内读取位置值）...")

    A = simulate_positioner()
    time.sleep(0.3)
    B = simulate_positioner()
    time.sleep(0.3)
    C = simulate_positioner()

    # 让三组数据在同一位置附近（模拟同一轮勘测），但精度值不同
    base_lat, base_lng, base_alt = A["lat"], A["lng"], A["alt"]
    B["lat"], B["lng"], B["alt"] = base_lat, base_lng, base_alt
    C["lat"], C["lng"], C["alt"] = base_lat, base_lng, base_alt

    _do_survey(region_code, A, B, C)


def _view_shard():
    """查看分部数据"""
    region_code = _select_region()
    cfg = REGION_CONFIG[region_code]

    with ShardDatabase(region_code) as db:
        summary = db.get_summary()
        print(f"\n{cfg['name']}分部 ({cfg['code']}) 汇总：")
        print(f"  农村数据: {summary['rural_count']} 条")
        print(f"  城市数据: {summary['urban_count']} 条")
        print(f"  合计: {summary['total']} 条")

        if summary["total"] > 0:
            show = input("\n是否查看详细数据（y/n）: ").strip().lower()
            if show == "y":
                data = db.get_all_gps(9999)
                for record in data:
                    print("\n" + "-" * 40)
                    print_record(record)


def _view_main_old():
    """查看总部数据"""
    with MainDatabase() as db:
        show_rural = input("查看类型 (r=农村 u=城市 a=全部): ").strip().lower()
        st = {"r": "rural", "u": "urban", "a": None}.get(show_rural, None)

        data = db.get_all_data(st)
        if not data:
            print("总部数据库中没有数据")
        else:
            print(f"\n总部共有 {len(data)} 条数据：")
            for record in data:
                print("\n" + "-" * 40)
                print_record(record)


def _view_summary():
    """查看总部汇总"""
    with MainDatabase() as db:
        summary = db.get_summary()

        print("\n" + "=" * 40)
        print("  总部 (Home-ALL) 数据汇总")
        print("=" * 40)
        print(f"  总记录数: {summary['total_records']}")
        print(f"  农村数据: {summary['rural_total']} 条")
        print(f"  城市数据: {summary['urban_total']} 条")
        print("\n  按地区分布：")
        for name, info in summary.get("by_region", {}).items():
            print(f"    {name} ({info['code']}): 农村 {info['rural']} + 城市 {info['urban']} = {info['total']} 条")


def detect_file_format(path):
    """Detect file format by reading first line (not extension)"""
    ext = __import__("os").path.splitext(path)[1].lower()
    if ext in (".xlsx", ".xls"):
        return "excel"
    # Try UTF-8 first, fallback to GBK

_enc = "utf-8-sig"
    _raw = None
    for _enc_try in ("utf-8-sig", "gbk"):
        try:
            _raw = open(path, "r", encoding=_enc_try).read(500)
            _enc = _enc_try
            break
        except UnicodeDecodeError:
            continue
    if _raw is None:
        return "csv"
    # Check for specific signatures anywhere in content
    if "#BESTPOSA" in _raw:
        return "bestposa"
    if "$GP" in _raw or "$GN" in _raw and 36 in [ord(c) for c in _raw[:5]]:
        return "nmea"
    # Check first non-comment line for JSON
    for _line in _raw.split(chr(10)):
        _s = _line.strip()
        if _s.startswith("[") or _s.startswith("{"):
            return "json"
        if _s:
            break
    if ext == ".json":
        return "json"
    return "csv"


def read_file_to_records(path):
    """Auto-detect file format, parse, return List[Dict] with A/B/C groups"""
    fmt = detect_file_format(path)
    if fmt == "excel":
        return _read_excel_survey_data(path)
    if fmt == "json":
        import json
        with open(path, "r", encoding="utf-8") as f:
            data = json.load(f)
        if isinstance(data, list) and all("A" in d for d in data):
            return data
        raise ValueError("JSON must be list of {{A:..., B:..., C:...}}")
    if fmt == "bestposa":
        return _read_bestpos_ascii(path)
    if fmt == "nmea":
        return _read_nmea_file(path)
    # CSV default
    return _read_csv_survey_data(path)


def _read_nmea_file(path):
    """Read NMEA 0183 file -> group GGA positions into A/B/C triples"""
    from plugins.nmea import NMEAParser
    fixes = []
    with open(path, "r", encoding="utf-8") as f:
        for line in f:
            line = line.strip()
            r = NMEAParser.parse(line)
            if r and "lat" in r:
                r.setdefault("e", 0.5)
                r.setdefault("n", 0.6)
                r.setdefault("u", 1.5)
                fixes.append(r)
    if len(fixes) < 3:
        raise ValueError(f"Need at least 3 GGA fixes, found {len(fixes)}")
    records = []
    for i in range(0, len(fixes) - 2, 3):
        records.append({"A": fixes[i], "B": fixes[i+1], "C": fixes[i+2]})
    return records


def _read_excel_survey_data(path: str) -> list:
    """从 Excel 文件读取勘测数据，返回 List[Dict] 供 import_from_chart 使用。

    Excel 列名（第一行表头，顺序不敏感）：
      A_lat, A_lng, A_alt, A_h, A_v, A_d
      B_lat, B_lng, B_alt, B_h, B_v, B_d
      C_lat, C_lng, C_alt, C_h, C_v, C_d
      可选: survey_type (rural/urban)
    """
    import openpyxl

    wb = openpyxl.load_workbook(path, read_only=True, data_only=True)
    ws = wb.active

    rows_iter = ws.iter_rows(values_only=True)

    # 第一行：表头 -> 列名到索引的映射
    try:
        headers = [str(c).strip().lower() if c is not None else "" for c in next(rows_iter)]
    except StopIteration:
        wb.close()
        raise ValueError("Excel 文件为空")

    col_map = {}
    for i, h in enumerate(headers):
        h_clean = h.strip().lower()
        if h_clean:
            col_map[h_clean] = i

    # 验证必需的 18 个字段
    required = ["a_lat", "a_lng", "a_alt", "a_e", "a_n", "a_u",
                 "b_lat", "b_lng", "b_alt", "b_e", "b_n", "b_u",
                 "c_lat", "c_lng", "c_alt", "c_e", "c_n", "c_u"]
    missing = [c for c in required if c not in col_map]
    if missing:
        wb.close()
        raise ValueError(f"Excel 缺少必要列: {', '.join(missing)}")

    records = []
    row_num = 1  # 已跳过表头
    for row in rows_iter:
        row_num += 1
        # 跳过全空行
        if all(cell is None or (isinstance(cell, str) and cell.strip() == "") for cell in row):
            continue

        try:
            def val(name):
                v = row[col_map[name]]
                if v is None:
                    raise ValueError(f"第{row_num}行 {name} 为空")
                return float(v)
            record = {
                "A": {"lat": val("a_lat"), "lng": val("a_lng"), "alt": val("a_alt"),
                      "e": val("a_e"), "n": val("a_n"), "u": val("a_u")},
                "B": {"lat": val("b_lat"), "lng": val("b_lng"), "alt": val("b_alt"),
                      "e": val("b_e"), "n": val("b_n"), "u": val("b_u")},
                "C": {"lat": val("c_lat"), "lng": val("c_lng"), "alt": val("c_alt"),
                      "e": val("c_e"), "n": val("c_n"), "u": val("c_u")},
            }
            if "survey_type" in col_map:
                st_raw = row[col_map["survey_type"]]
                if st_raw is not None:
                    st = str(st_raw).strip().lower()
                    if st in ("rural", "urban"):
                        record["survey_type"] = st
            records.append(record)
        except (TypeError, IndexError, ValueError) as e:
            wb.close()
            raise ValueError(f"第{row_num}行数据格式错误: {e}")
    wb.close()

    if not records:
        raise ValueError("Excel 文件中没有有效数据行")

    return records


def _read_bestpos_ascii(path: str) -> list:
    """Read #BESTPOSA ASCII file -> list of A/B/C grouped records (optimized with regex)"""
    from plugins.bestpos import BESTPOSASCIIParser

# Pre-compile regex patterns for faster parsing
    _BESTPOSA_LINE = re.compile(r"^#[A-Z]+,\S+,\d+,\S+,\S+,\S+,\S+,\S+,\S+,\S+,\S+,")
    _FLOAT_CHECK = re.compile(r"^-?\d+\.?\d*$")
    
    fixes = []
    _enc = "utf-8"
    try:
        open(path, "r", encoding=_enc).readline()
    except UnicodeDecodeError:
        _enc = "gbk"
    with open(path, "r", encoding=_enc) as f:
        for line in f:
            line = line.strip()
            if not line:
                continue
            if not _BESTPOSA_LINE.match(line):
                continue
            r = BESTPOSA_FAST_PARSE(line)
            if r:
                fixes.append(r)
    if not fixes:
        raise ValueError("No valid BESTPOS records found in file")
    # Group fixes by quality type
    # 按文件顺序分组（3条连续为1组），替代原按质量类型分组
    fixes.sort(key=lambda x: x.get("seq", 0))
    records = []
    for i in range(0, len(fixes) - 2, 3):
        records.append({"A": fixes[i], "B": fixes[i+1], "C": fixes[i+2]})
    return records


def BESTPOSA_FAST_PARSE(raw: str) -> dict or None:
    """Fast BESTPOSA line parser using pre-validated check"""
    if isinstance(raw, bytes):
        raw = raw.decode("utf-8", errors="ignore")
    raw = raw.strip()
    if not raw:
        return None
    
    # Quick validation: count commas
    _COMMA_COUNT = re.compile(r",")
    if len(_COMMA_COUNT.findall(raw)) < 23:
        return None
    
    parts = raw.split(",")
    if len(parts) < 24:
        return None
    
    # Fast numeric parsing with try/except
    try:
        lat = float(parts[11])
        lng = float(parts[12])
        alt = float(parts[13])
        std_lat = float(parts[16]) if parts[16] else 0.005
        std_lng = float(parts[17]) if parts[17] else 0.006
        std_hgt = float(parts[18]) if parts[18] else 0.015
        seq = float(parts[6]) if parts[6] else 0
    except (ValueError, IndexError):
        return None
    
    e_cm = round(std_lat * 100, 2)
    n_cm = round(std_lng * 100, 2)
    u_cm = round(std_hgt * 100, 2)
    
    pos_type = parts[10] if len(parts) > 10 else "UNKNOWN"
    quality = 4 if "INT" in pos_type else 2 if "FLOAT" in pos_type else 0
    
    return {
        "lat": round(lat, 6), "lng": round(lng, 6), "alt": round(alt, 2),
        "e": e_cm, "n": n_cm, "u": u_cm,
        "pos_type": pos_type, "quality": quality,
        "source": "BESTPOSA", "seq": seq
    }

def _read_csv_survey_data(path: str) -> list:
    """Read survey data from CSV/TXT file, return List[Dict] for import_from_chart.

    Headers (1st row, comma or tab separated):
      A_lat, A_lng, A_alt, A_h, A_v, A_d  (same for B/C groups)
      Optional: survey_type (rural/urban)
    """
    import csv

    required = ["a_lat", "a_lng", "a_alt", "a_e", "a_n", "a_u",
                 "b_lat", "b_lng", "b_alt", "b_e", "b_n", "b_u",
                 "c_lat", "c_lng", "c_alt", "c_e", "c_n", "c_u"]

    # Auto-detect delimiter
    with open(path, "r", encoding=_enc) as f:
        first_line = f.readline().strip()
    delimiter = "	" if "	" in first_line else ","

    records = []
    row_num = 0
    with open(path, "r", encoding="utf-8-sig") as f:
        reader = csv.DictReader(f, delimiter=delimiter)
        reader.fieldnames = [h.strip().lower() if h else "" for h in reader.fieldnames]

        missing = [c for c in required if c not in reader.fieldnames]
        if missing:
            raise ValueError("CSV missing columns: " + ", ".join(missing) +
                             ". Found: " + str([c for c in reader.fieldnames if c]))

        for row in reader:
            row_num += 1
            if not any(row.get(c, "").strip() for c in required):
                continue

            try:
                def get_val(name):
                    v = row.get(name, "").strip()
                    if v == "":
                        raise ValueError("Row " + str(row_num + 1) + " " + name + " is empty")
                    return float(v)

                record = {
                    "A": {"lat": get_val("a_lat"), "lng": get_val("a_lng"), "alt": get_val("a_alt"),
                          "e": get_val("a_e"), "n": get_val("a_n"), "u": get_val("a_u")},
                    "B": {"lat": get_val("b_lat"), "lng": get_val("b_lng"), "alt": get_val("b_alt"),
                          "e": get_val("b_e"), "n": get_val("b_n"), "u": get_val("b_u")},
                    "C": {"lat": get_val("c_lat"), "lng": get_val("c_lng"), "alt": get_val("c_alt"),
                          "e": get_val("c_e"), "n": get_val("c_n"), "u": get_val("c_u")},
                }

                st = row.get("survey_type", "").strip().lower()
                if st in ("rural", "urban"):
                    record["survey_type"] = st

                records.append(record)
            except (ValueError, TypeError) as e:
                raise ValueError("Row " + str(row_num + 1) + " data error: " + str(e))

    if not records:
        raise ValueError("No valid data rows in CSV file")

    return records

def _chart_import():
    """数据导入接口

    Supported formats:
    - JSON (.json) - file or paste
    - Excel (.xlsx / .xls)
    - CSV / TXT (.csv / .txt) - comma or tab separated
    """
    print("\n数据导入")
    print("支持格式:")
    print("  1) JSON 文件 (.json)")
    print("  2) Excel 文件 (.xlsx / .xls)")

print("  3) CSV / TXT 文件 (.csv / .txt)")
    print("  4) JSON 手动粘贴")

    path = input("\n输入文件路径: ").strip().strip('"').strip("'")

    try:
        if path.lower() == "json":
            raw = input("粘贴 JSON 数据: ").strip()
            data = json.loads(raw)
        elif path.lower().endswith((".xlsx", ".xls")):
            data = _read_excel_survey_data(path)
        elif path.lower().endswith((".csv", ".txt")):
            # First line detection: BESTPOSA or CSV
            with open(path, "r", encoding="utf-8") as _fh:
                _first = _fh.readline().strip()
            if _first.startswith("#BESTPOSA"):
                data = _read_bestpos_ascii(path)
            else:
                data = _read_csv_survey_data(path)
        else:
            with open(path, "r", encoding="utf-8") as f:
                raw = f.read()
            data = json.loads(raw)

        if not isinstance(data, list):
            print("错误: 数据必须是数组格式 [{...}, ...]")
            return

        region_code = _select_region()
        with ShardDatabase(region_code) as db:
            ok, msgs = db.import_from_chart(data, "rural")

        print("\n导入结果: " + str(ok) + "/" + str(len(data)) + " 条")
        for msg in msgs:
            print("  " + msg)

    except FileNotFoundError:
        print("文件不存在: " + path)
    except json.JSONDecodeError as e:
        print("JSON 解析错误: " + str(e))
    except ValueError as e:
        print("数据错误: " + str(e))
    except Exception as e:
        print("导入失败: " + type(e).__name__ + ": " + str(e))
def _startup_check():
    """Startup check: verify DB, config, and dependencies"""
    import os
    print("\n" + "=" * 55)
    print("  System Startup Check")
    print("=" * 55)
    checks = []

    # DB directory
    from db_config import DB_DIR
    checks.append(("DB directory", os.path.exists(DB_DIR)))

    # Region config
    from db_config import REGION_CONFIG
    checks.append(("Region config", len(REGION_CONFIG) > 0))

    # Main DB path
    from db_config import MAIN_DB_PATH
    checks.append(("Main DB path", isinstance(MAIN_DB_PATH, str)))

    # Import check
    try:
        import duckdb
        checks.append(("DuckDB", True))
    except ImportError:
        checks.append(("DuckDB", False))

    try:
        import openpyxl
        checks.append(("openpyxl", True))
    except ImportError:
        checks.append(("openpyxl", False))

    for name, ok in checks:
        status = "OK" if ok else "FAIL"
        print(f"  [{status}] {name}")

    if all(ok for _, ok in checks):
        print("\n  All checks passed.")
    else:
        print("\n  Some checks failed. Check dependencies.")

    return all(ok for _, ok in checks)


def _show_check_zone():
    """网络健康检查区"""
    from monitor.reporter import print_check_zone, get_recommendations
    from monitor.metrics_collector import MetricsCollector
    print("\n" + "=" * 55)
    print("  网络健康检查区")
    print("=" * 55)
    did = input("\n设备 ID (留空=全部): ").strip()
    print_check_zone(did if did else None)
    if did:
        m = MetricsCollector.get_instance().get_device_metrics(did)
        if m:
            print("\n建议:")
            for r in get_recommendations(m):
                print("  " + r)
    input("\n按 Enter 继续...")



def _data_management():
    """数据管理: 删除/清除缓存"""
    print("\n" + "=" * 55)
    print("  数据管理")
    print("=" * 55)
    print("  1. 删除指定数据 (按 batch_id)")
    print("  2. 清空全部数据 (分库和总库)")
    print("  3. 清除上传缓冲区和锁文件")
    print("  0. 返回")
    choice = input("\n请选择: ").strip()
    if choice == "0":
        return
    region_code = _select_region()
    with ShardDatabase(region_code) as db:
        if choice == "1":
            bid = input("输入 batch_id (例如 HW-7): ").strip()
            if bid:
                ok, msg = db.delete_survey_data(bid)
                print("\n  " + msg)
        elif choice == "2":
            print("\n  警告: 这将删除全部数据!")
            if input('输入 "yes" 确认: ').strip().lower() == "yes":
                ok, msg = db.delete_all_data(confirm=True)
                print("\n  " + msg)
            else:
                print("  已取消.")
        elif choice == "3":
            msg = db.clear_buffers()
            try:
                from cache.idempotency import IdempotencyManager
                IdempotencyManager().reset()
                msg += ", idempotency cache reset"
            except Exception:
                pass
            print("\n  " + msg)
    input("\n按 Enter 继续...")



def _upgrade_menu():
    """系统升级功能菜单：重复检测 / 精度校验 / 熔断 / 统计API"""
    print("\n" + "=" * 55)

print("  系统升级功能")
    print("=" * 55)

    # 延迟导入避免启动时加载
    from upgrade.duplicate_detector import get_default as get_dd
    from upgrade.sequence_checker import get_default as get_sc
    from upgrade.circuit_breaker import get_default as get_cb

    dd = get_dd()
    sc = get_sc()
    cb = get_cb()
    _api_server = None

    while True:
        print("\n请选择：")
        print("  1. 查看重复检测统计")
        print("  2. 查看序列完整性检测统计")
        print("  3. 查看熔断机制状态")
        print("  4. 手动恢复熔断设备")
        print("  5. 启动/重启 统计API (端口 8080)")
        print("  6. 停止统计API")
        print("  0. 返回主菜单")

        choice = input("\n请选择: ").strip()
        if choice == "0":
            return
        elif choice == "1":
            print("\n重复检测统计:")
            s = dd.stats()
            for k, v in s.items():
                print(f"  {k}: {v}")
        elif choice == "2":
            print("\n序列完整性检测统计:")
            s = sc.stats()
            for k, v in s.items():
                print(f"  {k}: {v}")
        elif choice == "3":
            print("\n熔断机制状态:")
            s = cb.stats()
            if not s:
                print("  无设备记录")
            for dev, info in s.items():
                status = "🔴 已隔离" if info.get("isolated") else "🟢 正常"
                print(f"  {dev}: {status}  样本={info['samples']}  错误率={info['error_rate']*100:.1f}%  "
                      f"剩余冷却={info.get('remaining_cooldown', 0)}s")
        elif choice == "4":
            did = input("\n输入设备ID (留空=全部恢复): ").strip()
            cb.reset(did or "")
            print(f"  已恢复: {did or '全部设备'}")
        elif choice == "5":
            try:
                from upgrade.stats_api import run_server
                if _api_server:
                    _api_server.shutdown()
                from upgrade import duplicate_detector, sequence_checker, circuit_breaker
                from upgrade.stats_api import register
                register("duplicate_detector", dd)
                register("sequence_checker", sc)
                register("circuit_breaker", cb)
                _api_server = run_server(port=8080)
                print("  统计API已启动: http://localhost:8080/api/survey/stats")
            except Exception as e:
                print(f"  启动失败: {e}")
        elif choice == "6":
            if _api_server:
                _api_server.shutdown()
                _api_server = None
                print("  统计API已停止")
            else:
                print("  统计API未运行")
        else:
            print("无效选择")
        input("\n按 Enter 继续...")


def _eventbus_menu():
    """V4-Local EventBus 状态与回放"""
    from upgrade.eventbus import bus, Topics
    print("\n" + "=" * 55)
    print("  V4-Local EventBus 状态与回放")
    print("=" * 55)
    while True:
        s = bus.stats()
        print(f"\n  总事件数: {s['total_events']}")
        print(f"  历史队列: {s['history_size']}/{s['max_history']}")
        print(f"  Pipeline: {'启用' if s['pipeline_active'] else '未启用'}")
        print(f"  活跃主题:")
        for t, c in s.get('topics', {}).items():
            if c > 0:
                print(f"    - {t} ({c} 个订阅者)")
        print("\n  操作: 1.刷新  2.回放历史  3.清空历史  0.返回")
        ch = input("选择: ").strip()
        if ch == "0": return
        elif ch == "1": continue
        elif ch == "2":
            events = bus.replay_all()
            print(f"\n  历史事件 ({len(events)} 条):")
            for ev in events[-20:]:
                print(f"    [{ev.topic}] {ev.id} src={ev.source} t={ev.timestamp:.1f}")
        elif ch == "3":
            bus.clear_history()
            print("  历史已清空")
        else: print("  无效选择")

def _cli_test_menu():
    """V4-Local CLI 测试引擎"""
    print("\n" + "=" * 55)
    print("  V4-Local CLI 测试引擎")
    print("=" * 55)
    print("\n  CLI 命令 (在终端中使用):")
    print("  python -m cli.run_test --device gps --count 5")
    print("  python -m cli.run_test --device cm510 --count 10")
    print("  python -m cli.stream_view --topic gps.data")
    print("  python -m cli.stream_view --all")
    print("  python -m cli.device_control list")
    print("  python -m cli.device_control start gps_sim --count 5")
    print()
    import subprocess, sys
    print("  输入 CLI 命令直接运行 (留空返回):")
    while True:
        cmd = input("  $ ").strip()
        if not cmd: return
        try:
            if cmd.startswith("python "):
                subprocess.run(cmd, shell=True, cwd=__import__('os').path.dirname(__file__))
            else:
                subprocess.run(f"python -m {cmd}", shell=True, cwd=__import__('os').path.dirname(__file__))
        except Exception as e:
            print(f"  错误: {e}")

def _gps_replay_menu():
    """V4-Local GPS 回放系统"""
    print("\n" + "=" * 55)
    print("  V4-Local GPS 回放系统")
    print("=" * 55)
    print("\n  功能: 通过 EventBus 回放 GPS 数据")
    print("\n  命令 (在终端中使用):")
    print("  python -c 'from replay.gps_replay import GPSReplayEngine; e=GPSReplayEngine();")
    print("  e.load_file(\"文件路径\"); e.run()'")
    print()
    import os
    from upgrade.eventbus import bus
    base = os.path.dirname(__file__)
    while True:
        print("\n  1. 回放模拟坐标 (绍兴)")
        print("  2. 回放模拟坐标 (杭州)")
        print("  3. 从文件回放")
        print("  0. 返回")
        ch = input("选择: ").strip()
        if ch == "0": return

elif ch == "1":
            from replay.gps_replay import GPSReplayEngine
            e = GPSReplayEngine(speed=5.0)
            cnt = e.run_geo(30.0, 120.5, 50.0, count=3)
            print(f"  回放完成: {cnt} 条坐标到绍兴")
        elif ch == "2":
            from replay.gps_replay import GPSReplayEngine
            e = GPSReplayEngine(speed=5.0)
            cnt = e.run_geo(30.27, 120.15, 50.0, count=3)
            print(f"  回放完成: {cnt} 条坐标到杭州")
        elif ch == "3":
            fp = input("文件路径: ").strip().strip('\"')
            if os.path.exists(fp):
                from replay.gps_replay import GPSReplayEngine
                e = GPSReplayEngine(speed=2.0)
                loaded = e.load_file(fp)
                cnt = e.run(source="replay:file")
                print(f"  加载 {loaded} 条, 回放 {cnt} 条")
            else:
                print(f"  文件不存在: {fp}")

def _plugin_menu():
    """V4-Local 插件管理"""
    from upgrade.plugin_sdk import manager
    print("\n" + "=" * 55)
    print("  V4-Local 插件管理")
    print("=" * 55)
    while True:
        plugins = manager.list()
        s = manager.stats()
        print(f"\n  已注册插件: {s['count']}")
        print(f"  沙箱模式: {'启用' if s['sandbox'] else '禁用'}")
        print(f"  超时: {s['timeout']}s")
        for p in plugins:
            print(f"    - {p['name']} v{p['version']}")
        print("\n  操作: 1.刷新  2.沙箱开/关  3.测试处理  0.返回")
        ch = input("选择: ").strip()
        if ch == "0": return
        elif ch == "1": continue
        elif ch == "2":
            manager.use_sandbox = not manager.use_sandbox
            print(f"  沙箱模式已切换: {'启用' if manager.use_sandbox else '禁用'}")
        elif ch == "3":
            result = manager.process({"topic": "test", "data": {"msg": "hello"}})
            for name, r in result.items():
                print(f"    {name}: {r.get('status','?')} {r.get('processing_ms','?')}ms")
        else: print("  无效选择")


def _result_service_menu():
    """V4-Local 结果服务 — 工业测试裁决"""
    print("\n" + "=" * 55)
    print("  V4-Local 结果服务 (Result Service)")
    print("=" * 55)
    print("\n  功能: 计算设备勘察结果 (1=正确 / 0=错误 / -1=未知)")
    print("  依据: 4.docx 结果输出接口增强版")
    print()
    from result_service import ResultRecord, ResultEngine, ResultStore, ResultAPI
    from result_service.result_store import store
    from result_service.result_api import api_server
    engine = ResultEngine(store)

    while True:
        total = store.count()
        print(f"\n  已记录设备: {total} 个")
        print("  1. 查询设备结果")
        print("  2. 测试计算设备结果")
        print("  3. 查看全部结果")
        print("  4. 启动 Result API (端口 8081)")
        print("  5. 停止 Result API")
        print("  6. 清空结果")
        print("  0. 返回")
        ch = input("\n选择: ").strip()
        if ch == "0":
            return
        elif ch == "1":
            did = input("设备ID: ").strip()
            r = store.get_or_default(did)
            emoji = {1: "\u2705", 0: "\u274c", -1: "\u2753"}
            print(f"  {emoji.get(r.get('result'),'?')} {did}: result={r.get('result')}  reason={r.get('reason')}")
        elif ch == "2":
            did = input("设备ID (留空=auto): ").strip() or "CM510_TEST"
            import time, random
            data = {"lat": 30.0 + random.random()/100, "lng": 120.5 + random.random()/100,
                    "alt": 50.0, "e": 0.01, "n": 0.01, "u": 0.02}
            rec = engine.compute(did, data)
            emoji = {1: "\u2705", 0: "\u274c", -1: "\u2753"}
            print(f"  {emoji.get(rec.result,'?')} {did}: result={rec.result}  reason={rec.reason}")
        elif ch == "3":
            all_r = store.get_all()
            emoji = {1: "\u2705", 0: "\u274c", -1: "\u2753"}
            for r in all_r[-20:]:
                print(f"  {emoji.get(r.get('result'),'?')} {r.get('device_id','?')}: {r.get('result')}  {r.get('reason','')}")
            if not all_r:
                print("  (无记录)")
        elif ch == "4":
            import threading
            t = threading.Thread(target=api_server.start, daemon=True)
            t.start()
            print("  Result API 已启动: http://localhost:8081/result/")
        elif ch == "5":
            api_server.stop()
            print("  Result API 已停止")
        elif ch == "6":
            store.clear()
            print("  结果已清空")


def cli_main():
    """Entry point with CLI argument support"""
    parser = argparse.ArgumentParser(description="Drone Survey Data Management")
    parser.add_argument("--import", dest="import_file", help="Import data file (JSON/XLSX/CSV/TXT)")
    parser.add_argument("--export", choices=["csv", "json"], help="Export data format")
    parser.add_argument("--db", default="hangzhou", choices=["hangzhou", "shaoxing"], help="Database region")
    parser.add_argument("--summary", action="store_true", help="Show main DB summary")
    parser.add_argument("--check", action="store_true", help="Run system startup check")
    parser.add_argument("--verbose", action="store_true", help="Verbose output")

    # Parse known args first, ignore unknown (for interactive mode)
    args, _ = parser.parse_known_args()

    # Non-interactive mode
    if args.check:
        _startup_check()
        return
    if args.summary:
        from main_db import MainDatabase
        with MainDatabase() as m:
            s = m.get_summary()
            print(f"\nTotal: {s['total_records']} records")
            print(f"Rural: {s['rural_total']}, Urban: {s['urban_total']}")
            for name, info in s.get("by_region", {}).items():
                print(f"  {name}: {info['total']}")
        return
    if args.import_file:
        _run_cli_import(args.import_file, args.db)
        return
    if args.export:
        _run_cli_export(args.export, args.db)
        return

    # Interactive mode (default)
    _startup_check()
    main()


def _run_cli_import(filepath, region):
    """Non-interactive import"""
    from shard_db import ShardDatabase
    if filepath.lower().endswith((".xlsx", ".xls")):

from app import _read_excel_survey_data as reader
    elif filepath.lower().endswith((".csv", ".txt")):
        from app import _read_csv_survey_data as reader
    else:
        print(f"Unknown format: {filepath}")
        return
    try:
        data = reader(filepath)
        with ShardDatabase(region) as db:
            ok, msgs = db.import_from_chart(data, "rural")
        print(f"Imported {ok}/{len(data)} records")
        for m in msgs:
            print(f"  {m}")
    except Exception as e:
        print(f"Import failed: {e}")


def _run_cli_export(fmt, region):
    """Non-interactive export"""
    from shard_db import ShardDatabase
    import json
    with ShardDatabase(region) as db:
        data = db.get_all_gps(9999)
    filename = f"export_{region}_{int(time.time())}.{fmt}"
    if fmt == "json":
        with open(filename, "w", encoding="utf-8") as f:
            json.dump(data, f, ensure_ascii=False, indent=2)
    else:
        import csv
        if not data:
            print("No data to export")
            return
        with open(filename, "w", encoding="utf-8-sig", newline="") as f:
            w = csv.DictWriter(f, fieldnames=data[0].keys())
            w.writeheader()
            w.writerows(data)
    print(f"Exported {len(data)} records to {filename}")


if __name__ == "__main__":
    cli_main()

def _view_main():
    """查看总部服务器数据 (新4表)"""
    from main_db import MainDatabase
    with MainDatabase() as db:
        summary = db.get_summary()
        print("\n  总部数据库统计:")
        for k, v in summary.items():
            print(f"    {k}: {v}")
        print("\n  最近GPS记录:")
        gps = db.get_all_gps(20)
        for r in gps:
            print(f"    {r.get('batch_id','?'):<20} {r.get('region_code','?'):<4} "
                  f"grp={r.get('group_label','?'):<2} "
                  f"lat={r.get('lat',0):.4f} lng={r.get('lng',0):.4f} "
                  f"E={r.get('e',0):.3f} N={r.get('n',0):.3f} U={r.get('u',0):.3f}")
        results = db.get_all_results(10)
        if results:
            print("\n  最近结果:")
            for r in results:
                em = {1:"OK",0:"FAIL",-1:"?" }
                print(f"    {r.get('batch_id','?'):<20} result={em.get(r.get('result'),'?')} "
                      f"reason={r.get('reason','')}")

#### db_base.py

﻿"""数据库基类：共享方法 + 上下文管理器"""

from typing import Dict, List


class DatabaseBase:
    """数据库基类，提供 _row_to_dict、_get_columns 和上下文管理器支持"""

    def __init__(self):
        self.conn = None

    def connect(self):
        """子类重写：建立连接"""
        raise NotImplementedError

    def disconnect(self):
        """子类重写：关闭连接"""
        raise NotImplementedError

    def __enter__(self):
        self.connect()
        return self

    def __exit__(self, exc_type, exc_val, exc_tb):
        self.disconnect()

    def _row_to_dict(self, row, columns) -> Dict:
        return dict(zip(columns, row))

    def _get_columns(self, table: str) -> List[str]:
        if not self.conn:
            return []
        return [desc[0] for desc in
                self.conn.execute(f"SELECT * FROM {table} LIMIT 0").description]

#### db_config.py

# -*- coding: utf-8 -*-
"""
5.docx §9 数据库改造：统一为 4 张标准表
  device_data — 设备注册信息
  gps_data    — GPS 定位数据
  result_data — 结果裁决记录
  event_log   — 事件日志
"""
import os, math

BASE_DIR = os.path.dirname(os.path.abspath(__file__))
DB_DIR = os.path.join(BASE_DIR, "databases")
os.makedirs(DB_DIR, exist_ok=True)

REGION_CONFIG = {
    "hangzhou": {
        "code": "HW", "name": "杭州",
        "db_path": os.path.join(DB_DIR, "Home-HW.duckdb"),
        "rural_alt_min": 0.0, "rural_alt_max": 200.0,
        "boundary": {"points": [(30.0,119.5),(30.6,119.5),(30.6,120.8),(30.0,120.8)]},
    },
    "shaoxing": {
        "code": "SX", "name": "绍兴",
        "db_path": os.path.join(DB_DIR, "Home-SX.duckdb"),
        "rural_alt_min": 0.0, "rural_alt_max": 250.0,
        "boundary": {"points": [(29.5,120.2),(30.1,120.2),(30.1,121.2),(29.5,121.2)]},
    },
}

MAIN_DB_PATH = os.path.join(DB_DIR, "Home-ALL.duckdb")
PRECISION_THRESHOLD = 0.05  # 5cm
APPENDER_BATCH_SIZE = 100
PARQUET_TEMP_DIR = os.path.join(DB_DIR, 'parquet_exports')
PARTITION_BY = 'region_code'
os.makedirs(PARQUET_TEMP_DIR, exist_ok=True)

from utils.geo import build_bbox
for _cfg in REGION_CONFIG.values():
    if "boundary" in _cfg and "points" in _cfg["boundary"]:
        _cfg["boundary"]["_bbox"] = build_bbox(_cfg["boundary"]["points"])

# ── 表名 ──
TABLE_DEVICE_DATA = "device_data"
TABLE_GPS_DATA = "gps_data"
TABLE_RESULT_DATA = "result_data"
TABLE_EVENT_LOG = "event_log"

# ── 4 张标准表定义 (5.docx §9) ──

CREATE_DEVICE_DATA = f"""
CREATE TABLE IF NOT EXISTS {TABLE_DEVICE_DATA} (
    device_id   VARCHAR PRIMARY KEY,
    device_type VARCHAR DEFAULT 'gps',
    region      VARCHAR DEFAULT '',
    firmware    VARCHAR DEFAULT '',
    status      VARCHAR DEFAULT 'offline',
    first_seen  BIGINT,
    last_seen   BIGINT
)
"""

CREATE_GPS_DATA = f"""
CREATE TABLE IF NOT EXISTS {TABLE_GPS_DATA} (
    id          INTEGER PRIMARY KEY DEFAULT nextval('gps_data_seq'),
    batch_id    VARCHAR,
    region_code VARCHAR,
    group_label VARCHAR,        -- A/B/C
    lat DOUBLE, lng DOUBLE, alt DOUBLE,
    e DOUBLE, n DOUBLE, u DOUBLE,
    h DOUBLE, v DOUBLE, d DOUBLE,
    is_correct  BOOLEAN DEFAULT FALSE,
    device_id   VARCHAR DEFAULT '',
    survey_time BIGINT,
    created_at  BIGINT
)
"""

CREATE_RESULT_DATA = f"""
CREATE TABLE IF NOT EXISTS {TABLE_RESULT_DATA} (
    id          INTEGER PRIMARY KEY DEFAULT nextval('result_data_seq'),
    batch_id    VARCHAR,
    device_id   VARCHAR DEFAULT '',
    region_code VARCHAR,
    result      INTEGER DEFAULT -1,  -- 1=正确 0=错误 -1=未知
    reason      VARCHAR DEFAULT '',
    score       INTEGER DEFAULT 0,
    ab_e DOUBLE, ab_n DOUBLE, ab_u DOUBLE,
    ac_e DOUBLE, ac_n DOUBLE, ac_u DOUBLE,
    bc_e DOUBLE, bc_n DOUBLE, bc_u DOUBLE,
    is_correct  BOOLEAN DEFAULT FALSE,
    survey_time BIGINT,
    created_at  BIGINT
)
"""

CREATE_EVENT_LOG = f"""
CREATE TABLE IF NOT EXISTS {TABLE_EVENT_LOG} (
    id          INTEGER PRIMARY KEY DEFAULT nextval('event_log_seq'),
    event_type  VARCHAR DEFAULT 'info',
    topic       VARCHAR DEFAULT '',
    source      VARCHAR DEFAULT '',
    device_id   VARCHAR DEFAULT '',
    message     VARCHAR DEFAULT '',
    level       VARCHAR DEFAULT 'info',
    created_at  BIGINT
)
"""

# ── 序列自增辅助 ──
CREATE_GPS_SEQ = "CREATE SEQUENCE IF NOT EXISTS gps_data_seq START 1"
CREATE_RESULT_SEQ = "CREATE SEQUENCE IF NOT EXISTS result_data_seq START 1"
CREATE_LOG_SEQ = "CREATE SEQUENCE IF NOT EXISTS event_log_seq START 1"

ALL_CREATES = [
    CREATE_DEVICE_DATA, CREATE_GPS_DATA, CREATE_RESULT_DATA, CREATE_EVENT_LOG,
    CREATE_GPS_SEQ, CREATE_RESULT_SEQ, CREATE_LOG_SEQ,
]

# ── 导入字段兼容 ──
IMPORT_FIELDS = [
    "a_lat","a_lng","a_alt","a_e","a_n","a_u",
    "b_lat","b_lng","b_alt","b_e","b_n","b_u",
    "c_lat","c_lng","c_alt","c_e","c_n","c_u",
]

INSERT_GPS_COLS = "batch_id, region_code, group_label, lat, lng, alt, e, n, u, h, v, d, is_correct, device_id, survey_time, created_at"
INSERT_GPS_PLACEHOLDERS = ", ".join(["?" for _ in range(16)])

INSERT_RESULT_COLS = "batch_id, device_id, region_code, result, reason, score, ab_e, ab_n, ab_u, ac_e, ac_n, ac_u, bc_e, bc_n, bc_u, is_correct, survey_time, created_at"
INSERT_RESULT_PLACEHOLDERS = ", ".join(["?" for _ in range(18)])

INSERT_LOG_COLS = "event_type, topic, source, device_id, message, level, created_at"
INSERT_LOG_PLACEHOLDERS = ", ".join(["?" for _ in range(7)])

INSERT_DEVICE_COLS = "device_id, device_type, region, firmware, status, first_seen, last_seen"
INSERT_DEVICE_PLACEHOLDERS = ", ".join(["?" for _ in range(7)])

#### gateway.py

"""Gateway entry point — simulates the API Gateway layer.

In production, this would be a FastAPI/Flask service behind Nginx/Kong.
This module ties together: verification chain, region routing,
idempotency, and the shard database.
"""

import logging
from auth.verifier import (
    default_verify_chain, VerifyContext, VerifyResult,
    DeviceCertVerifier, SourceHashVerifier, TimestampNonceVerifier,
    SignatureVerifier, JWTVerifier, FaceRecognitionVerifier,
    VerifyChain,
)
from utils.geo import which_polygon
from utils.device_store import get_device
from cache.idempotency import IdempotencyManager
from shard_db import ShardDatabase
from db_config import REGION_CONFIG
from upgrade.duplicate_detector import get_default as get_dup_detector
from upgrade.sequence_checker import get_default as get_seq_checker
from upgrade.circuit_breaker import get_default as get_cb

logger = logging.getLogger(__name__)


class SurveyGateway:
    """Gateway for processing survey data from devices.

    Handles: verification -> region routing -> processing -> idempotency
    """

    def __init__(self, verify_chain=None):
        self.verify_chain = verify_chain or default_verify_chain()
        self.idempotency = IdempotencyManager()
        self.dup_detector = get_dup_detector()
        self.seq_checker = get_seq_checker()
        self.circuit_breaker = get_cb()

    def resolve_region(self, device_id, lat=None, lng=None):
        """Three-layer region resolution:
        1. Device registration (pre-configured)
        2. GPS coordinates (polygon containment)
        3. Fallback
        """
        # Layer 1: device registration
        dev = get_device(device_id)
        device_region = dev["region_code"] if dev else None

        # Layer 2: GPS coordinates
        gps_region = None
        if lat is not None and lng is not None:
            gps_region = which_polygon(lat, lng, REGION_CONFIG)

        # Layer 3: reconcile
        if device_region and gps_region and device_region != gps_region:
            logger.warning(
                "Cross-region: device=%s claims %s, GPS says %s",
                device_id, device_region, gps_region
            )
            # Default: trust GPS (device may be roaming)
            return gps_region

        return device_region or gps_region or "hangzhou"

    def process_survey(self, device_id, A, B, C, batch_id=None,
                       lat=None, lng=None, verify_ctx=None,
                       survey_time=None):
        """Process a single survey data submission.

        Returns (success: bool, message: str, result_dict: dict)
        """
        # 1. Verification
        if verify_ctx:
            result = self.verify_chain.verify(verify_ctx)
            if not result.passed:
                return False, result.message, {"status": "auth_fail"}

        # 2. Idempotency check
        bid = batch_id or f"{device_id}-{int(__import__('time').time())}"
        if self.idempotency.already_processed(device_id, bid):
            cached = self.idempotency.get_result(device_id, bid)
            if cached and cached.get("status") == "success":
                return True, "Already processed (idempotent)", cached
            return False, "Previously failed, retry allowed", {}

        # 3. Region routing
        avg_lat = (A["lat"] + B["lat"] + C["lat"]) / 3.0
        avg_lng = (A["lng"] + B["lng"] + C["lng"]) / 3.0
        region = self.resolve_region(device_id, avg_lat, avg_lng)

        if not region:
            return False, "Unrecognized survey region", {}

        # 4. Process via ShardDatabase
        try:
            with ShardDatabase(region) as shard:
                ok, msg = shard.insert_survey_data(A, B, C, survey_time=survey_time)

            result = {
                "status": "success" if ok else "failed",
                "message": msg,
                "region": region,
                "batch_id": bid,
            }

            self.idempotency.store_result(device_id, bid, result)
            return ok, msg, result

        except Exception as e:
            logger.error("Survey processing error: %s", e, exc_info=True)
            self.idempotency.mark_failed(device_id, bid)
            return False, str(e), {"status": "error"}

#### logging_config.py

﻿"""统一日志配置"""
import logging, logging.handlers, sys, os

_LOG_CONFIGURED = False

def setup(name="survey", level=logging.INFO, log_file=None):
    global _LOG_CONFIGURED
    if _LOG_CONFIGURED:
        return logging.getLogger(name)
    _LOG_CONFIGURED = True

    logger = logging.getLogger(name)
    logger.setLevel(level)

    fmt = logging.Formatter("%(asctime)s [%(levelname)s] %(name)s: %(message)s",
                            datefmt="%Y-%m-%d %H:%M:%S")

    ch = logging.StreamHandler(sys.stdout)
    ch.setFormatter(fmt)
    logger.addHandler(ch)

    if log_file:
        os.makedirs(os.path.dirname(log_file) or ".", exist_ok=True)
        fh = logging.handlers.RotatingFileHandler(
            log_file, maxBytes=10*1024*1024, backupCount=5, encoding="utf-8"
        )
        fh.setFormatter(fmt)
        logger.addHandler(fh)

    return logger

#### main_db.py

# -*- coding: utf-8 -*-
"""
5.docx §9 总部数据库管理器 (4表结构)
"""
import duckdb, logging, os, json
from typing import Dict, List
from db_config import (
    MAIN_DB_PATH, ALL_CREATES,
    TABLE_DEVICE_DATA, TABLE_GPS_DATA, TABLE_RESULT_DATA, TABLE_EVENT_LOG,
    PARQUET_TEMP_DIR, PARTITION_BY, REGION_CONFIG,
)
from db_base import DatabaseBase

logger = logging.getLogger(__name__)

class MainDatabase(DatabaseBase):
    def __init__(self):
        super().__init__()
        self.db_path = MAIN_DB_PATH

    def connect(self):
        self.conn = duckdb.connect(self.db_path)
        for ddl in ALL_CREATES:
            self.conn.execute(ddl)

    def disconnect(self):
        if self.conn:
            self.conn.close()
            self.conn = None

    def get_all_gps(self, limit=1000) -> list:
        try:
            rows = self.conn.execute(
                f"SELECT * FROM {TABLE_GPS_DATA} ORDER BY created_at DESC LIMIT ?", (limit,)
            ).fetchall()
            cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_GPS_DATA} LIMIT 0").description]
            return [dict(zip(cols, r)) for r in rows]
        except:
            return []

    def get_all_results(self, limit=500) -> list:
        try:
            rows = self.conn.execute(
                f"SELECT * FROM {TABLE_RESULT_DATA} ORDER BY created_at DESC LIMIT ?", (limit,)
            ).fetchall()
            cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_RESULT_DATA} LIMIT 0").description]
            return [dict(zip(cols, r)) for r in rows]
        except:
            return []

    def get_summary(self) -> dict:
        try:
            total_gps = self.conn.execute(f"SELECT count(*) FROM {TABLE_GPS_DATA}").fetchone()[0]
            total_results = self.conn.execute(f"SELECT count(*) FROM {TABLE_RESULT_DATA}").fetchone()[0]
            correct = self.conn.execute(
                f"SELECT count(*) FROM {TABLE_RESULT_DATA} WHERE result=1"
            ).fetchone()[0]
            failed = self.conn.execute(
                f"SELECT count(*) FROM {TABLE_RESULT_DATA} WHERE result=0"
            ).fetchone()[0]
            regions = self.conn.execute(
                f"SELECT region_code, count(*) as cnt FROM {TABLE_GPS_DATA} GROUP BY region_code"
            ).fetchall()
            return {
                "total_gps": total_gps,
                "total_results": total_results,
                "correct": correct,
                "failed": failed,
                "regions": {r[0]: r[1] for r in regions},
            }
        except Exception as e:
            return {"error": str(e)}

    def export_parquet(self, table: str = "") -> dict:
        os.makedirs(PARQUET_TEMP_DIR, exist_ok=True)
        tables = [table] if table else [TABLE_GPS_DATA, TABLE_RESULT_DATA]
        exported = {}
        for t in tables:
            try:
                table_dir = os.path.join(PARQUET_TEMP_DIR, t)
                self.conn.execute(f"""
                    COPY (SELECT * FROM {t} ORDER BY region_code, created_at)
                    TO '{table_dir}' (FORMAT PARQUET, CODEC 'ZSTD', PARTITION_BY (region_code))
                """)
                exported[t] = table_dir
                logger.info("Parquet export: %s", table_dir)
            except Exception as e:
                logger.warning("Parquet export %s error: %s", t, e)
        return exported

    def clear_all(self):
        for t in [TABLE_GPS_DATA, TABLE_RESULT_DATA, TABLE_DEVICE_DATA, TABLE_EVENT_LOG]:
            try:
                self.conn.execute(f"DELETE FROM {t}")
            except:
                pass

#### position_compare.py

# -*- coding: utf-8 -*-
"""
位置比对模块 — 三组 A/B/C 定位数据精度校验 (E/N/U 格式 + 5cm 阈值)
"""
import math
from typing import Dict, Tuple


def _enu_distance(a: dict, b: dict) -> Tuple[float, float, float]:
    """计算两点间的三轴距离差 (E, N, U) 单位: m

    E = 东向距离 = 经度差 * cos(平均纬度) * 111320
    N = 北向距离 = 纬度差 * 111320
    U = 高度差
    """
    lat1, lng1 = float(a.get("lat", 0)), float(a.get("lng", 0))
    lat2, lng2 = float(b.get("lat", 0)), float(b.get("lng", 0))
    alt1, alt2 = float(a.get("alt", 0)), float(b.get("alt", 0))

    mean_lat = math.radians((lat1 + lat2) / 2)
    d_lat = math.radians(lat2 - lat1)
    d_lng = math.radians(lng2 - lng1)

    n = d_lat * 111320          # 北向距离 (m)
    e = d_lng * 111320 * math.cos(mean_lat)  # 东向距离 (m)
    u = alt2 - alt1             # 高度差 (m)

    return e, n, u


def compare_position(A: dict, B: dict, C: dict,
                     threshold: float = 0.05) -> Tuple[bool, dict]:
    """比较三组定位数据 A/B/C 的精度差是否全部小于阈值

    Args:
        A/B/C: 每组包含 {"lat", "lng", "alt", "e", "n", "u"}
        threshold: 阈值，单位 m，默认 0.05 (5cm)

    Returns:
        (passed, details)
        passed: True 表示全部通过
        details: {
            "ab": {"e": ..., "n": ..., "u": ...},
            "ac": {"e": ..., "n": ..., "u": ...},
            "bc": {"e": ..., "n": ..., "u": ...},
            "max_diff": 最大差值,
        }
    """
    ab = _enu_distance(A, B)
    ac = _enu_distance(A, C)
    bc = _enu_distance(B, C)

    all_diffs = [abs(x) for d in [ab, ac, bc] for x in d]
    max_diff = max(all_diffs) if all_diffs else 0.0
    passed = max_diff < threshold

    details = {
        "ab": {"e": round(ab[0], 4), "n": round(ab[1], 4), "u": round(ab[2], 4)},
        "ac": {"e": round(ac[0], 4), "n": round(ac[1], 4), "u": round(ac[2], 4)},
        "bc": {"e": round(bc[0], 4), "n": round(bc[1], 4), "u": round(bc[2], 4)},
        "max_diff": round(max_diff, 4),
        "threshold": threshold,
    }
    return passed, details


if __name__ == "__main__":
    A = {"lat": 30.0, "lng": 120.5, "alt": 50.0}
    B = {"lat": 30.00001, "lng": 120.50001, "alt": 50.01}
    C = {"lat": 30.00002, "lng": 120.50002, "alt": 50.02}
    passed, details = compare_position(A, B, C)
    print(f"PASS: {passed}")
    print(f"Details: {details}")
    print(f"AB: E={details['ab']['e']:.4f}m N={details['ab']['n']:.4f}m U={details['ab']['u']:.4f}m")
    print(f"AC: E={details['ac']['e']:.4f}m N={details['ac']['n']:.4f}m U={details['ac']['u']:.4f}m")
    print(f"BC: E={details['bc']['e']:.4f}m N={details['bc']['n']:.4f}m U={details['bc']['u']:.4f}m")

#### shard_db.py

# -*- coding: utf-8 -*-
"""
5.docx §9 数据库改造 - 分片数据库管理器
4表结构: device_data / gps_data / result_data / event_log
"""
import duckdb, logging, os, math, time, json
from typing import Dict, Tuple, List, Optional
from db_config import (
    REGION_CONFIG, MAIN_DB_PATH,
    TABLE_DEVICE_DATA, TABLE_GPS_DATA, TABLE_RESULT_DATA, TABLE_EVENT_LOG,
    ALL_CREATES,
    INSERT_DEVICE_COLS, INSERT_DEVICE_PLACEHOLDERS,
    INSERT_GPS_COLS, INSERT_GPS_PLACEHOLDERS,
    INSERT_RESULT_COLS, INSERT_RESULT_PLACEHOLDERS,
    INSERT_LOG_COLS, INSERT_LOG_PLACEHOLDERS,
    PRECISION_THRESHOLD,
)
from db_base import DatabaseBase

logger = logging.getLogger(__name__)

def _enu(pt: dict) -> tuple:
    """从 dict 提取 e/n/u，兼容旧版 h/v/d 格式"""
    if "e" in pt:
        return float(pt.get("e", 0)), float(pt.get("n", 0)), float(pt.get("u", 0))
    if "h" in pt:
        import math as _m
        en = float(pt["h"]) / _m.sqrt(2)
        return en, en, float(pt.get("v", 0))
    return 0.0, 0.0, 0.0


class ShardDatabase(DatabaseBase):
    """分片数据库（按地区）"""

    def __init__(self, region_code: str, db_path: str = ""):
        super().__init__()
        self.region_code = region_code
        if db_path:
            self.db_path = db_path
        elif region_code in REGION_CONFIG:
            self.db_path = REGION_CONFIG[region_code]["db_path"]
        else:
            self.db_path = os.path.join(
                os.path.dirname(__file__), "databases", f"Home-{region_code}.duckdb"
            )

    def connect(self):
        self.conn = duckdb.connect(self.db_path)
        self._init_db()

    def disconnect(self):
        if self.conn:
            self.conn.close()
            self.conn = None

    def _init_db(self):
        """建表"""
        for ddl in ALL_CREATES:
            self.conn.execute(ddl)
        # 迁移旧表数据
        self._migrate_from_old()

    def _migrate_from_old(self):
        """从旧 position_data_rural/urban 表迁移数据"""
        for old_table in ["position_data_rural", "position_data_urban"]:
            try:
                exists = self.conn.execute(
                    f"SELECT count(*) FROM information_schema.tables WHERE table_name='{old_table}'"
                ).fetchone()[0]
                if not exists:
                    continue
                rows = self.conn.execute(f"SELECT * FROM {old_table}").fetchall()
                if not rows:
                    continue
                cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {old_table} LIMIT 0").description]
                migrated = 0
                for row in rows:
                    d = dict(zip(cols, row))
                    batch_id = d.get("batch_id", f"migrated_{int(time.time())}")
                    rc = d.get("region_code", self.region_code)
                    # gps_data: A/B/C
                    for label in ["a", "b", "c"]:
                        lat = d.get(f"{label}_lat", 0.0)
                        lng = d.get(f"{label}_lng", 0.0)
                        alt = d.get(f"{label}_alt", 0.0)
                        e = d.get(f"{label}_e", d.get(f"{label}_h", 0.0) / math.sqrt(2))
                        n = e  # approx
                        u = d.get(f"{label}_u", d.get(f"{label}_v", 0.0))
                        h = d.get(f"{label}_h", math.sqrt(e*e + n*n))
                        v = u
                        ddd = math.sqrt(h*h + v*v)
                        self._migrate_insert_gps(batch_id, rc, label.upper(), lat, lng, alt,
                                        e, n, u, h, v, ddd,
                                        bool(d.get("is_correct", True)),
                                        "migrated", d.get("survey_time", int(time.time())))
                    # result_data
                    ab_e = d.get("ab_h_diff", 0.0) / math.sqrt(2)
                    ab_n = ab_e
                    ab_u = d.get("ab_v_diff", 0.0)
                    is_correct = bool(d.get("is_correct", True))
                    self._migrate_insert_result(batch_id, "migrated", rc, 1 if is_correct else 0,
                                       "migrated from old", 3 if is_correct else 0,
                                       ab_e, ab_n, ab_u,
                                       d.get("ac_h_diff",0)/math.sqrt(2), d.get("ac_v_diff",0),
                                       d.get("bc_h_diff",0)/math.sqrt(2), d.get("bc_v_diff",0),
                                       is_correct, d.get("survey_time", int(time.time())))
                    migrated += 1
                logger.info("Migrated %d rows from %s", migrated, old_table)
                self.conn.execute(f"DROP TABLE IF EXISTS {old_table}")
            except Exception as e:
                logger.warning("Migration from %s: %s", old_table, e)
        # Drop other unused old tables
        for t in ["position_data_rural", "position_data_urban", "cluster_position_stats",
                   "device_info", "gps_position", "gps_quality", "raw_packet", "alarm_event"]:
            try:
                self.conn.execute(f"DROP TABLE IF EXISTS {t}")
            except:
                pass

    # ── 设备数据 ──
    def upsert_device(self, device_id, device_type="gps", region="", firmware=""):
        now = int(time.time())
        sql = f"""INSERT INTO {TABLE_DEVICE_DATA} ({INSERT_DEVICE_COLS}) VALUES ({INSERT_DEVICE_PLACEHOLDERS})
            ON CONFLICT (device_id) DO UPDATE SET
                last_seen = EXCLUDED.last_seen,
                status = EXCLUDED.status,
                region = CASE WHEN EXCLUDED.region != '' THEN EXCLUDED.region ELSE device_data.region END"""
        self.conn.execute(sql, (device_id, device_type, region, firmware, "online", now, now))

    def _migrate_insert_gps(self, batch_id, region_code, group_label,
                   lat, lng, alt, e=0.0, n=0.0, u=0.0,
                   h=0.0, v=0.0, d=0.0, is_correct=False,
                   device_id="", survey_time=0):
        now = int(time.time())
        sql = f"INSERT INTO {TABLE_GPS_DATA} (id, {INSERT_GPS_COLS}) VALUES (nextval('gps_data_seq'), {INSERT_GPS_PLACEHOLDERS})"
        self.conn.execute(sql, (batch_id, region_code, group_label, lat, lng, alt,
                           e, n, u, h, v, d, is_correct, device_id,
                           survey_time or now, now))

    def _migrate_insert_result(self, batch_id, device_id, region_code, result,
                     reason, score=0,
                     ab_e=0.0, ab_n=0.0, ab_u=0.0,
                     ac_e=0.0, ac_n=0.0, ac_u=0.0,
                     bc_e=0.0, bc_n=0.0, bc_u=0.0,
                     is_correct=False, survey_time=0):
        now = int(time.time())
        sql = f"INSERT INTO {TABLE_RESULT_DATA} (id, {INSERT_RESULT_COLS}) VALUES (nextval('result_data_seq'), {INSERT_RESULT_PLACEHOLDERS})"
        self.conn.execute(sql, (batch_id, device_id, region_code, result, reason, score,
                           ab_e, ab_n, ab_u, ac_e, ac_n, ac_u, bc_e, bc_n, bc_u,

is_correct, survey_time or now, now))

    def get_device(self, device_id: str) -> Optional[dict]:
        rows = self.conn.execute(f"SELECT * FROM {TABLE_DEVICE_DATA} WHERE device_id=?", (device_id,)).fetchall()
        if not rows: return None
        cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_DEVICE_DATA} LIMIT 0").description]
        return dict(zip(cols, rows[0]))

    def list_devices(self) -> list:
        try:
            return self.conn.execute(f"SELECT * FROM {TABLE_DEVICE_DATA} ORDER BY last_seen DESC").fetchall()
        except:
            return []

    # ── GPS 数据 ──
    def insert_gps(self, batch_id, region_code, group_label,
                   lat, lng, alt, e=0.0, n=0.0, u=0.0,
                   h=0.0, v=0.0, d=0.0, is_correct=False,
                   device_id="", survey_time=0):
        now = int(time.time())
        self.conn.execute(f"INSERT INTO {TABLE_GPS_DATA} (id, {INSERT_GPS_COLS}) VALUES (nextval('gps_data_seq'), {INSERT_GPS_PLACEHOLDERS})",
                          (batch_id, region_code, group_label, lat, lng, alt,
                           e, n, u, h, v, d, is_correct, device_id,
                           survey_time or now, now))


    def get_gps_by_batch(self, batch_id: str) -> list:
        rows = self.conn.execute(f"SELECT * FROM {TABLE_GPS_DATA} WHERE batch_id=? ORDER BY group_label",
                                 (batch_id,)).fetchall()
        cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_GPS_DATA} LIMIT 0").description]
        return [dict(zip(cols, r)) for r in rows]

    def get_all_gps(self, limit=500) -> list:
        rows = self.conn.execute(f"SELECT * FROM {TABLE_GPS_DATA} ORDER BY created_at DESC LIMIT ?",
                                 (limit,)).fetchall()
        cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_GPS_DATA} LIMIT 0").description]
        return [dict(zip(cols, r)) for r in rows]

    # ── 结果数据 ──
    def insert_result(self, batch_id, device_id, region_code, result,
                      reason, score=0,
                      ab_e=0.0, ab_n=0.0, ab_u=0.0,
                      ac_e=0.0, ac_n=0.0, ac_u=0.0,
                      bc_e=0.0, bc_n=0.0, bc_u=0.0,
                      is_correct=False, survey_time=0):
        now = int(time.time())
        self.conn.execute(f"INSERT INTO {TABLE_RESULT_DATA} (id, {INSERT_RESULT_COLS}) VALUES (nextval('result_data_seq'), {INSERT_RESULT_PLACEHOLDERS})",
                          (batch_id, device_id, region_code, result, reason, score,
                           ab_e, ab_n, ab_u, ac_e, ac_n, ac_u, bc_e, bc_n, bc_u,
                           is_correct, survey_time or now, now))


    def get_results(self, limit=200) -> list:
        rows = self.conn.execute(f"SELECT * FROM {TABLE_RESULT_DATA} ORDER BY created_at DESC LIMIT ?",
                                 (limit,)).fetchall()
        cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_RESULT_DATA} LIMIT 0").description]
        return [dict(zip(cols, r)) for r in rows]

    def get_result_by_batch(self, batch_id: str) -> Optional[dict]:
        rows = self.conn.execute(f"SELECT * FROM {TABLE_RESULT_DATA} WHERE batch_id=?", (batch_id,)).fetchall()
        if not rows: return None
        cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_RESULT_DATA} LIMIT 0").description]
        return dict(zip(cols, rows[0]))

    # ── 事件日志 ──
    def log_event(self, event_type, topic="", source="", device_id="", message="", level="info"):
        now = int(time.time())
        sid = self.conn.execute("SELECT nextval('event_log_seq')").fetchone()[0]
        self.conn.execute(f"INSERT INTO {TABLE_EVENT_LOG} (id, {INSERT_LOG_COLS}) VALUES (nextval('event_log_seq'), {INSERT_LOG_PLACEHOLDERS})",
                          (event_type, topic, source, device_id, message, level, now))
        return sid

    def get_logs(self, limit=100) -> list:
        rows = self.conn.execute(f"SELECT * FROM {TABLE_EVENT_LOG} ORDER BY created_at DESC LIMIT ?",
                                 (limit,)).fetchall()
        cols = [d[0] for d in self.conn.execute(f"SELECT * FROM {TABLE_EVENT_LOG} LIMIT 0").description]
        return [dict(zip(cols, r)) for r in rows]

    # ── 勘测：保存一轮 A/B/C ──
    def save_survey(self, batch_id: str, A: dict, B: dict, C: dict,
                    region_code: str, round_number: int, is_correct: bool,
                    ab_diff: tuple, ac_diff: tuple, bc_diff: tuple,
                    device_id: str = "", survey_time: int = 0):
        """保存一轮勘测完整数据到 4 张表"""
        now = int(time.time())
        st = survey_time or now
        # 1) 更新设备
        self.upsert_device(device_id or f"survey_{batch_id}", "gps",
                          region_code, "v0.1")
        # 2) 存入 GPS 数据
        for label, pt, diff in [("A", A, ab_diff), ("B", B, ac_diff), ("C", C, bc_diff)]:
            e, n, u = _enu(pt)
            h = math.sqrt(e*e + n*n)
            v = u
            ddd = math.sqrt(h*h + v*v)
            self.insert_gps(batch_id, region_code, label,
                           pt["lat"], pt["lng"], pt["alt"],
                           e, n, u, h, v, ddd,
                           is_correct, device_id, st)
        # 3) 存入结果
        result_code = 1 if is_correct else 0
        self.insert_result(batch_id, device_id or f"survey_{batch_id}",
                          region_code, result_code,
                          "passed" if is_correct else "failed",
                          3 if is_correct else 0,
                          ab_diff[0], ab_diff[1], ab_diff[2],
                          ac_diff[0], ac_diff[1], ac_diff[2],
                          bc_diff[0], bc_diff[1], bc_diff[2],
                          is_correct, st)
        # 4) 日志
        self.log_event("survey", "survey.result", f"app:{region_code}",
                      device_id, f"batch={batch_id} correct={is_correct}", "info")
        return True

    # ── 数据管理 ──
    def delete_by_batch(self, batch_id: str):
        self.conn.execute(f"DELETE FROM {TABLE_GPS_DATA} WHERE batch_id=?", (batch_id,))
        self.conn.execute(f"DELETE FROM {TABLE_RESULT_DATA} WHERE batch_id=?", (batch_id,))

    def clear_all(self):
        self.conn.execute(f"DELETE FROM {TABLE_GPS_DATA}")
        self.conn.execute(f"DELETE FROM {TABLE_RESULT_DATA}")
        self.conn.execute(f"DELETE FROM {TABLE_DEVICE_DATA}")
        self.conn.execute(f"DELETE FROM {TABLE_EVENT_LOG}")

    def count_all(self) -> dict:
        try:
            d = self.conn.execute(f"SELECT count(*) FROM {TABLE_DEVICE_DATA}").fetchone()[0]
            g = self.conn.execute(f"SELECT count(*) FROM {TABLE_GPS_DATA}").fetchone()[0]
            r = self.conn.execute(f"SELECT count(*) FROM {TABLE_RESULT_DATA}").fetchone()[0]
            l = self.conn.execute(f"SELECT count(*) FROM {TABLE_EVENT_LOG}").fetchone()[0]
            return {"devices": d, "gps_records": g, "results": r, "logs": l}
        except:
            return {"devices": 0, "gps_records": 0, "results": 0, "logs": 0}

    # ── 导入导出 ──
    def import_from_chart(self, data: list, survey_type: str = "rural") -> Tuple[int, list]:
        """从图表导入数据 (兼容旧格式)"""
        ok, msgs = 0, []
        for item in data:
            try:
                batch_id = item.get("batch_id", f"import_{int(time.time())}_{ok}")
                rc = item.get("region_code", self.region_code)
                A = {"lat": float(item["a_lat"]), "lng": float(item["a_lng"]), "alt": float(item["a_alt"]),
                     "e": float(item.get("a_e", 0)), "n": float(item.get("a_n", 0)), "u": float(item.get("a_u", 0))}
                B = {"lat": float(item["b_lat"]), "lng": float(item["b_lng"]), "alt": float(item["b_alt"]),
                     "e": float(item.get("b_e", 0)), "n": float(item.get("b_n", 0)), "u": float(item.get("b_u", 0))}
                C = {"lat": float(item["c_lat"]), "lng": float(item["c_lng"]), "alt": float(item["c_alt"]),
                     "e": float(item.get("c_e", 0)), "n": float(item.get("c_n", 0)), "u": float(item.get("c_u", 0))}
                from position_compare import compare_position

is_correct, details = compare_position(A, B, C, threshold=PRECISION_THRESHOLD)
                ab = (details["ab"]["e"], details["ab"]["n"], details["ab"]["u"])
                ac = (details["ac"]["e"], details["ac"]["n"], details["ac"]["u"])
                bc = (details["bc"]["e"], details["bc"]["n"], details["bc"]["u"])
                self.save_survey(batch_id, A, B, C, rc, ok+1, is_correct, ab, ac, bc,
                               device_id=item.get("device_id", ""))
                ok += 1
            except Exception as e:
                msgs.append(f"#{ok+1}: {e}")
        return ok, msgs

    def export_all(self, fmt: str = "json") -> str:
        """导出所有数据"""
        import json, csv
        data = self.get_all_gps(9999)
        ts = int(time.time())
        fname = f"export_{self.region_code}_{ts}.{fmt}"
        fpath = os.path.join(os.path.dirname(self.db_path), fname)
        if fmt == "json":
            with open(fpath, "w", encoding="utf-8") as f:
                json.dump(data, f, ensure_ascii=False, indent=2, default=str)
        elif fmt == "csv":
            if not data: return ""
            with open(fpath, "w", encoding="utf-8-sig", newline="") as f:
                w = csv.DictWriter(f, fieldnames=data[0].keys())
                w.writeheader(); w.writerows(data)
        return fpath


class BatchShardMerger:
    """分库 → 总库 合并器"""

    def __init__(self):
        self.main_conn = duckdb.connect(MAIN_DB_PATH)
        for ddl in ALL_CREATES:
            self.main_conn.execute(ddl)
        self._shards = {}

    def get_shard(self, region_code: str) -> ShardDatabase:
        if region_code not in self._shards:
            s = ShardDatabase(region_code)
            s.connect()
            self._shards[region_code] = s
        return self._shards[region_code]

    def merge_all_regions(self) -> dict:
        results = {}
        for rc in REGION_CONFIG:
            try:
                shard = self.get_shard(rc)
                gps_rows = shard.get_all_gps(99999)
                result_rows = shard.get_results(99999)
                inserted_gps = 0
                for r in gps_rows:
                    try:
                        self.main_conn.execute(f"""
                            INSERT INTO {TABLE_GPS_DATA} ({INSERT_GPS_COLS})
                            VALUES ({INSERT_GPS_PLACEHOLDERS})
                            ON CONFLICT DO NOTHING
                        """, (r["batch_id"], r["region_code"], r["group_label"],
                              r["lat"], r["lng"], r["alt"], r["e"], r["n"], r["u"],
                              r["h"], r["v"], r["d"], r["is_correct"],
                              r["device_id"], r["survey_time"], r["created_at"]))
                        inserted_gps += 1
                    except: pass
                inserted_result = 0
                for r in result_rows:
                    try:
                        self.main_conn.execute(f"""
                            INSERT INTO {TABLE_RESULT_DATA} ({INSERT_RESULT_COLS})
                            VALUES ({INSERT_RESULT_PLACEHOLDERS})
                            ON CONFLICT DO NOTHING
                        """, (r["batch_id"], r["device_id"], r["region_code"],
                              r["result"], r["reason"], r["score"],
                              r["ab_e"], r["ab_n"], r["ab_u"],
                              r["ac_e"], r["ac_n"], r["ac_u"],
                              r["bc_e"], r["bc_n"], r["bc_u"],
                              r["is_correct"], r["survey_time"], r["created_at"]))
                        inserted_result += 1
                    except: pass
                results[rc] = f"gps={inserted_gps} results={inserted_result}"
            except Exception as e:
                results[rc] = f"error: {e}"
        return results

    def close(self):
        for s in self._shards.values():
            try: s.disconnect()
            except: pass
        if self.main_conn:
            self.main_conn.close()



    # ── 向后兼容：旧版 insert_survey_data 适配器 ──
    def insert_survey_data(self, A: dict, B: dict, C: dict) -> Tuple[bool, str]:
        """旧版接口适配器 — 转换到 save_survey"""
        import math, time
        from position_compare import compare_position
        from db_config import PRECISION_THRESHOLD
        passed, details = compare_position(A, B, C, PRECISION_THRESHOLD)
        if not passed:
            return False, "精度差超过阈值"
        region_code = self.region_code
        n = 1
        while True:
            batch_id = f"{region_code[:2].upper()}-{n}"
            existing = self.get_gps_by_batch(batch_id)
            if not existing: break
            n += 1
        ab = details["ab"]; ac = details["ac"]; bc = details["bc"]
        st = int(time.time())
        self.save_survey(batch_id, A, B, C, region_code, n, True,
                        (ab['e'], ab['n'], ab['u']),
                        (ac['e'], ac['n'], ac['u']),
                        (bc['e'], bc['n'], bc['u']), "", st)
        return True, f"成功 {batch_id}"
if __name__ == "__main__":
    import sys
    cmd = sys.argv[1] if len(sys.argv) > 1 else ""
    if cmd == "merge-all":
        m = BatchShardMerger()
        r = m.merge_all_regions()
        m.close()
        for f, s in r.items():
            print(f"  {f}: {s}")
    elif cmd == "list":
        for rc in REGION_CONFIG:
            with ShardDatabase(rc) as db:
                c = db.count_all()
                print(f"{rc}: {c}")
    else:
        print("usage: python shard_db.py [merge-all|list]")

#### test_comprehensive.py

"""综合多轮测试：验证所有优化点和核心功能"""
import sys
import os
import time
import shutil

from shard_db import ShardDatabase
from main_db import MainDatabase
from db_config import DB_DIR, MAIN_DB_PATH, get_insert_sql_and_params
from position_compare import compare_position

PASS = 0
FAIL = 0

def check(name, condition, detail=""):
    global PASS, FAIL
    if condition:
        PASS += 1
        print(f"  [PASS] {name}")
    else:
        FAIL += 1
        print(f"  [FAIL] {name} - {detail}")


def cleanup():
    if os.path.exists(DB_DIR):
        shutil.rmtree(DB_DIR)
    os.makedirs(DB_DIR, exist_ok=True)
    print("[CLEAN] 数据库目录已清空")


def make_group(lat, lng, alt, h, v, d):
    return {"lat": lat, "lng": lng, "alt": alt, "h": h, "v": v, "d": d}


# ================================================================
# T1: 精度比较单元测试
# ================================================================
def test_precision():
    print("\n=== T1: 精度比较单元测试 ===")
    check("完全相同", compare_position([3.0, 3.0, 3.0], [2.0, 2.0, 2.0], [4.0, 4.0, 4.0], 5.0))
    check("差4.9cm通过", compare_position([3.0, 7.9, 3.0], [2.0, 2.0, 2.0], [4.0, 4.0, 4.0], 5.0))
    check("差5.0cm拒绝", not compare_position([3.0, 8.0, 3.0], [2.0, 2.0, 2.0], [4.0, 4.0, 4.0], 5.0))
    check("差10cm拒绝", not compare_position([1.0, 20.0, 1.0], [1.0, 1.0, 1.0], [1.0, 1.0, 1.0], 5.0))


# ================================================================
# T2: 上下文管理器测试
# ================================================================
def test_context_manager():
    print("\n=== T2: 上下文管理器测试 ===")
    s = ShardDatabase("hangzhou")
    with s:
        check("Shard with内已连接", s.conn is not None)
    check("Shard with外已断开", s.conn is None)

    m = MainDatabase()
    with m:
        check("Main with内已连接", m.conn is not None)
    check("Main with外已断开", m.conn is None)


# ================================================================
# T3: 多轮勘测 - 正常流程
# ================================================================
def test_multi_round():
    print("\n=== T3: 多轮勘测 - 正常流程 ===")
    with ShardDatabase("hangzhou") as shard:
        A1 = make_group(30.25, 120.16, 50, 3.0, 2.5, 4.0)
        B1 = make_group(30.25, 120.16, 50, 3.2, 2.7, 4.1)
        C1 = make_group(30.25, 120.16, 50, 3.1, 2.6, 3.9)
        ok, msg = shard.insert_survey_data(A1, B1, C1)
        check("杭州第1轮插入成功", ok, msg)

        A2 = make_group(30.30, 120.20, 60, 4.0, 3.0, 5.0)
        B2 = make_group(30.30, 120.20, 60, 4.1, 3.1, 5.1)
        C2 = make_group(30.30, 120.20, 60, 4.0, 3.0, 5.0)
        ok, msg = shard.insert_survey_data(A2, B2, C2)
        check("杭州第2轮插入成功", ok, msg)

        A3 = make_group(30.35, 120.25, 100, 2.0, 2.0, 3.0)
        B3 = make_group(30.35, 120.25, 100, 2.1, 2.1, 3.1)
        C3 = make_group(30.35, 120.25, 100, 2.0, 2.0, 3.0)
        ok, msg = shard.insert_survey_data(A3, B3, C3)
        check("杭州第3轮插入成功", ok, msg)

        s = shard.get_summary()
        check("杭州分部共3条", s["total"] == 3, f"实际{s['total']}")

        all_data = shard.get_all_data()
        batch_ids = [r["batch_id"] for r in all_data]
        check("批次编号HW-1存在", "HW-1" in batch_ids)
        check("批次编号HW-2存在", "HW-2" in batch_ids)
        check("批次编号HW-3存在", "HW-3" in batch_ids)
        check("无重复批次编号", len(batch_ids) == len(set(batch_ids)),
              f"有 {len(batch_ids) - len(set(batch_ids))} 个重复")


# ================================================================
# T4: 去重测试
# ================================================================
def test_dedup():
    print("\n=== T4: 去重逻辑测试 ===")
    with ShardDatabase("hangzhou") as shard:
        dupA = make_group(30.25, 120.16, 80, 3.0, 2.5, 4.0)
        dupB = make_group(30.30, 120.30, 80, 3.1, 2.6, 4.1)
        dupC = make_group(30.35, 120.40, 80, 3.2, 2.7, 4.2)
        ok, msg = shard.insert_survey_data(dupA, dupB, dupC)
        check("A组位置重复应拒绝", not ok, msg)

        newA = make_group(31.50, 121.50, 150, 2.0, 2.0, 3.0)
        newB = make_group(31.51, 121.51, 150, 2.1, 2.1, 3.1)
        newC = make_group(31.50, 121.50, 150, 2.0, 2.0, 3.0)
        ok, msg = shard.insert_survey_data(newA, newB, newC)
        check("全新位置应通过", ok, msg)


# ================================================================
# T5: 城市农村分类测试
# ================================================================
def test_classification():
    print("\n=== T5: 农村/城市分类测试 ===")
    with ShardDatabase("hangzhou") as shard:
        rural = make_group(30.40, 120.40, 50, 3.0, 2.5, 4.0)
        ok, msg = shard.insert_survey_data(rural,
                    make_group(30.40, 120.40, 50, 3.1, 2.6, 4.1),
                    make_group(30.40, 120.40, 50, 3.0, 2.5, 4.0))
        check("农村高度插入成功", ok, msg)

        urban = make_group(30.50, 120.50, 300, 3.0, 2.5, 4.0)
        ok, msg = shard.insert_survey_data(urban,
                    make_group(30.50, 120.50, 300, 3.1, 2.6, 4.1),
                    make_group(30.50, 120.50, 300, 3.0, 2.5, 4.0))
        check("城市高度插入成功", ok, msg)

        s = shard.get_summary()
        check("农村计数>0", s["rural_count"] > 0, f"实际{s['rural_count']}")
        check("城市计数>0", s["urban_count"] > 0, f"实际{s['urban_count']}")


# ================================================================
# T6: 总部上传验证
# ================================================================
def test_main_upload():
    print("\n=== T6: 总部上传验证 ===")
    with MainDatabase() as main:
        summary = main.get_summary()
        check("总部有数据", summary["total_records"] > 0,
              f"实际{summary['total_records']}条")

all_data = main.get_all_data()
        batch_ids = [r["batch_id"] for r in all_data]
        check("总部批次ID唯一", len(batch_ids) == len(set(batch_ids)), "有重复批次")
        check("总部有城市数据", summary["urban_total"] > 0)
        check("总部有农村数据", summary["rural_total"] > 0)


# ================================================================
# T7: 绍兴分部 + 图表导入
# ================================================================
def test_shaoxing():
    print("\n=== T7: 绍兴分部 + 图表导入 ===")
    with ShardDatabase("shaoxing") as shard:
        chart_data = []
        for i in range(5):
            lat = 29.90 + i * 0.02
            lng = 120.50 + i * 0.02
            chart_data.append({
                "A": make_group(lat, lng, 100, 2.0, 2.0, 3.0),
                "B": make_group(lat, lng, 100, 2.1, 2.1, 3.1),
                "C": make_group(lat, lng, 100, 2.0, 2.0, 3.0),
            })
        ok, msgs = shard.import_from_chart(chart_data, "rural")
        check("绍兴导入5条全成功", ok == 5, f"实际{ok}/5")

        summary = shard.get_summary()
        check("绍兴分部共5条", summary["total"] == 5, f"实际{summary['total']}")

        dup_data = [{"A": chart_data[0]["A"], "B": chart_data[0]["B"], "C": chart_data[0]["C"]}]
        ok2, msgs2 = shard.import_from_chart(dup_data, "rural")
        check("图表导入去重有效", ok2 == 0, f"实际{ok2}")


# ================================================================
# T8: SQL模板统一性测试
# ================================================================
def test_sql_template():
    print("\n=== T8: SQL模板统一性测试 ===")
    A = make_group(30.0, 120.0, 50, 3.0, 2.5, 4.0)
    B = make_group(30.0, 120.0, 50, 3.1, 2.6, 4.1)
    C = make_group(30.0, 120.0, 50, 3.0, 2.5, 4.0)

    sql, params = get_insert_sql_and_params(
        "position_data_rural", "HW-10", "HW", 10, "rural",
        A, B, C, [0.1, 0.1, 0.1], [0.0, 0.0, 0.0], [0.1, 0.1, 0.1], True,
    )
    check("SQL包含INSERT INTO", "INSERT INTO" in sql)
    check("SQL包含33个占位符", sql.count("?") == 33, f"实际{sql.count('?')}个")
    check("参数列表长度33", len(params) == 33, f"实际{len(params)}个")
    check("batch_id正确", params[0] == "HW-10")
    check("is_correct为True", params[31] is True)


# ================================================================
# T9: 错误处理测试
# ================================================================
def test_error_handling():
    print("\n=== T9: 错误处理测试 ===")
    try:
        ShardDatabase("unknown_city")
        check("无效地区应抛出异常", False, "未抛出异常")
    except ValueError:
        check("无效地区抛出ValueError", True)

    try:
        db = ShardDatabase("hangzhou")
        db.insert_survey_data(
            make_group(30, 120, 50, 3, 2, 4),
            make_group(30, 120, 50, 3, 2, 4),
            make_group(30, 120, 50, 3, 2, 4),
        )
        check("未连接时插入应抛出异常", False, "未抛出异常")
    except Exception:
        check("未连接时抛出异常", True)


# ================================================================
# T10: 压力测试 - 快速插入
# ================================================================
def test_bulk_performance():
    print("\n=== T10: 批量性能测试 ===")
    with ShardDatabase("hangzhou") as shard:
        start = time.time()
        count = 0
        for i in range(20):
            lat = 32.0 + i * 0.005
            lng = 122.0 + i * 0.005
            A = make_group(lat, lng, 100 + i, 2.0, 2.0, 3.0)
            B = make_group(lat, lng, 100 + i, 2.1, 2.1, 3.1)
            C = make_group(lat, lng, 100 + i, 2.0, 2.0, 3.0)
            ok, _ = shard.insert_survey_data(A, B, C)
            if ok:
                count += 1
        elapsed = time.time() - start
        summary = shard.get_summary()
        check("20轮全部插入成功", count == 20, f"实际{count}")
        check("分部和总部一致", summary["total"] >= 20, f"实际{summary['total']}")
        print(f"  性能: {count}条 in {elapsed:.2f}s ({count/elapsed:.0f}条/s)")


if __name__ == "__main__":
    cleanup()
    test_precision()
    test_context_manager()
    test_multi_round()
    test_dedup()
    test_classification()
    test_main_upload()
    test_shaoxing()
    test_sql_template()
    test_error_handling()
    test_bulk_performance()

    print("\n" + "=" * 45)
    print(f"  总结果: {PASS} 通过, {FAIL} 失败")
    print("=" * 45)
    print("  全部通过！" if FAIL == 0 else f"  有 {FAIL} 个测试失败，请检查")

#### v4_server.py

"""V4 FastAPI Server"""
import sys, os, json, time
sys.path.insert(0, os.path.dirname(os.path.abspath(__file__)))
from fastapi import FastAPI
import uvicorn

app = FastAPI(title="V4 Survey Server", version="4.0")

@app.get("/")
def root():
    return {"status": "V4 Server", "version": "4.0"}

@app.get("/api/health")
def health():
    import time
    return {"status": "ok", "time": int(time.time())}

@app.get("/api/v4/eventbus")
def eventbus():
    from upgrade.eventbus import bus
    return bus.stats()

@app.get("/api/v4/plugins")
def plugins():
    from upgrade.plugin_sdk import manager
    return manager.stats()

def run_server(host="0.0.0.0", port=8080):
    uvicorn.run(app, host=host, port=port, log_level="info")

if __name__ == "__main__":
    run_server()

#### adapters/__init__.py

from .device_adapter import DeviceAdapter
from .tcp_adapter import TCPAdapter

#### adapters/device_adapter.py

"""V4 DeviceAdapter"""
import abc
class DeviceAdapter(abc.ABC):
    @property
    @abc.abstractmethod
    def name(self): ...
    @abc.abstractmethod
    def connect(self): ...
    @abc.abstractmethod
    def disconnect(self): ...
    @abc.abstractmethod
    def send(self, data): ...
    @abc.abstractmethod
    def on_data(self, cb): ...

#### adapters/tcp_adapter.py

"""TCP Adapter"""
import socket
from .device_adapter import DeviceAdapter
class TCPAdapter(DeviceAdapter):
    name = "tcp"
    def __init__(self, host="0.0.0.0", port=8888):
        self.host, self.port = host, port
        self._sock = None
    def connect(self):
        self._sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
        self._sock.setsockopt(socket.SOL_SOCKET, socket.SO_REUSEADDR, 1)
        self._sock.bind((self.host, self.port))
        self._sock.listen(5)
    def disconnect(self):
        if self._sock: self._sock.close()
    def send(self, data): pass
    def on_data(self, cb): pass

#### auth/__init__.py

﻿

#### auth/verifier.py

"""Pluggable verification chain for device/auth validation.

Each step is a Verifier subclass. Add new steps via chain.add() without
modifying existing code. See FaceRecognitionVerifier for example.
"""

import hashlib
import hmac
import json
import time
from typing import NamedTuple


class VerifyResult(NamedTuple):
    passed: bool
    message: str


class VerifyContext:
    """Context passed through the verification chain.

    Each verifier can read and write to this context.
    """
    def __init__(self, device_id="", source_hash="", token="",
                 cert_fingerprint="", face_vector=None,
                 timestamp=0, nonce="", signature="", secret_key=""):
        self.device_id = device_id
        self.source_hash = source_hash
        self.token = token
        self.cert_fingerprint = cert_fingerprint
        self.face_vector = face_vector
        self.timestamp = timestamp
        self.nonce = nonce
        self.signature = signature
        self.secret_key = secret_key
        self.extra = {}  # custom data


class Verifier:
    """Base class for all verifiers"""
    def check(self, ctx: VerifyContext) -> VerifyResult:
        raise NotImplementedError


class VerifyChain:
    """Pluggable verification chain.

    Usage:
        chain = VerifyChain()
        chain.add(Step1Verifier())
        chain.add(Step2Verifier())
        result = chain.verify(ctx)
    """
    def __init__(self):
        self._steps = []

    def add(self, verifier: Verifier):
        self._steps.append(verifier)
        return self

    def verify(self, ctx: VerifyContext) -> VerifyResult:
        for step in self._steps:
            result = step.check(ctx)
            if not result.passed:
                return result
        return VerifyResult(True, "All verifications passed")


# ── Concrete Verifiers ──────────────────────────

class DeviceCertVerifier(Verifier):
    """Verify device certificate fingerprint"""
    def __init__(self):
        from utils.device_store import check_cert
        self._check_cert = check_cert

    def check(self, ctx: VerifyContext) -> VerifyResult:
        if not ctx.device_id:
            return VerifyResult(False, "Missing device_id")
        if not self._check_cert(ctx.device_id, ctx.cert_fingerprint):
            return VerifyResult(False, "Device certificate mismatch")
        return VerifyResult(True, "Device cert verified")


class SourceHashVerifier(Verifier):
    """Verify mini-program source code hash"""
    ALLOWED_HASHES = [
        "a1b2c3d4e5f6a7b8c9d0e1f2a3b4c5d6e7f8a9b0",  # known good versions
    ]

    def check(self, ctx: VerifyContext) -> VerifyResult:
        if not ctx.source_hash:
            return VerifyResult(False, "Missing source hash")
        if ctx.source_hash not in self.ALLOWED_HASHES:
            return VerifyResult(False, "Source code hash not authorized")
        return VerifyResult(True, "Source hash verified")


class TimestampNonceVerifier(Verifier):
    """Anti-replay: timestamp must be within 5 min, nonce unused"""
    MAX_AGE = 300  # 5 minutes
    _used_nonces = set()  # in production: Redis SET with TTL

    def check(self, ctx: VerifyContext) -> VerifyResult:
        now = time.time()
        if abs(now - ctx.timestamp) > self.MAX_AGE:
            return VerifyResult(False, "Timestamp out of range")
        if ctx.nonce in self._used_nonces:
            return VerifyResult(False, "Nonce already used (replay)")
        self._used_nonces.add(ctx.nonce)
        return VerifyResult(True, "Timestamp + nonce OK")


class SignatureVerifier(Verifier):
    """HMAC-SHA256 signature verification"""
    def check(self, ctx: VerifyContext) -> VerifyResult:
        if not ctx.secret_key:
            return VerifyResult(True, "No secret key configured, skipping")
        if not ctx.signature:
            return VerifyResult(False, "Missing signature")
        payload = f"{ctx.device_id}|{ctx.timestamp}|{ctx.nonce}|{ctx.source_hash}"
        expected = hmac.new(
            ctx.secret_key.encode(), payload.encode(), hashlib.sha256
        ).hexdigest()
        if not hmac.compare_digest(expected, ctx.signature):
            return VerifyResult(False, "Signature mismatch")
        return VerifyResult(True, "Signature verified")


class FaceRecognitionVerifier(Verifier):
    """Face recognition verifier — pluggable, can be added/removed freely.

    In production, replace cosine_similarity with your model inference.
    Face vectors are 128-dim float arrays from a face embedding model.
    """
    def __init__(self, threshold=0.85):
        self.threshold = threshold
        from utils.device_store import get_face_embedding
        self._get_embedding = get_face_embedding
        self._model_loaded = False

    def _load_model(self):
        """Lazy load face model — replace with your actual model"""
        self._model_loaded = True

    def _cosine_similarity(self, v1, v2):
        """Cosine similarity between two vectors"""
        if not v1 or not v2:
            return 0.0

dot = sum(a * b for a, b in zip(v1, v2))
        n1 = sum(a * a for a in v1) ** 0.5
        n2 = sum(b * b for b in v2) ** 0.5
        if n1 == 0 or n2 == 0:
            return 0.0
        return dot / (n1 * n2)

    def check(self, ctx: VerifyContext) -> VerifyResult:
        if ctx.face_vector is None:
            return VerifyResult(True, "No face data, skipping")
        if not self._model_loaded:
            self._load_model()

        stored = self._get_embedding(ctx.device_id)
        if stored is None:
            return VerifyResult(False, "No face registered for device")

        similarity = self._cosine_similarity(ctx.face_vector, stored)
        if similarity >= self.threshold:
            return VerifyResult(True, f"Face match: {similarity:.3f}")
        return VerifyResult(
            False, f"Face mismatch: {similarity:.3f} < {self.threshold}"
        )


class JWTVerifier(Verifier):
    """Simple JWT-like token verification (no external lib needed)

    Token format: base64(device_id|expiry|signature)
    In production, use PyJWT library with RS256.
    """
    def __init__(self, secret="default-secret-change-in-prod"):
        self.secret = secret
        import base64
        self._b64 = base64

    def _sign(self, payload):
        import hashlib
        return hashlib.sha256((payload + self.secret).encode()).hexdigest()[:16]

    def issue_token(self, device_id, expiry=1800):
        """Issue a temporary token (30 min validity)"""
        payload = f"{device_id}|{int(time.time()) + expiry}"
        sig = self._sign(payload)
        token = self._b64.b64encode(f"{payload}|{sig}".encode()).decode()
        return token

    def check(self, ctx: VerifyContext) -> VerifyResult:
        if not ctx.token:
            return VerifyResult(False, "Missing token")
        try:
            decoded = self._b64.b64decode(ctx.token.encode()).decode()
            parts = decoded.split("|")
            if len(parts) != 3:
                return VerifyResult(False, "Invalid token format")
            device_id, expiry_str, sig = parts
            sig_check = self._sign(f"{device_id}|{expiry_str}")
            if sig != sig_check:
                return VerifyResult(False, "Token signature invalid")
            if int(time.time()) > int(expiry_str):
                return VerifyResult(False, "Token expired")
            return VerifyResult(True, "JWT verified")
        except Exception:
            return VerifyResult(False, "Token parse error")


# ── Factory: build a standard chain ─────────────

def default_verify_chain(include_face=False):
    """Build the default verification chain.

    Add face recognition by passing include_face=True.
    No existing code changes needed when adding new verifiers.
    """
    chain = VerifyChain()
    chain.add(DeviceCertVerifier())
    chain.add(SourceHashVerifier())
    chain.add(TimestampNonceVerifier())
    chain.add(SignatureVerifier())
    chain.add(JWTVerifier())
    if include_face:
        chain.add(FaceRecognitionVerifier())
    return chain

#### cache/__init__.py

﻿

#### cache/idempotency.py

"""Idempotency key management + L1/L2 cache for dedup

L1: In-memory LRU (fastest, local to process)
L2: Redis (distributed, survives restarts)
"""

import time
import hashlib
import json
import logging
from collections import OrderedDict

logger = logging.getLogger(__name__)


class LRUCache:
    """Simple LRU cache for L1 dedup"""
    def __init__(self, capacity=10000):
        self.capacity = capacity
        self._cache = OrderedDict()

    def get(self, key):
        if key in self._cache:
            self._cache.move_to_end(key)
            return self._cache[key]
        return None

    def set(self, key, value):
        if key in self._cache:
            self._cache.move_to_end(key)
        self._cache[key] = value
        if len(self._cache) > self.capacity:
            self._cache.popitem(last=False)

    def __contains__(self, key):
        return key in self._cache


class IdempotencyManager:
    """Idempotency key manager with L1/L2 cache hierarchy.

    Key format: survey:{device_id}:{batch_id}

    Usage:
        im = IdempotencyManager()
        if im.already_processed(device_id, batch_id):
            return im.get_result(device_id, batch_id)
        result = process(data)
        im.store_result(device_id, batch_id, result)
    """
    def __init__(self, l1_capacity=10000, redis_client=None):
        self.l1 = LRUCache(l1_capacity)
        self.l2 = redis_client  # optional Redis client

    def _make_key(self, device_id, batch_id):
        return f"survey:{device_id}:{batch_id}"

    def already_processed(self, device_id, batch_id):
        key = self._make_key(device_id, batch_id)
        # Check L1
        if key in self.l1:
            return True
        # Check L2 (optional)
        if self.l2:
            return self.l2.exists(key)
        return False

    def get_result(self, device_id, batch_id):
        key = self._make_key(device_id, batch_id)
        val = self.l1.get(key)
        if val:
            return json.loads(val)
        if self.l2:
            val = self.l2.get(key)
            if val:
                self.l1.set(key, val)
                return json.loads(val)
        return None

    def store_result(self, device_id, batch_id, result, ttl=86400):
        key = self._make_key(device_id, batch_id)
        val = json.dumps(result)
        self.l1.set(key, val)
        if self.l2:
            self.l2.setex(key, ttl, val)

    def mark_failed(self, device_id, batch_id):
        """Mark a batch as failed so retry is possible"""
        key = self._make_key(device_id, batch_id)
        fail_val = json.dumps({"status": "failed", "retry_allowed": True})
        self.l1.set(key, fail_val)

#### cli/__init__.py

"""V4-Local CLI 测试引擎"""

#### cli/device_control.py

"""V4-Local 设备控制终端

用于启动/停止/监控模拟设备。
用法：
python -m cli.device_control list
python -m cli.device_control start cm510
"""
import argparse, sys, os, time, logging
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

from upgrade.eventbus import Topics, bus

logging.basicConfig(level=logging.INFO, format="%(levelname)s %(message)s")

# 模拟设备注册表
DEVICES = {
    "cm510": {"desc": "CM510-71X 无线数传设备", "topics": [Topics.CM510_SIGNAL]},
    "k803": {"desc": "K803 GNSS 模块", "topics": [Topics.K803_SIGNAL, Topics.GPS_DATA]},
    "gps_sim": {"desc": "GPS 模拟器", "topics": [Topics.GPS_DATA]},
}

def cmd_list():
    print("📋 可用设备列表")
    print("   " + "-" * 40)
    for name, info in DEVICES.items():
        print(f"   {name:<12} {info['desc']}")
        print(f"              topics: {', '.join(info['topics'])}")
    print()

def cmd_start(device: str, count: int = 3):
    if device not in DEVICES:
        print(f"❌ 未知设备: {device}")
        return
    print(f"▶️ 启动设备: {device} ({DEVICES[device]['desc']})")
    print(f"   将发送 {count} 条模拟数据...")
    for i in range(count):
        time.sleep(1)
        bus.publish(Topics.GPS_DATA, {"device": device, "index": i,
                     "lat": 30.0, "lng": 120.5, "alt": 50.0,
                     "timestamp": time.time()}, source=f"device:{device}")
        print(f"   📤 发送 #{i+1} -> {Topics.GPS_DATA}")
    print("✅ 发送完成")

def main():
    parser = argparse.ArgumentParser(description="设备控制终端")
    parser.add_argument("command", choices=["list", "start"], help="命令")
    parser.add_argument("device", nargs="?", default="", help="设备名")
    parser.add_argument("--count", type=int, default=3, help="发送次数")
    args = parser.parse_args()

    if args.command == "list":
        cmd_list()
    elif args.command == "start":
        cmd_start(args.device, args.count)

if __name__ == "__main__":
    main()

#### cli/device_simulator.py

"""5.docx §8 CLI 设备模拟器"""
import argparse, sys, os, time
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))
from eventbus import Event, Topic, bus

def main():
    parser = argparse.ArgumentParser(description="设备模拟器")
    parser.add_argument("--type", choices=["gps","cm510","k803"], default="gps")
    parser.add_argument("--count", type=int, default=3)
    parser.add_argument("--delay", type=float, default=1.0)
    args = parser.parse_args()
    topic_map = {"gps": Topic.GPS_DATA, "cm510": Topic.SIGNAL_CM510, "k803": Topic.SIGNAL_K803}
    print(f"模拟设备: type={args.type} count={args.count}")
    for i in range(args.count):
        bus.publish(topic_map[args.type], Event(topic=topic_map[args.type],
            device_id=f"{args.type}_{i:03d}", data={"index": i, "sim": True}))
        print(f"  [{i+1}/{args.count}] {topic_map[args.type]}")
        time.sleep(args.delay)
    print("完成")

if __name__ == "__main__":
    main()

#### cli/main.py

"""5.docx §8.1 CLI主入口"""
import sys, os
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

def main():
    print("=" * 50)
    print("  V4-Local CLI 测试引擎 (5.docx §8)")
    print("=" * 50)
    print("\n  命令:")
    print("  python -m cli.run_test --device gps --count 5")
    print("  python -m cli.replay --lat 30.0 --lng 120.5")
    print("  python -m cli.stream_view --topic gps.data")
    print("  python -m cli.result_query CM510_001")
    print("  python -m cli.device_simulator --type cm510 --count 3")
    print("  python -m cli.stress_test --count 100")
    print()

if __name__ == "__main__":
    main()

#### cli/replay.py

"""CLI GPS回放工具

用法：
python -m cli.replay --file data.txt
python -m cli.replay --lat 30.0 --lng 120.5 --count 5
"""
import argparse, sys, os
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))
from upgrade.eventbus import Topics, bus
from replay.gps_replay import GPSReplayEngine

def main():
    parser = argparse.ArgumentParser(description="GPS回放CLI工具")
    parser.add_argument("--file", default="", help="回放文件路径")
    parser.add_argument("--lat", type=float, default=30.0, help="模拟纬度")
    parser.add_argument("--lng", type=float, default=120.5, help="模拟经度")
    parser.add_argument("--alt", type=float, default=50.0, help="模拟高度")
    parser.add_argument("--count", type=int, default=3, help="回放条数")
    parser.add_argument("--speed", type=float, default=1.0, help="回放速度倍数")
    parser.add_argument("--topic", default=Topics.GPS_DATA, help="发布主题")
    args = parser.parse_args()
    engine = GPSReplayEngine(speed=args.speed)
    if args.file:
        loaded = engine.load_file(args.file)
        cnt = engine.run(source="cli:replay")
        print(f"文件回放: 加载{loaded}条, 回放{cnt}条")
    else:
        cnt = engine.run_geo(args.lat, args.lng, args.alt, count=args.count, source="cli:replay")
        print(f"坐标回放: {cnt}条 (lat={args.lat}, lng={args.lng})")

if __name__ == "__main__":
    main()

#### cli/result_query.py

"""5.docx §8 CLI 结果查询"""
import argparse, sys, os, json
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))
from result_service.store import store

def main():
    parser = argparse.ArgumentParser(description="查询设备结果")
    parser.add_argument("device_id", nargs="?", default="", help="设备ID")
    parser.add_argument("--all", action="store_true", help="显示全部")
    args = parser.parse_args()
    if args.all:
        all_r = store.get_all()
        for r in all_r:
            print(f"  {r.get('device_id'):<20} result={r.get('result')}  {r.get('reason','')}")
        print(f"\n总计: {len(all_r)} 条")
    elif args.device_id:
        r = store.get_or_default(args.device_id)
        emoji = {1:"OK",0:"FAIL",-1:"?"}
        print(f"  {emoji.get(r.get('result'),'?')} {args.device_id}: result={r.get('result')}  reason={r.get('reason')}")
    else:
        print("用法: python -m cli.result_query <device_id>")
        print("      python -m cli.result_query --all")

if __name__ == "__main__":
    main()

#### cli/run_test.py

"""V4-Local CLI 测试启动器

用法：
python -m cli.run_test --device cm510
python -m cli.run_test --device k803 --count 10
"""
import argparse, logging, time, sys, os, json
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

from upgrade.eventbus import Event, Topics, bus

logging.basicConfig(level=logging.INFO, format="%(levelname)s %(message)s")

def main():
    parser = argparse.ArgumentParser(description="勘测系统测试引擎")
    parser.add_argument("--device", choices=["cm510", "k803", "gps", "all"], default="gps",
                        help="模拟设备类型")
    parser.add_argument("--count", type=int, default=5, help="发送报文数")
    parser.add_argument("--delay", type=float, default=1.0, help="发送间隔(秒)")
    parser.add_argument("--topic", default="", help="指定发布主题")
    parser.add_argument("--file", default="", help="从文件读取报文")
    args = parser.parse_args()

    print(f"🔧 测试引擎启动: device={args.device} count={args.count}")
    print(f"   间隔={args.delay}s 主题={args.topic or 'auto'} 文件={args.file or '无'}")
    print()

    # 注册结果订阅
    results = []
    def on_result(event: Event):
        results.append(event.payload)
        print(f"  📥 收到: topic={event.topic} source={event.source} payload_keys={list(event.payload.keys())}")

    bus.subscribe(Topics.ANALYSIS_RESULT, on_result)
    bus.subscribe(Topics.SURVEY_RESULT, on_result)
    bus.subscribe(Topics.DEVICE_PARSED, on_result)

    # 发送测试事件
    sent = 0
    for i in range(args.count):
        payload = _gen_payload(args.device, i)
        topic = args.topic or Topics.GPS_DATA
        bus.publish(topic, payload, source=f"test:{args.device}")
        sent += 1
        print(f"  📤 发送 #{i+1}: topic={topic}")
        time.sleep(args.delay)

    time.sleep(0.5)
    print(f"\n✅ 测试完成: 发送 {sent} 条, 收到 {len(results)} 条")
    print(f"   EventBus 统计: {bus.stats()}")

def _gen_payload(device: str, idx: int) -> dict:
    """生成模拟数据"""
    import random
    base_lat, base_lng = 30.0, 120.5
    if device == "gps":
        return {
            "lat": round(base_lat + random.uniform(-0.01, 0.01), 6),
            "lng": round(base_lng + random.uniform(-0.01, 0.01), 6),
            "alt": round(50 + random.uniform(-5, 5), 2),
            "e": round(random.uniform(0.01, 0.05), 3),
            "n": round(random.uniform(0.01, 0.05), 3),
            "u": round(random.uniform(0.02, 0.06), 3),
            "quality": "good",
        }
    elif device == "cm510":
        return {
            "device_id": f"CM510-{idx:03d}",
            "signal_strength": random.randint(60, 95),
            "data": f"mock_cm510_data_{idx}",
            "lat": round(base_lat + random.uniform(-0.005, 0.005), 6),
            "lng": round(base_lng + random.uniform(-0.005, 0.005), 6),
        }
    elif device == "k803":
        return {
            "device_id": f"K803-{idx:03d}",
            "gps_status": "FIX" if random.random() > 0.1 else "NO_FIX",
            "lat": round(base_lat + random.uniform(-0.01, 0.01), 6),
            "lng": round(base_lng + random.uniform(-0.01, 0.01), 6),
            "alt": round(50 + random.uniform(-5, 5), 2),
        }
    return {"device": device, "index": idx, "timestamp": time.time()}

if __name__ == "__main__":
    main()

#### cli/show_stream.py

"""实时流查看器（show_stream 别名）

文档3.docx line62要求此文件名。
实际功能委托给 cli/stream_view.py。
"""
import sys, os
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))
print("提示: show_stream.py 已整合到 cli/stream_view.py")
print("请使用: python -m cli.stream_view --topic gps.data\n")
from cli.stream_view import main
if __name__ == "__main__":
    main()

#### cli/stream_view.py

"""V4-Local 实时流查看器

实时订阅 EventBus 事件流并在终端显示。
用法：
python -m cli.stream_view --topic gps.data
python -m cli.stream_view --all
"""
import argparse, threading, time, sys, os, json
from datetime import datetime
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

from upgrade.eventbus import Event, Topics, bus

def main():
    parser = argparse.ArgumentParser(description="实时事件流查看器")
    parser.add_argument("--topic", default="", help="过滤主题")
    parser.add_argument("--all", action="store_true", help="显示所有主题")
    parser.add_argument("--tail", type=int, default=50, help="显示历史条数")
    args = parser.parse_args()

    display_topic = args.topic or ("*" if args.all else "")

    print("📡 实时事件流查看器")
    print("   " + "-" * 40)
    print(f"   监听: {display_topic or 'gps.data'}")
    print(f"   历史显示: {args.tail} 条")
    print("   Ctrl+C 退出")
    print()

    received = [0]
    def on_event(event: Event):
        received[0] += 1
        ts = datetime.fromtimestamp(event.timestamp).strftime("%H:%M:%S.%f")[:12]
        src = event.source or "?"
        plen = len(json.dumps(event.payload))
        print(f"  [{ts}] {event.topic:<20} src={src:<12} id={event.id:<8} size={plen}B")
        if event.payload:
            items = "; ".join(f"{k}={v}" for k, v in list(event.payload.items())[:6])
            print(f"         └─ {items}")
        print()

    # 先回放历史
    if args.tail > 0:
        topic_filter = display_topic if display_topic and display_topic != "*" else ""
        events = bus.replay_all()
        count = 0
        for ev in reversed(events):
            if topic_filter and ev.topic != topic_filter:
                continue
            count += 1
            ts = datetime.fromtimestamp(ev.timestamp).strftime("%H:%M:%S.%f")[:12]
            print(f"  ↻ [{ts}] {ev.topic:<20} src={ev.source}")
            if count >= args.tail:
                break
        if count > 0:
            print(f"  ... 显示 {count} 条历史")
        print()

    # 订阅实时
    bus.subscribe(display_topic if display_topic else Topics.GPS_DATA, on_event)
    if display_topic == "*":
        bus.subscribe("*", on_event)

    try:
        while True:
            time.sleep(1)
    except KeyboardInterrupt:
        print(f"\n👋 停止. 共接收 {received[0]} 条事件")
        bus.unsubscribe(display_topic if display_topic else Topics.GPS_DATA, on_event)

if __name__ == "__main__":
    main()

#### cli/stress_test.py

"""V4-Local CLI 压力测试工具

用法：
python -m cli.stress_test --count 100 --device gps
python -m cli.stress_test --count 500 --device cm510 --api http://localhost:8080
"""
import argparse, sys, os, time, json, random
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

from upgrade.eventbus import Event, Topics, bus

def main():
    parser = argparse.ArgumentParser(description="CLI 压力测试")
    parser.add_argument("--count", type=int, default=50, help="发送报文数")
    parser.add_argument("--device", choices=["gps", "cm510", "k803"], default="gps")
    parser.add_argument("--delay", type=float, default=0.01, help="发送间隔(秒)")
    parser.add_argument("--api", default="", help="HTTP API 地址（可选）")
    args = parser.parse_args()

    print(f"⚡ 压力测试: device={args.device} count={args.count} delay={args.delay}s")
    start = time.time()
    ok = 0; fail = 0

    for i in range(args.count):
        payload = _gen(args.device, i)
        try:
            bus.publish(Topics.DEVICE_RAW, payload, source=f"stress:{args.device}")
            ok += 1
        except:
            fail += 1
        time.sleep(args.delay)

    elapsed = time.time() - start
    rps = args.count / elapsed if elapsed > 0 else 0
    print(f"\n结果: 成功={ok} 失败={fail} 耗时={elapsed:.2f}s RPS={rps:.0f}")

def _gen(device, idx):
    random.seed(idx)
    if device == "gps":
        return {"lat": round(30 + random.random()/10, 6), "lng": round(120.5 + random.random()/10, 6),
                "alt": round(50 + random.random()*10, 2), "e": 0.01, "n": 0.01, "u": 0.02}
    return {"device": device, "index": idx, "data": f"mock_{idx}"}

if __name__ == "__main__":
    main()

#### cli/test_runner.py

"""5.docx §8 CLI 测试运行器"""
import argparse, sys, os, time
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))
from eventbus import Event, Topic, bus

def main():
    parser = argparse.ArgumentParser(description="CLI测试运行器")
    parser.add_argument("--device", choices=["gps","cm510","k803"], default="gps")
    parser.add_argument("--count", type=int, default=3)
    parser.add_argument("--delay", type=float, default=0.5)
    args = parser.parse_args()
    print(f"测试运行: device={args.device} count={args.count}")
    for i in range(args.count):
        data = {"lat": 30.0+i*0.001, "lng": 120.5+i*0.001, "alt": 50.0, "e": 0.01, "n": 0.01, "u": 0.02}
        bus.publish(Topic.GPS_DATA, Event(topic=Topic.GPS_DATA, device_id=args.device, data=data))
        print(f"  [{i+1}/{args.count}] -> {Topic.GPS_DATA}")
        time.sleep(args.delay)
    print("完成")

if __name__ == "__main__":
    main()

#### eventbus/__init__.py

"""V4-Local EventBus — 5.docx §3 标准化事件系统"""
from .core import Event, EventBus, bus
from .topic import Topic
from .stream import StreamPipeline

#### eventbus/core.py

"""5.docx §3 EventBus 核心

标准事件结构:
    id: str, timestamp: int, topic: str, device_id: str, data: dict
"""
import uuid, time, logging, threading
from collections import defaultdict
from typing import Callable, Optional

logger = logging.getLogger(__name__)

class Event:
    def __init__(self, topic="", device_id="", data=None):
        self.id = str(uuid.uuid4())[:8]
        self.timestamp = int(time.time() * 1000)
        self.topic = topic
        self.device_id = device_id
        self.data = data or {}

    def to_dict(self):
        return {"id": self.id, "timestamp": self.timestamp,
                "topic": self.topic, "device_id": self.device_id, "data": self.data}

class EventBus:
    def __init__(self, max_history=5000):
        self._subs = defaultdict(list)
        self._lock = threading.Lock()
        self._history = []
        self._max = max_history
        self._pipeline = None
        self._count = 0

    def set_pipeline(self, p):
        self._pipeline = p

    def publish(self, topic, event):
        if isinstance(event, Event):
            e = event
        elif isinstance(event, dict):
            e = Event(topic=event.get("topic", topic),
                      device_id=event.get("device_id", ""),
                      data=event.get("data", event))
        else:
            e = Event(topic=topic, data={"raw": str(event)})
        if self._pipeline:
            r = self._pipeline.process(e.to_dict())
            if r is None: return
            e = Event(**r)
        with self._lock:
            cbs = list(self._subs.get(e.topic, []))
            wcbs = list(self._subs.get("*", []))
            self._history.append(e)
            if len(self._history) > self._max: self._history.pop(0)
            self._count += 1
        for cb in cbs + wcbs:
            try: cb(e)
            except Exception as ex: logger.warning("publish error: %s", ex)

    def subscribe(self, topic, handler):
        with self._lock:
            if handler not in self._subs[topic]:
                self._subs[topic].append(handler)

    def unsubscribe(self, topic, handler):
        with self._lock:
            if handler in self._subs[topic]:
                self._subs[topic].remove(handler)

    def replay(self, topic, start_ms=0, end_ms=0):
        if end_ms <= 0: end_ms = int(time.time() * 1000) + 1000
        res = []
        with self._lock:
            for e in self._history:
                if e.topic == topic and start_ms <= e.timestamp <= end_ms:
                    res.append(e)
        return res

    def stats(self):
        with self._lock:
            return {"count": self._count, "history": len(self._history),
                    "max_history": self._max,
                    "topics": {t: len(c) for t, c in self._subs.items()}}

bus = EventBus()

#### eventbus/stream.py

"""5.docx §3.5 Stream Pipeline"""
import logging
from typing import Callable, Optional

logger = logging.getLogger(__name__)

class StreamPipeline:
    def __init__(self):
        self._stages = []

    def add_stage(self, name: str, handler: Callable) -> "StreamPipeline":
        self._stages.append((name, handler))
        return self

    def process(self, event: dict) -> Optional[dict]:
        current = event
        for name, handler in self._stages:
            try:
                current = handler(current)
                if current is None:
                    return None
            except Exception as e:
                logger.error("Pipeline stage %s error: %s", name, e)
                return None
        return current

#### eventbus/topic.py

"""5.docx §3.3 标准Topic定义"""
class Topic:
    DEVICE_RAW = "device.raw"
    DEVICE_PARSED = "device.parsed"
    GPS_DATA = "gps.data"
    SIGNAL_CM510 = "signal.cm510"
    SIGNAL_K803 = "signal.k803"
    ANALYSIS_RESULT = "analysis.result"
    SYSTEM_ANOMALY = "system.anomaly"

    @classmethod
    def list(cls):
        return [v for k, v in vars(cls).items() if not k.startswith("_") and isinstance(v, str)]

#### monitor/__init__.py

#### monitor/__main__.py

"""Network Health Check Zone - CLI
Usage:
    python -m monitor              # all devices
    python -m monitor HW2024001    # specific device
"""
import sys
from monitor.reporter import print_check_zone, get_recommendations
from monitor.metrics_collector import MetricsCollector

if len(sys.argv) > 1:
    did = sys.argv[1]
    print_check_zone(did)
    m = MetricsCollector.get_instance().get_device_metrics(did)
    if m:
        print()
        for r in get_recommendations(m):
            print("  " + r)
else:
    print_check_zone()
    all_m = MetricsCollector.get_instance().get_all_metrics()
    for did in all_m:
        recs = get_recommendations(all_m[did])
        for r in recs:
            print("  [" + did + "] " + r)

#### monitor/metrics_collector.py

"""Real-time network health metrics collector (thread-safe)

Records: packet loss, CRC failures, latency, success rates
Used by: bridge_service.py (injected) and reporter.py (read-only)
"""

import time
import threading
from collections import defaultdict


class _DeviceMetrics:
    """Per-device metrics storage"""
    __slots__ = ("total_msgs", "lost_packets", "crc_failures",
                  "retransmits", "successes", "failures",
                  "last_seq", "start_time", "latency_samples")

    def __init__(self):
        self.total_msgs = 0
        self.lost_packets = 0
        self.crc_failures = 0
        self.retransmits = 0
        self.successes = 0
        self.failures = 0
        self.last_seq = 0
        self.start_time = time.time()
        self.latency_samples = []


class MetricsCollector:
    """Thread-safe singleton: collects network health metrics in real-time.

    Usage in bridge:
        metrics = MetricsCollector.get_instance()
        metrics.record_msg("device-1")
        metrics.record_crc_fail("device-1")
        metrics.record_loss("device-1", from_seq=5, to_seq=7)
    """
    _instance = None
    _lock = threading.Lock()

    def __init__(self):
        self._devices = defaultdict(_DeviceMetrics)
        self._data_lock = threading.Lock()

    @classmethod
    def get_instance(cls):
        with cls._lock:
            if cls._instance is None:
                cls._instance = cls()
            return cls._instance

    # ── Record methods ──

    def record_msg(self, device_id: str, seq: int = 0):
        """Record one received message"""
        with self._data_lock:
            dm = self._devices[device_id]
            dm.total_msgs += 1
            if seq > 0:
                if dm.last_seq > 0 and seq > dm.last_seq + 1:
                    gap = seq - dm.last_seq - 1
                    dm.lost_packets += gap
                dm.last_seq = seq

    def record_crc_fail(self, device_id: str):
        """Record one CRC integrity failure"""
        with self._data_lock:
            self._devices[device_id].crc_failures += 1

    def record_loss(self, device_id: str, seq_from: int, seq_to: int):
        """Record detected packet loss range"""
        with self._data_lock:
            self._devices[device_id].lost_packets += (seq_to - seq_from + 1)

    def record_retransmit(self, device_id: str):
        """Record one retransmission request"""
        with self._data_lock:
            self._devices[device_id].retransmits += 1

    def record_success(self, device_id: str):
        """Record one successful survey processing"""
        with self._data_lock:
            self._devices[device_id].successes += 1

    def record_failure(self, device_id: str):
        """Record one failed survey processing"""
        with self._data_lock:
            self._devices[device_id].failures += 1

    def record_latency(self, device_id: str, ms: float):
        """Record a message round-trip latency sample (ms)"""
        with self._data_lock:
            dm = self._devices[device_id]
            dm.latency_samples.append(ms)
            if len(dm.latency_samples) > 1000:
                dm.latency_samples = dm.latency_samples[-500:]

    # ── Query methods ──

    def get_device_metrics(self, device_id: str) -> dict:
        """Get computed metrics for one device"""
        with self._data_lock:
            dm = self._devices.get(device_id)
            if not dm:
                return {}
            return self._compute(dm, device_id)

    def get_all_metrics(self) -> dict:
        """Get metrics for all devices"""
        with self._data_lock:
            result = {}
            for device_id, dm in self._devices.items():
                result[device_id] = self._compute(dm, device_id)
            return result

    def get_summary(self) -> dict:
        """Get aggregate metrics across all devices"""
        with self._data_lock:
            total = _DeviceMetrics()
            for dm in self._devices.values():
                total.total_msgs += dm.total_msgs
                total.lost_packets += dm.lost_packets
                total.crc_failures += dm.crc_failures
                total.retransmits += dm.retransmits
                total.successes += dm.successes
                total.failures += dm.failures
                total.latency_samples.extend(dm.latency_samples)
            total.start_time = min(
                (dm.start_time for dm in self._devices.values()), default=time.time()
            )
            return self._compute(total, "ALL_DEVICES")

    def reset(self, device_id: str = None):
        """Reset metrics for one device or all"""
        with self._data_lock:
            if device_id:
                self._devices.pop(device_id, None)
            else:
                self._devices.clear()

    @staticmethod
    def _compute(dm, device_id):
        """Compute rates and status from raw counters"""
        total = dm.total_msgs
        loss_rate = (dm.lost_packets / total * 100) if total > 0 else 0.0
        crc_rate = (dm.crc_failures / total * 100) if total > 0 else 0.0
        retrans_rate = (dm.retransmits / total * 100) if total > 0 else 0.0
        success_rate = (dm.successes / (dm.successes + dm.failures) * 100) \

if (dm.successes + dm.failures) > 0 else 0.0

        # Compute average latency
        avg_latency = 0.0
        if dm.latency_samples:
            avg_latency = sum(dm.latency_samples) / len(dm.latency_samples)

        # Overall status
        if loss_rate > 5.0 or crc_rate > 2.0:
            status = "CRITICAL"
        elif loss_rate > 2.0 or crc_rate > 1.0:
            status = "WARNING"
        else:
            status = "NORMAL"

        duration = time.time() - dm.start_time
        return {
            "device_id": device_id,
            "total_msgs": total,
            "lost_packets": dm.lost_packets,
            "loss_rate": round(loss_rate, 2),
            "crc_failures": dm.crc_failures,
            "crc_rate": round(crc_rate, 2),
            "retransmits": dm.retransmits,
            "retrans_rate": round(retrans_rate, 2),
            "successes": dm.successes,
            "failures": dm.failures,
            "success_rate": round(success_rate, 2),
            "avg_latency_ms": round(avg_latency, 1),
            "status": status,
            "duration_sec": int(duration),
            "start_time": dm.start_time,
        }

#### monitor/reporter.py

"""Network Health Check Zone — formatted terminal report

Usage:
    from monitor.reporter import print_check_zone
    print_check_zone()           # all devices
    print_check_zone("HW2024001")  # one device
    
    or via CLI:
    python -m monitor.reporter
"""

import time
from monitor.metrics_collector import MetricsCollector


def _bar(value, max_val=10, width=20):
    """Render a horizontal bar"""
    filled = int((value / max_val) * width) if max_val > 0 else 0
    filled = min(filled, width)
    bar = "#" * filled + "." * (width - filled)
    return bar


def _fmt_time(ts):
    return time.strftime("%Y-%m-%d %H:%M:%S", time.localtime(ts))


def _fmt_duration(sec):
    if sec < 60:
        return f"{sec}s"
    elif sec < 3600:
        return f"{sec//60}m {sec%60}s"
    else:
        return f"{sec//3600}h {(sec%3600)//60}m"


def generate_check_zone(device_id: str = None) -> str:
    """Generate the formatted Check Zone report string"""
    collector = MetricsCollector.get_instance()

    if device_id:
        metrics_list = [collector.get_device_metrics(device_id)]
    else:
        all_metrics = collector.get_all_metrics()
        metrics_list = list(all_metrics.values())

    if not metrics_list:
        return "+====================================+\n" \
               "|     Network Health Check Zone       |\n" \
               "+====================================+\n" \
               "|  No data collected yet.             |\n" \
               "|  Start the bridge and send survey   |\n" \
               "|  data to populate metrics.          |\n" \
               "+====================================+"

    lines = []
    lines.append("+======================================================+")
    lines.append("|         Network Health Check Zone                    |")
    lines.append("+======================================================+")

    for m in metrics_list:
        if not m:
            continue

        lines.append(f"| Device: {m['device_id']:<44}|")
        lines.append(f"| Since:  {_fmt_time(m['start_time']):<20}"
                     f"  Duration: {_fmt_duration(m['duration_sec']):>15} |")
        lines.append("|                                              |")
        lines.append("|  +---------------------+----------+---------+ |")

        def row(label, value, rate_str="", warn_at=0):
            status = " "
            if rate_str and rate_str != "N/A":
                rate = float(rate_str.rstrip("% "))
                if rate > 5.0:
                    status = "!"
                elif rate > 2.0:
                    status = "!"
                else:
                    status = "*"
            return f"|  | {label:<19} | {str(value):>8} | {rate_str:>7} {status}| |"

        lines.append(row("Total Messages", m['total_msgs']))
        lines.append(row("Lost Packets", m['lost_packets'],
                         f"{m['loss_rate']}%" if m['loss_rate'] > 0 else "0%"))
        lines.append(row("CRC Failures", m['crc_failures'],
                         f"{m['crc_rate']}%" if m['crc_rate'] > 0 else "0%"))
        lines.append(row("Retransmits", m['retransmits'],
                         f"{m['retrans_rate']}%" if m['retrans_rate'] > 0 else "0%"))
        lines.append(row("Success Rate",
                         f"{m['success_rate']}%" if m['success_rate'] > 0 else "N/A"))
        lines.append("|  +---------------------+----------+---------+ |")
        lines.append("|                                              |")

        # Visual bars
        max_rate = max(m['loss_rate'], m['crc_rate'], m['retrans_rate'], 1)
        max_bar = max(max_rate, 10)
        lines.append(f"|  Packet Loss:  {_bar(m['loss_rate'], max_bar)} {m['loss_rate']}%  |")
        lines.append(f"|  CRC Errors:   {_bar(m['crc_rate'], max_bar)} {m['crc_rate']}%  |")
        lines.append(f"|  Retransmits:  {_bar(m['retrans_rate'], max_bar)} {m['retrans_rate']}%  |")

        if m['avg_latency_ms'] > 0:
            lines.append(f"|  Avg Latency:  {m['avg_latency_ms']} ms                         |")

        lines.append("|                                              |")

        # Status
        status_icon = {"NORMAL": "*", "WARNING": "!", "CRITICAL": "!"}
        icon = status_icon.get(m['status'], "?")
        lines.append(f"|  Overall: {icon} {m['status']:<41}|")
        lines.append(f"|  Thresholds: loss<2% crc<1% latency<500ms     |")

    lines.append("+======================================================+")
    return "\n".join(lines)


def print_check_zone(device_id: str = None):
    """Print the Check Zone to terminal"""
    print(generate_check_zone(device_id))


def get_recommendations(metrics: dict) -> list:
    """Generate actionable recommendations based on metrics"""
    recs = []
    if metrics.get('crc_rate', 0) > 1.0:
        recs.append(f"! CRC errors at {metrics['crc_rate']}%: "
                    f"Check GPS module cable shielding, "
                    f"reduce electromagnetic interference")
    if metrics.get('loss_rate', 0) > 2.0:
        recs.append(f"! Packet loss at {metrics['loss_rate']}%: "
                    f"Weak network signal, consider directional antenna "
                    f"or cellular booster")
    if metrics.get('loss_rate', 0) > 5.0:
        recs.append(f"! Critical packet loss: "
                    f"Survey will have frequent failures. "
                    f"Move to area with better reception")
    if metrics.get('avg_latency_ms', 0) > 500:
        recs.append(f"! High latency ({metrics['avg_latency_ms']}ms): "
                    f"Consider closer MQTT broker or 5G network")
    if metrics.get('success_rate', 100) < 90:
        recs.append(f"! Low survey success rate ({metrics['success_rate']}%): "
                    f"Likely due to data corruption or network issues above")
    if not recs:
        recs.append(f"* All metrics within normal range. No action needed.")
    return recs

#### mqtt/__init__.py

#### mqtt/__main__.py

"""MQTT Bridge runner - start this on the server side"""
import logging
from logging_config import setup as setup_logging
setup_logging("mqtt")
logging.basicConfig(level=logging.INFO,
    format="%(asctime)s [%(levelname)s] %(name)s: %(message)s")

try:
    from dotenv import load_dotenv
    load_dotenv()
except ImportError:
    pass

from mqtt.bridge_service import SurveyBridge
bridge = SurveyBridge()
print("Starting MQTT bridge...")
bridge.run()

#### mqtt/bridge_service.py

﻿
"""MQTT bridge service 鈥?runs ON THE SERVER

Receives survey data from MQTT broker, processes via existing gateway.py,
and publishes results back.

Usage:
    from mqtt.bridge_service import SurveyBridge
    bridge = SurveyBridge()
    bridge.run()
"""

import json
import logging
import threading

import paho.mqtt.client as mqtt

from mqtt.crc import verify_message, SequenceTracker
from mqtt.config import MQTTConfig
from gateway import SurveyGateway
from monitor.metrics_collector import MetricsCollector
from plugins.base import auto_detect

logger = logging.getLogger(__name__)


class SurveyBridge:
    """MQTT 鈫?Gateway bridge: receives MQTT messages, calls gateway.process_survey()"""

    def __init__(self, mqtt_config=None, gateway=None):
        self.mqtt_cfg = mqtt_config or MQTTConfig(client_id="survey-bridge")
        self.gw = gateway or SurveyGateway()
        self.metrics = MetricsCollector.get_instance()
        self._trackers = {}  # device_id -> SequenceTracker
        self._client = self._build_client()

    def _build_client(self):
        client = mqtt.Client(
            client_id=self.mqtt_cfg.client_id,
            clean_session=True,
            protocol=mqtt.MQTTv311,
        )
        if self.mqtt_cfg.use_tls:
            ca = self.mqtt_cfg.ca_path or None
            cert = self.mqtt_cfg.cert_path or None
            key = self.mqtt_cfg.key_path or None
            if ca:
                client.tls_set(ca_certs=ca, certfile=cert, keyfile=key)
            else:
                client.tls_set()
        if self.mqtt_cfg.username:
            client.username_pw_set(self.mqtt_cfg.username, self.mqtt_cfg.password)
        client.on_connect = self._on_connect
        client.on_message = self._on_message
        return client

    def _on_connect(self, client, userdata, flags, rc):
        if rc == 0:
            logger.info("Bridge connected to %s", self.mqtt_cfg.broker_host)
            # Subscribe to all survey data topics
            client.subscribe("survey/data/#", qos=1)
        else:
            logger.error("Bridge connect failed: rc=%d", rc)

    def _on_message(self, client, userdata, msg):
        """Process incoming survey data 鈥?THIS IS THE ENTRY POINT"""
        try:
            payload = json.loads(msg.payload)
        except json.JSONDecodeError as e:
            logger.error("JSON parse error: %s", e)
            return  # QoS 1 will retry

        # 1. CRC integrity check
        if not verify_message(payload):
            logger.warning("CRC mismatch, discarding (QoS 1 will retry)")
            self.metrics.record_crc_fail(device_id)
            return

        device_id = payload.get("device_id", "unknown")

        # 2. Sequence gap detection
        seq = payload.get("seq", 0)
        if device_id not in self._trackers:
            self._trackers[device_id] = SequenceTracker()
        self.metrics.record_msg(device_id, seq)
        if gaps:
            for start, end in gaps:
                logger.warning("Missed seq %d-%d from %s, requesting retransmit", start, end, device_id)
                self.metrics.record_loss(device_id, start, end)
                self._request_retransmit(device_id, start, end)

        # 3. Extract survey data
        A = payload.get("A")
        B = payload.get("B")
        C = payload.get("C")
        if not all([A, B, C]):
            logger.warning("Incomplete survey data from %s", device_id)
            self._publish_response(device_id, {"status": "failed",
                                                "message": "Missing A/B/C data",
                                                "action": "RESET"})
            return

        # 4. Process via existing gateway
        try:
            ok, msg_text, result = self.gw.process_survey(
                device_id=device_id,
                A=A, B=B, C=C,
                batch_id=payload.get("batch_id"),
                survey_time=payload.get("survey_time"),
            )
            if ok:
                self.metrics.record_success(device_id)
                self._publish_response(device_id, {
                    "status": "success",
                    "batch_id": result.get("batch_id", ""),
                })
            else:
                self.metrics.record_failure(device_id)
                self._publish_response(device_id, {
                    "status": "failed",
                    "message": msg_text,
                    "action": "RESET",
                })
        except Exception as e:
            logger.error("process_survey error: %s", e, exc_info=True)
            self._publish_response(device_id, {
                "status": "error",
                "message": str(e),
            })

    def _process_json_payload(self, payload):
        """Process pre-parsed JSON (A/B/C format) from any source"""
        device_id = payload.get("device_id", "unknown")

        # CRC check
        if not verify_message(payload):
            logger.warning("CRC mismatch, discarding")
            self.metrics.record_crc_fail(device_id)
            return

        # Sequence tracking
        seq = payload.get("seq", 0)
        if device_id not in self._trackers:
            self._trackers[device_id] = SequenceTracker()
        gaps = self._trackers[device_id].check(seq)
        self.metrics.record_msg(device_id, seq)
        if gaps:
            for start, end in gaps:
                logger.warning("Missed seq %d-%d from %s", start, end, device_id)

self.metrics.record_loss(device_id, start, end)
                self._request_retransmit(device_id, start, end)

        # Extract A/B/C
        A = payload.get("A")
        B = payload.get("B")
        C = payload.get("C")
        if not all([A, B, C]):
            logger.warning("Incomplete survey data from %s", device_id)
            self._publish_response(device_id, {"status": "failed",
                                                "message": "Missing A/B/C data",
                                                "action": "RESET"})
            return

        # Process via gateway
        try:
            ok, msg_text, result = self.gw.process_survey(
                device_id=device_id, A=A, B=B, C=C,
                batch_id=payload.get("batch_id"),
                survey_time=payload.get("survey_time"),
            )
            if ok:
                self.metrics.record_success(device_id)
                self._publish_response(device_id, {
                    "status": "success",
                    "batch_id": result.get("batch_id", ""),
                })
            else:
                self.metrics.record_failure(device_id)
                self._publish_response(device_id, {
                    "status": "failed", "message": msg_text, "action": "RESET",
                })
        except Exception as e:
            logger.error("process_survey error: %s", e, exc_info=True)
            self._publish_response(device_id, {"status": "error", "message": str(e)})

    # Buffer for accumulating single-position fixes into A/B/C
    _fix_buffer = {}  # device_id -> [fix, fix, fix]

    def _process_single_fix(self, fix: dict, device_id: str = "unknown"):
        """Accumulate single position fixes (from NMEA/BESTPOS) into groups of 3

        Each fix is a dict with lat/lng/alt/h/v/d fields.
        When 3 fixes accumulate, calls process_survey() with A/B/C.
        """
        # Ensure position data exists
        if not fix or "lat" not in fix:
            return

        # Default H/V/D if not provided by parser
        fix.setdefault("h", 150.0)
        fix.setdefault("v", 200.0)
        fix.setdefault("d", 250.0)
        fix.setdefault("alt", 0.0)

        # Accumulate
        if device_id not in self._fix_buffer:
            self._fix_buffer[device_id] = []
        self._fix_buffer[device_id].append(fix)

        # When we have 3 fixes, dispatch
        if len(self._fix_buffer[device_id]) >= 3:
            fixes = self._fix_buffer[device_id][-3:]  # take last 3
            self._fix_buffer[device_id] = []  # reset buffer

            survey_time = fixes[0].get("survey_time", 0) or int(time.time())

            try:
                ok, msg_text, result = self.gw.process_survey(
                    device_id=device_id,
                    A=fixes[0], B=fixes[1], C=fixes[2],
                    survey_time=survey_time,
                )
                if ok:
                    self.metrics.record_success(device_id)
                    self._publish_response(device_id, {
                        "status": "success",
                        "batch_id": result.get("batch_id", ""),
                    })
                else:
                    self.metrics.record_failure(device_id)
                    self._publish_response(device_id, {
                        "status": "failed", "message": msg_text, "action": "RESET",
                    })
                    # Clear buffer on failure so next readings start fresh
                    self._fix_buffer[device_id] = []
            except Exception as e:
                logger.error("single fix error: %s", e, exc_info=True)
                self._publish_response(device_id, {"status": "error", "message": str(e)})
                self._fix_buffer[device_id] = []

    def _publish_response(self, device_id, data):
        """Publish result back to the device"""
        topic = f"survey/response/{device_id}"
        self._client.publish(topic, json.dumps(data), qos=1)

    def _request_retransmit(self, device_id, seq_from, seq_to):
        """Request retransmission of missed packets"""
        self.metrics.record_retransmit(device_id)
        topic = f"survey/retransmit/{device_id}"
        self._client.publish(topic, json.dumps({
            "action": "retransmit",
            "from": seq_from,
            "to": seq_to,
        }), qos=1)

    # 鈹€鈹€ Public API 鈹€鈹€

    def run(self, host=None, port=None):
        """Connect and start the bridge (blocking, Ctrl+C to stop)"""
        self._running = True
        self._client.connect(
            host or self.mqtt_cfg.broker_host,
            port or self.mqtt_cfg.broker_port,
            keepalive=self.mqtt_cfg.keepalive,
        )
        try:
            while self._running:
                self._client.loop(timeout=1.0)
        except KeyboardInterrupt:
            pass
        finally:
            self._running = False
            logger.info("Bridge stopping, flushing pending data...")
            # Cannot flush shard buffers here (they're in gateway calls)
            # but MQTT disconnect will clean up pending messages
            self._client.disconnect()

    def stop(self):
        """Request graceful shutdown (non-blocking)"""
        self._running = False

    def start_background(self):
        """Start bridge in background thread (non-blocking)"""
        t = threading.Thread(target=self.run, daemon=True)
        t.start()
        return t

#### mqtt/config.py

"""MQTT + device configuration (env vars > config dict > defaults)

All secrets/configurations come from environment variables in production.
For development, you can pass a dict or use a config.yaml file.

Priority: env var > config dict > default value
"""

import os
import logging

logger = logging.getLogger(__name__)


def env_or(key, default=""):
    """Get from env var (case-insensitive), fallback to default"""
    return os.environ.get(key, os.environ.get(key.upper(), default))


class MQTTConfig:
    """MQTT connection configuration"""

    def __init__(self, **overrides):
        self._overrides = overrides

    @property
    def broker_host(self) -> str:
        return self._overrides.get("broker_host") or env_or("MQTT_HOST", "localhost")

    @property
    def broker_port(self) -> int:
        return int(self._overrides.get("broker_port") or env_or("MQTT_PORT", "8883"))

    @property
    def use_tls(self) -> bool:
        val = self._overrides.get("use_tls") or env_or("MQTT_TLS", "true")
        return str(val).lower() in ("true", "1", "yes")

    @property
    def ca_path(self) -> str:
        return self._overrides.get("ca_path") or env_or("MQTT_CA", "")

    @property
    def cert_path(self) -> str:
        return self._overrides.get("cert_path") or env_or("MQTT_CERT", "")

    @property
    def key_path(self) -> str:
        return self._overrides.get("key_path") or env_or("MQTT_KEY", "")

    @property
    def username(self) -> str:
        return self._overrides.get("username") or env_or("MQTT_USER", "")

    @property
    def password(self) -> str:
        return self._overrides.get("password") or env_or("MQTT_PASS", "")

    @property
    def client_id(self) -> str:
        return self._overrides.get("client_id") or env_or("DEVICE_ID", "survey-device")

    @property
    def keepalive(self) -> int:
        return int(self._overrides.get("keepalive") or env_or("MQTT_KEEPALIVE", "60"))


class DeviceConfig:
    """Device survey configuration"""

    def __init__(self, **overrides):
        self._overrides = overrides

    @property
    def device_id(self) -> str:
        return self._overrides.get("device_id") or env_or("DEVICE_ID", "DEV-001")

    @property
    def hmac_key(self) -> str:
        return self._overrides.get("hmac_key") or env_or("HMAC_KEY", "")

    @property
    def survey_interval(self) -> int:
        return int(self._overrides.get("survey_interval") or env_or("SURVEY_INTERVAL", "60"))

    @property
    def gps_port(self) -> str:
        return self._overrides.get("gps_port") or env_or("GPS_PORT", "/dev/ttyAMA0")

    @property
    def gps_baud(self) -> int:
        return int(self._overrides.get("gps_baud") or env_or("GPS_BAUD", "9600"))

#### mqtt/crc.py

"""Data integrity: CRC32 checksum + sequence tracking per message

Each MQTT message carries:
  - crc: CRC32 of all other fields (detect corruption)
  - seq: monotonic sequence number (detect gaps/drops)
"""

import zlib
import json
import logging

logger = logging.getLogger(__name__)


def compute_crc(data: dict) -> int:
    """Compute CRC32 of a dict (sort keys for deterministic result)"""
    payload = json.dumps(data, sort_keys=True, separators=(",", ":"))
    return zlib.crc32(payload.encode()) & 0xFFFFFFFF


def verify_message(msg: dict) -> bool:
    """Verify the CRC32 field in a message matches the content

    The msg dict must contain a 'crc' key with the expected CRC value.
    Returns True if CRC matches or no CRC field (backward compat).
    """
    expected = msg.get("crc")
    if expected is None:
        return True  # no CRC = skip check (backward compat)
    # Recompute without the crc field
    check_data = dict(msg)
    check_data.pop("crc", None)
    actual = compute_crc(check_data)
    if actual != expected:
        logger.warning("CRC mismatch: expected=%s actual=%s", expected, actual)
        return False
    return True


def add_crc(msg: dict) -> dict:
    """Add CRC32 field to a message before sending"""
    msg["crc"] = compute_crc(msg)
    return msg


class SequenceTracker:
    """Track sequence numbers to detect message gaps

    Usage:
        tracker = SequenceTracker()
        gaps = tracker.check(seq=7)  # returns [(5,6)] if seq 5,6 were missed
    """
    def __init__(self, initial_seq=0):
        self._last_seq = initial_seq

    def check(self, seq: int) -> list:
        """Return list of (from, to) tuple ranges that were missed"""
        if seq <= self._last_seq:
            return []  # duplicate or out-of-order, ignore
        gaps = []
        if seq > self._last_seq + 1:
            gaps.append((self._last_seq + 1, seq - 1))
        self._last_seq = seq
        return gaps

    @property
    def last_seq(self) -> int:
        return self._last_seq

#### plugins/__init__.py

#### plugins/base.py

"""Parser plugin base class

All parsers must inherit ParserPlugin and implement:
  - can_handle(raw) -> bool
  - parse(raw) -> dict | None
"""

class ParserPlugin:
    """Base class for format parsers (NMEA, BESTPOS, etc.)"""

    @staticmethod
    def can_handle(raw) -> bool:
        """Return True if this parser can handle the given raw data"""
        raise NotImplementedError

    @staticmethod
    def parse(raw) -> dict or None:
        """Parse raw data into standard format.

        Returns dict with keys:
          - position: {"lat": float, "lng": float, "alt": float,
                        "h": float, "v": float, "d": float}
          - device_id: str (optional, auto-detect or blank)
          - survey_time: int (optional, unix timestamp)
          - quality: int (optional, 0-1 for validity)
        Returns None if parsing fails.
        """
        raise NotImplementedError


def auto_detect(raw) -> list:
    """Try all registered plugins in order. Returns [(name, result), ...]"""
    results = []
    # Late imports to avoid circular deps
    from .nmea import NMEAParser
    from .bestpos import BESTPOSParser, BESTPOSASCIIParser

    for name, parser in [("nmea", NMEAParser), ("bestpos", BESTPOSParser), ("bestposa", BESTPOSASCIIParser)]:
        try:
            if parser.can_handle(raw):
                result = parser.parse(raw)
                if result:
                    results.append((name, result))
        except Exception as e:
            import logging
            logging.getLogger(__name__).warning("Parser %s error: %s", name, e)
    return results

#### plugins/bestpos.py

"""BESTPOS (NovAtel binary) parser plugin

Handles: NovAtel OEM7 binary log format
Output: standard position dict with stddev-based accuracy

Binary header sync: 0xAA 0x44 0x12 0x1C
Message ID for BESTPOS: 42 (0x002A)
"""

import struct
import time
from .base import ParserPlugin


class BESTPOSParser(ParserPlugin):
    """Parse NovAtel BESTPOS binary logs"""

    SYNC = b"\xAA\x44\x12\x1C"
    MSG_ID_BESTPOS = 42

    # Position type enum
    POS_TYPE = {
        0: "NONE",
        32: "SINGLE",
        16: "RTXDGPS",
        4: "RTKFLOAT",
        5: "RTKFIXED",
    }

    @staticmethod
    def can_handle(raw) -> bool:
        if isinstance(raw, str):
            return False
        if not isinstance(raw, (bytes, bytearray)):
            return False
        return len(raw) >= 80 and raw[:4] == BESTPOSParser.SYNC

    @staticmethod
    def parse(raw: bytes) -> dict or None:
        """Parse BESTPOS binary message -> position dict

        Expects at least 84 bytes:
          - 28 byte header
          - ~52 byte body (varies by version)
          - 4 byte CRC
        """
        if not BESTPOSParser.can_handle(raw):
            return None

        try:
            # Header (28 bytes)
            # struct format: <HHHHIIBBxI (sync already checked)
            hdr = struct.unpack("<HHHHIIBBxI", raw[4:32])
            msg_id = hdr[0]
            msg_len = hdr[4]

            if msg_id != BESTPOSParser.MSG_ID_BESTPOS:
                return None

            # Body - common version (offset 28).
            # Fields: solstat(4B), postype(4B), lat(4B), lng(4B),
            #         hgt(8B), undulation(8B), vel(4B), heading(4B),
            #         num_tracked(2B), num_used(2B),
            #         hdop(4B), vdop(4B), pdop(4B),
            #         lat_std(8B), lng_std(8B), hgt_std(8B)
            body = raw[28:84]

            # Unpack body
            (solstat, postype, lat, lng,
             hgt, undulation,
             vel, heading,
             num_tracked, num_used,
             hdop, vdop, pdop,
             lat_std, lng_std, hgt_std) = struct.unpack("<IIffddffHHffffdddd", body[:72])

            # Check solution validity
            quality = 1 if solstat == 4 else 0  # SOLSTAT = 4 means valid

            # Standard deviations are in meters. Convert to cm.
            h_cm = round(lat_std * 100, 2) if lat_std > 0 else 150.0
            v_cm = round(lng_std * 100, 2) if lng_std > 0 else 200.0
            d_cm = round(hgt_std * 100, 2) if hgt_std > 0 else 250.0

            return {
                "lat": round(lat, 6),
                "lng": round(lng, 6),
                "alt": round(hgt, 2),
                "h": h_cm,
                "v": v_cm,
                "d": d_cm,
                "quality": quality,
                "num_sats": num_used,
                "postype": BESTPOSParser.POS_TYPE.get(postype, f"UNKNOWN({postype})"),
                "source": "BESTPOS",
            }
        except (struct.error, IndexError) as e:
            import logging
            logging.getLogger(__name__).warning("BESTPOS parse error: %s", e)
            return None

class BESTPOSASCIIParser(ParserPlugin):
    """Parse NovAtel BESTPOS ASCII logs (#BESTPOSA format)

    Each line is a complete position fix with std dev values.
    Fields: lat[11], lng[12], alt[13], std_lat[17], std_lng[18], std_hgt[19]

    Std dev (meters) → ENU precision (cm): e=std_lat*100, n=std_lng*100, u=std_hgt*100
    """

    @staticmethod
    def can_handle(raw) -> bool:
        if isinstance(raw, bytes):
            try:
                raw = raw.decode("utf-8", errors="ignore")
            except Exception:
                return False
        return raw.strip().startswith("#BESTPOSA")

    @staticmethod
    def parse(raw) -> dict or None:
        """Parse one #BESTPOSA line → position dict with e,n,u precision"""
        if isinstance(raw, bytes):
            raw = raw.decode("utf-8", errors="ignore")
        raw = raw.strip()
        if not raw.startswith("#BESTPOSA"):
            return None

        parts = raw.split(",")
        if len(parts) < 24:
            return None

        try:
            lat = float(parts[11])
            lng = float(parts[12])
            alt = float(parts[13])

            # Std dev (meters) -> cm precision
            std_lat = float(parts[16]) if parts[16] else 0.005
            std_lng = float(parts[17]) if parts[17] else 0.006
            std_hgt = float(parts[18]) if parts[18] else 0.015

            e_cm = round(std_lat * 100, 2)
            n_cm = round(std_lng * 100, 2)
            u_cm = round(std_hgt * 100, 2)

            # Position type
            pos_type = parts[10] if len(parts) > 10 else "UNKNOWN"
            quality = 1
            if pos_type == "NARROW_INT":
                quality = 1  # RTK fixed - best

elif pos_type == "NARROW_FLOAT":
                quality = 2  # RTK float - good
            elif pos_type == "SINGLE":
                quality = 3  # Single point - poor
                e_cm = max(e_cm, 50.0)  # floor at 50cm for single
                n_cm = max(n_cm, 50.0)
                u_cm = max(u_cm, 100.0)

            return {
                "lat": round(lat, 6),
                "lng": round(lng, 6),
                "alt": round(alt, 2),
                "e": e_cm,
                "n": n_cm,
                "u": u_cm,
                "quality": quality,
                "pos_type": pos_type,
                "num_sats": int(parts[23]) if parts[23] else 0,
                "source": "BESTPOSA",
            }
        except (ValueError, IndexError):
            return None

#### plugins/cm510_parser.py

"""CM510 设备协议解析插件

CM510是一款无线数据传输设备，支持：
- GPS定位数据上报
- 设备状态数据上报
- 心跳数据上报

报文格式：
$CM510,<device_id>,<lat>,<lng>,<speed>,<heading>,<altitude>,<satellites>,<signal>,<timestamp>*<CRC>
"""
import re, time, logging

logger = logging.getLogger(__name__)

# 预编译正则
_CM510_PATTERN = re.compile(r"^\$CM510,(\w+),([^,]*),([^,]*),([^,]*),([^,]*),([^,]*),([^,]*),([^,]*),([^,]*)\*(\w{2})$")
_NMEA_LAT = re.compile(r"^(\d{2})(\d{2}\.\d+)$")
_NMEA_LNG = re.compile(r"^(\d{3})(\d{2}\.\d+)$")


class CM510Parser:
    """CM510 设备协议解析器"""

    @staticmethod
    def can_parse(data) -> bool:
        if isinstance(data, bytes):
            try:
                data = data.decode("utf-8", errors="ignore")
            except Exception:
                return False
        return isinstance(data, str) and data.startswith("$CM510")

    @staticmethod
    def parse(data) -> dict or None:
        """解析 CM510 报文 -> 统一位置字典"""
        if isinstance(data, bytes):
            data = data.decode("utf-8", errors="ignore")
        data = data.strip()
        
        m = _CM510_PATTERN.match(data)
        if not m:
            return None
        
        try:
            device_id = m.group(1)
            lat = CM510Parser._parse_nmea_coord(m.group(2), "N")
            lng = CM510Parser._parse_nmea_coord(m.group(3), "E")
            speed = float(m.group(4)) if m.group(4) else 0.0
            heading = float(m.group(5)) if m.group(5) else 0.0
            altitude = float(m.group(6)) if m.group(6) else 0.0
            satellites = int(m.group(7)) if m.group(7) else 0
            signal = int(m.group(8)) if m.group(8) else 0
            timestamp = int(m.group(9)) if m.group(9) else int(time.time())
            
            return {
                "device_id": device_id,
                "lat": round(lat, 6),
                "lng": round(lng, 6),
                "alt": round(altitude, 2),
                "speed": round(speed, 2),
                "heading": round(heading, 2),
                "satellites": satellites,
                "signal_level": signal,
                "timestamp": timestamp,
                "pos_type": "SINGLE" if satellites < 4 else "NARROW_FLOAT" if satellites < 8 else "NARROW_INT",
                "source": "CM510",
            }
        except (ValueError, IndexError) as e:
            logger.warning("CM510 parse error: %s", e)
            return None

    @staticmethod
    def _parse_nmea_coord(raw: str, hemi: str) -> float:
        """NMEA 坐标格式 -> 十进制"""
        if not raw:
            return 0.0
        if "." not in raw:
            return float(raw)
        try:
            if len(raw.split(".")[0]) == 5:  # DDDMM.MMMM (经度)
                deg = int(raw[:3])
                minute = float(raw[3:])
            else:  # DDMM.MMMM (纬度)
                deg = int(raw[:2])
                minute = float(raw[2:])
            result = deg + minute / 60.0
            return -result if hemi in ("S", "W") else result
        except (ValueError, IndexError):
            return 0.0


# 注册到自动检测链
_PARSER_INSTANCE = None
def get_default():
    global _PARSER_INSTANCE
    if _PARSER_INSTANCE is None:
        _PARSER_INSTANCE = CM510Parser()
    return _PARSER_INSTANCE

#### plugins/cm510_plugin.py

"""CM510 协议解析插件（cm510_plugin 别名）

文档3.docx line274要求此文件名。
实际解析逻辑在 plugins/cm510_parser.py。
"""
from plugins.cm510_parser import CM510Parser as CM510Plugin

__all__ = ["CM510Plugin"]

#### plugins/gps_analyzer.py

"""GPS 数据分析插件

分析 GPS 定位数据质量：漂移检测、跳点检测、精度评估。
"""
import logging, time
from upgrade.plugin_sdk import BasePlugin

logger = logging.getLogger(__name__)

class GPSAnalyzerPlugin(BasePlugin):
    @property
    def name(self): return "gps_analyzer"
    @property
    def version(self): return "0.1.0"

    def on_event(self, data):
        payload = data.get("data", {})
        lat, lng = payload.get("lat", 0), payload.get("lng", 0)
        e, n, u = payload.get("e", 0), payload.get("n", 0), payload.get("u", 0)
        issues = []
        if not (20 < lat < 40): issues.append(f"lat={lat} out of range")
        if not (100 < lng < 130): issues.append(f"lng={lng} out of range")
        if e > 0.1 or n > 0.1: issues.append("precision degraded")
        valid = len(issues) == 0
        return {
            "status": "success",
            "result": {"valid": valid, "issues": issues, "quality": "good" if valid else "poor"}
        }

#### plugins/gps_plugin.py

"""5.docx §6.1 GPS插件"""
from upgrade.plugin_sdk import BasePlugin

class GPSPlugin(BasePlugin):
    @property
    def name(self): return "gps_plugin"
    @property
    def version(self): return "0.1.0"
    def on_event(self, data):
        payload = data.get("data", {})
        lat = float(payload.get("lat", 0))
        lng = float(payload.get("lng", 0))
        valid = 20 < lat < 40 and 100 < lng < 130
        return {"status": "success", "result": {"valid": valid, "data": payload}}

#### plugins/k803_parser.py

"""K803 GNSS 模块报文解析插件

解析 K803 EK0407 板卡输出的自定义定位报文。
"""
import re, logging
from upgrade.plugin_sdk import BasePlugin

logger = logging.getLogger(__name__)

class K803ParserPlugin(BasePlugin):
    @property
    def name(self): return "k803_parser"
    @property
    def version(self): return "0.1.0"

    def on_event(self, data):
        payload = data.get("data", {})
        raw = payload.get("raw", "")
        if not raw:
            return {"status": "fail", "result": {}, "error": "no raw data"}
        parsed = self._parse(raw)
        if parsed:
            return {"status": "success", "result": {"parsed": parsed}}
        return {"status": "success", "result": {"parsed": None}}

    def _parse(self, raw):
        # K803 典型报文: $K803,lat,lng,alt,quality*XX
        m = re.match(r"^$K803,([d.]+),([d.]+),([d.]+),(d+)", raw)
        if m:
            return {"lat": float(m.group(1)), "lng": float(m.group(2)),
                    "alt": float(m.group(3)), "quality": int(m.group(4)),
                    "format": "K803"}
        return None

#### plugins/nmea.py

"""NMEA 0183 parser plugin

Handles: $GPGGA, $GPGSA, $GPRMC, $GPGSV
Output: standard position dict

Usage:
    from plugins.nmea import NMEAParser
    if NMEAParser.can_handle(raw):
        result = NMEAParser.parse(raw)
"""

import time
from .base import ParserPlugin


class NMEAParser(ParserPlugin):
    """Parse NMEA 0183 sentences into standard position format"""

    # Buffer: device_id -> [fix_dict, ...] accumulate 3 fixes for A/B/C
    _buffer = {}

    @staticmethod
    def can_handle(raw) -> bool:
        if isinstance(raw, bytes):
            try:
                raw = raw.decode("ascii", errors="ignore")
            except Exception:
                return False
        return raw.startswith("$GP") or raw.startswith("$GN")

    @staticmethod
    def parse_gga(sentence: str) -> dict or None:
        """Parse $GPGGA sentence -> position dict"""
        parts = sentence.split(",")
        if len(parts) < 10 or not parts[2] or not parts[4]:
            return None
        try:
            if parts[6] in ("", "0"):
                return None  # no fix

            # Latitude: DDMM.MMMM
            raw_lat = parts[2]
            lat = int(raw_lat[:2]) + float(raw_lat[2:]) / 60.0
            if parts[3] == "S":
                lat = -lat

            # Longitude: DDDMM.MMMM
            raw_lng = parts[4]
            lng = int(raw_lng[:3]) + float(raw_lng[3:]) / 60.0
            if parts[5] == "W":
                lng = -lng

            alt = float(parts[9]) if parts[9] else 0.0
            quality = int(parts[6]) if parts[6] else 0
            num_sats = int(parts[7]) if parts[7] else 0

            return {
                "lat": round(lat, 6),
                "lng": round(lng, 6),
                "alt": round(alt, 2),
                "quality": quality,
                "num_sats": num_sats,
                "source": "GGA",
            }
        except (ValueError, IndexError):
            return None

    @staticmethod
    def parse_gsa(sentence: str) -> dict or None:
        """Parse $GPGSA sentence -> DOP values"""
        parts = sentence.split(",")
        if len(parts) < 18:
            return None
        try:
            pdop = float(parts[15]) if parts[15] else 0.0
            hdop = float(parts[16]) if parts[16] else 0.0
            vdop = float(parts[17].split("*")[0]) if parts[17] else 0.0
            return {"hdop": hdop, "vdop": vdop, "pdop": pdop, "source": "GSA"}
        except (ValueError, IndexError):
            return None

    @staticmethod
    def dop_to_cm(dop: float, base_accuracy_cm: float = 150.0) -> float:
        """Convert DOP to approximate cm accuracy

        HDOP/VDOP are unitless multipliers.
        Common formula: accuracy = DOP * UERE (user equivalent range error)
        For consumer GPS, typical UERE = 1.5m -> 150cm
        """
        return round(dop * base_accuracy_cm, 2)

    @classmethod
    def parse(cls, raw) -> dict or None:
        """Parse one NMEA sentence. Returns position dict or None."""
        if isinstance(raw, bytes):
            raw = raw.decode("ascii", errors="ignore")

        raw = raw.strip()
        if not raw:
            return None

        # Route to specific parser based on sentence type
        sentence_type = raw[3:6] if len(raw) > 6 else ""

        if sentence_type == "GGA":
            return cls.parse_gga(raw)
        elif sentence_type == "GSA":
            return cls.parse_gsa(raw)
        elif sentence_type == "RMC":
            # RMC has timestamp but no position in a usable format for us
            # Just skip - we only use GGA for position
            return None
        elif sentence_type in ("GSV", "GLL", "VTG"):
            return None  # supplementary data, not primary position

        return None

    @classmethod
    def parse_position_with_dop(cls, gga_raw: str, gsa_raw: str = None) -> dict or None:
        """Parse GGA + optional GSA into complete position with accuracy estimates"""
        pos = cls.parse_gga(gga_raw)
        if not pos:
            return None

        # Try to get DOP values for H/V/D estimation
        if gsa_raw:
            dop = cls.parse_gsa(gsa_raw)
            if dop:
                pos["h"] = cls.dop_to_cm(dop["hdop"])
                pos["v"] = cls.dop_to_cm(dop["vdop"])
                pos["d"] = cls.dop_to_cm(dop["pdop"])
        else:
            # Default accuracy values if no DOP available
            pos["h"] = 150.0
            pos["v"] = 200.0
            pos["d"] = 250.0

        return pos

#### plugins/sdk/__init__.py

"""Plugin SDK 标准化包"""
from .schema import PluginInput, PluginOutput, EventSchema
from .context import PluginContext

#### plugins/sdk/base.py

"""Plugin SDK Base — 插件标准基类

符合 V4-Local Step4 规范。
重导出 upgrade/plugin_sdk 的 BasePlugin。
"""
from upgrade.plugin_sdk import BasePlugin, PluginContext, PluginInput, PluginOutput
__all__ = ["BasePlugin", "PluginContext", "PluginInput", "PluginOutput"]

#### plugins/sdk/context.py

"""V4-Local Plugin SDK 上下文管理"""
from dataclasses import dataclass, field

@dataclass
class PluginContext:
    plugin_name: str = ""
    plugin_version: str = "0.1.0"
    config: dict = field(default_factory=dict)
    shared_storage: dict = field(default_factory=dict)

#### plugins/sdk/schema.py

"""V4-Local Plugin SDK Schema 标准"""
from dataclasses import dataclass, field, asdict
from typing import Optional, Any

@dataclass
class PluginInput:
    topic: str = ""
    data: dict = field(default_factory=dict)
    source: str = ""
    timestamp: float = 0.0

@dataclass
class PluginOutput:
    status: str = "success"
    result: dict = field(default_factory=dict)
    error: str = ""
    processing_ms: float = 0.0

@dataclass
class EventSchema:
    id: str = ""
    topic: str = ""
    source: str = ""
    payload: dict = field(default_factory=dict)
    timestamp: float = 0.0

    def to_dict(self) -> dict:
        return asdict(self)

    @classmethod
    def from_dict(cls, d: dict) -> "EventSchema":
        return cls(**{k: v for k, v in d.items() if k in cls.__dataclass_fields__})

#### processor/__init__.py

"""5.docx §4 Processor层 — 数据标准化处理"""
from .gps_processor import GPSProcessor
from .cm510_parser import CM510Processor
from .k803_parser import K803Processor
from .rule_engine import RuleEngine

#### processor/cm510_parser.py

"""5.docx §4.2 CM510处理器"""
class CM510Processor:
    def process(self, raw: dict) -> dict:
        data = raw.get("data", raw)
        return {
            "device_id": raw.get("device_id", data.get("device_id", "cm510")),
            "status": "ok",
            "data": {"signal": data.get("signal", ""), "raw": str(data)}
        }

#### processor/gps_processor.py

"""5.docx §4.2 GPS处理器"""
import time, logging
logger = logging.getLogger(__name__)

class GPSProcessor:
    def process(self, raw: dict) -> dict:
        return {
            "device_id": raw.get("device_id", "gps"),
            "status": "ok",
            "data": {
                "lat": raw.get("lat", 0),
                "lng": raw.get("lng", 0),
                "alt": raw.get("alt", 0),
                "e": raw.get("e", 0),
                "n": raw.get("n", 0),
                "u": raw.get("u", 0),
            }
        }

#### processor/k803_parser.py

"""5.docx §4.2 K803处理器"""
import re
class K803Processor:
    def process(self, raw: dict) -> dict:
        data = raw.get("data", {})
        raw_str = data.get("raw", "") if isinstance(data, dict) else ""
        parsed = None
        m = re.search(r"$K803,([d.]+),([d.]+),([d.]+),(d+)", str(raw))
        if m:
            parsed = {"lat": float(m.group(1)), "lng": float(m.group(2)),
                      "alt": float(m.group(3)), "quality": int(m.group(4))}
        return {"device_id": raw.get("device_id", "k803"), "status": "ok",
                "data": parsed or {"raw": str(raw)}}

#### processor/rule_engine.py

"""5.docx §4.2 Rule Engine"""
import time, logging
logger = logging.getLogger(__name__)

class RuleEngine:
    def evaluate(self, device_id: str, data: dict) -> dict:
        issues = []
        lat = float(data.get("lat", 0))
        lng = float(data.get("lng", 0))
        if not (20 < lat < 40): issues.append(f"lat={lat}")
        if not (100 < lng < 130): issues.append(f"lng={lng}")
        e, n, u = float(data.get("e", 0)), float(data.get("n", 0)), float(data.get("u", 0))
        if max(e, n, u) > 0.1: issues.append("precision")
        valid = len(issues) == 0
        return {"device_id": device_id, "valid": valid, "issues": issues, "result": 1 if valid else 0}

#### replay/__init__.py

"""V4-Local GPS 回放系统"""

#### replay/file_replay.py

"""5.docx §5.1 文件回放"""
import time, logging, os, sys
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))
from eventbus import Event, Topic, bus
from replay.timeline_engine import TimelineEngine

logger = logging.getLogger(__name__)

class FileReplay:
    def __init__(self, speed=1.0):
        self.speed = speed
        self.timeline = TimelineEngine()
        self._replayed = 0

    def load(self, filepath):
        if not os.path.exists(filepath): raise FileNotFoundError(filepath)
        count = 0
        with open(filepath, "r", encoding="utf-8") as f:
            for line in f:
                line = line.strip()
                if not line: continue
                topic = Topic.GPS_DATA if line.startswith(("#","$")) else Topic.DEVICE_RAW
                self.timeline.add_event({"topic": topic, "raw": line}, 0.05)
                count += 1
        logger.info("Loaded %d lines from %s", count, filepath)
        return count

    def geo(self, lat=30.0, lng=120.5, count=10, alt=50.0):
        for i in range(count):
            self.timeline.add_event({
                "topic": Topic.GPS_DATA,
                "data": {"lat": lat+i*0.001, "lng": lng+i*0.001, "alt": alt, "e":0.01, "n":0.01, "u":0.02}
            }, 0.1)
        return count

    def run(self, source="replay"):
        events = self.timeline.get_events()
        for ev in events:
            t = ev.get("topic", Topic.GPS_DATA)
            d = ev.get("data", ev)
            bus.publish(t, Event(topic=t, device_id=source, data=d))
            self._replayed += 1
            time.sleep(self.timeline.get_delay(self._replayed-1, 0.05) / self.speed)
        return self._replayed

    def stats(self):
        return {"loaded": self.timeline.count(), "replayed": self._replayed, "speed": self.speed}

#### replay/gps_replay.py

"""V4-Local GPS 数据回放引擎

功能：
- 从文件加载 GPS 报文并按时间线回放
- 支持延时模拟（真实时间比例/加速）
- 通过 EventBus 发布回放事件
- 支持循环回放
"""
import time, logging, json, os, sys
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

from upgrade.eventbus import Event, Topics, bus
from .timeline_engine import TimelineEngine

logger = logging.getLogger(__name__)

class GPSReplayEngine:
    def __init__(self, speed: float = 1.0):
        self.speed = speed
        self.timeline = TimelineEngine()
        self._running = False
        self._replayed = 0

    def load_file(self, filepath: str) -> int:
        """从文件加载 GPS/报文数据"""
        if not os.path.exists(filepath):
            raise FileNotFoundError(f"文件不存在: {filepath}")
        count = 0
        with open(filepath, "r", encoding="utf-8") as f:
            for line in f:
                line = line.strip()
                if not line:
                    continue
                # 自动检测格式
                if line.startswith("#BESTPOSA"):
                    ev = {"type": "bestpos", "raw": line}
                elif line.startswith("$"):
                    ev = {"type": "nmea", "raw": line}
                else:
                    ev = {"type": "json", "raw": line}
                self.timeline.add_event(ev)
                count += 1
        logger.info("Loaded %d events from %s", count, filepath)
        return count

    def load_data(self, data_list: list) -> int:
        """直接加载数据列表"""
        for item in data_list:
            self.timeline.add_event(item)
        return len(data_list)

    def run(self, loop: bool = False, source: str = "replay") -> int:
        """执行回放"""
        events = self.timeline.get_events()
        if not events:
            logger.warning("No events to replay")
            return 0
        self._running = True
        self._replayed = 0
        start_time = time.time()
        for i, ev in enumerate(events):
            if not self._running:
                break
            # 发布到 EventBus
            if ev.get("type") == "bestpos":
                payload = {"raw": ev["raw"], "replay_index": i}
                bus.publish(Topics.DEVICE_RAW, payload, source=source)
            elif ev.get("type") == "nmea":
                payload = {"raw": ev["raw"], "replay_index": i}
                bus.publish(Topics.GPS_DATA, payload, source=source)
            else:
                payload = {**ev, "replay_index": i}
                bus.publish(Topics.GPS_DATA, payload, source=source)
            self._replayed += 1
            # 模拟延时（按速度比例）
            delay = self.timeline.get_delay(i, default=0.1)
            if delay > 0 and i < len(events) - 1:
                time.sleep(delay / self.speed)
        elapsed = time.time() - start_time
        logger.info("Replay done: %d events in %.2fs (speed=%.1fx)", self._replayed, elapsed, self.speed)
        return self._replayed

    def run_geo(self, lat: float, lng: float, alt: float, count: int = 1,
                source: str = "replay:geo") -> int:
        """回放指定坐标的模拟 GPS 数据"""
        for i in range(count):
            payload = {"lat": lat, "lng": lng, "alt": alt,
                       "replay_index": i, "type": "geo_sim"}
            bus.publish(Topics.GPS_DATA, payload, source=source)
            self._replayed += 1
            time.sleep(0.1)
        return count

    def stop(self):
        self._running = False

    def stats(self) -> dict:
        return {
            "events_loaded": self.timeline.count(),
            "events_replayed": self._replayed,
            "speed": self.speed,
            "running": self._running,
        }

#### replay/signal_replay.py

"""V4-Local 信号回放系统 — 通用信号重放

与 GPSReplayEngine 共享 TimelineEngine，
支持从文件回放各种类型信号。
"""
import time, logging, os, sys
sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

from upgrade.eventbus import Event, Topics, bus
from replay.timeline_engine import TimelineEngine

logger = logging.getLogger(__name__)

class SignalReplayEngine:
    def __init__(self, speed=1.0):
        self.speed = speed
        self.timeline = TimelineEngine()
        self._replayed = 0

    def load_file(self, filepath):
        if not os.path.exists(filepath): raise FileNotFoundError(filepath)
        count = 0
        with open(filepath, "r", encoding="utf-8") as f:
            for line in f:
                line = line.strip()
                if not line: continue
                ev = {"type": "raw", "raw": line, "line_num": count + 1}
                self.timeline.add_event(ev, delay_after=0.05)
                count += 1
        logger.info("Loaded %d signals from %s", count, filepath)
        return count

    def run(self, source="signal_replay"):
        events = self.timeline.get_events()
        for i, ev in enumerate(events):
            payload = {**ev, "replay_idx": i}
            topic = Topics.DEVICE_RAW
            if ev.get("type") == "raw":
                raw = ev.get("raw", "")
                if raw.startswith("#BESTPOSA"): topic = Topics.DEVICE_RAW
                elif raw.startswith("$"): topic = Topics.GPS_DATA
            bus.publish(topic, payload, source=source)
            self._replayed += 1
            delay = self.timeline.get_delay(i, 0.05)
            if delay > 0: time.sleep(delay / self.speed)
        return self._replayed

    def stats(self):
        return {"events_loaded": self.timeline.count(), "replayed": self._replayed, "speed": self.speed}

#### replay/timeline_engine.py

"""V4-Local 时间线引擎 — 管理回放事件序列和时间间隔"""
import time, logging

logger = logging.getLogger(__name__)

class TimelineEvent:
    def __init__(self, data: dict, timestamp: float = 0.0):
        self.data = data
        self.timestamp = timestamp or time.time()
        self.delay_after = 0.0  # 秒

    def to_dict(self) -> dict:
        return {**self.data, "_ts": self.timestamp, "_delay": self.delay_after}

class TimelineEngine:
    def __init__(self):
        self._events: list[TimelineEvent] = []
        self._index = 0

    def add_event(self, data: dict, delay_after: float = 0.1):
        """添加事件，delay_after 为该事件后的等待时间"""
        ev = TimelineEvent(data, delay_after=delay_after)
        self._events.append(ev)

    def add_events(self, data_list: list[dict], delay: float = 0.1):
        for d in data_list:
            self.add_event(d, delay)

    def get_events(self) -> list[dict]:
        return [ev.to_dict() for ev in self._events]

    def get_delay(self, index: int, default: float = 0.1) -> float:
        if 0 <= index < len(self._events):
            return self._events[index].delay_after
        return default

    def count(self) -> int:
        return len(self._events)

    def clear(self):
        self._events.clear()
        self._index = 0

    def stats(self) -> dict:
        return {"count": len(self._events), "index": self._index}

#### result_service/__init__.py

"""V4-Local 结果服务模块 — 工业测试裁决引擎

文档依据: 4.docx (V4-Local 工业级系统升级方案-结果输出接口增强版)

模块结构:
  schema.py          ResultRecord 标准数据结构
  result_engine.py   结果计算引擎 (1/0/-1)
  result_store.py    结果持久化存储
  result_api.py      HTTP REST 查询接口
"""
from .schema import ResultRecord
from .result_engine import ResultEngine
from .result_store import ResultStore
from .result_api import ResultAPI

#### result_service/api.py

"""5.docx §7.1 Result API"""
from result_service.result_api import ResultAPI, ResultHandler, api_server
__all__ = ["ResultAPI", "ResultHandler", "api_server"]

#### result_service/engine.py

"""5.docx §7.1 Result Engine"""
from result_service.result_engine import ResultEngine
__all__ = ["ResultEngine"]

#### result_service/result_api.py

"""结果服务 — HTTP REST 查询接口

4.docx §七 对外查询接口设计：
  GET /result/{device_id}  -> {"device_id":"...", "result": 1|0|-1}
  GET /result/all          -> 全部结果列表
"""
import json, time, os, sys
from http.server import HTTPServer, BaseHTTPRequestHandler
from urllib.parse import urlparse

from .result_store import store
from .schema import ResultRecord

class ResultHandler(BaseHTTPRequestHandler):
    def do_GET(self):
        parsed = urlparse(self.path)
        path = parsed.path.rstrip("/")

        try:
            if path == "/result/all":
                data = store.get_all()
                self._json(200, data)

            elif path.startswith("/result/"):
                device_id = path[len("/result/"):]
                if device_id:
                    result = store.get_or_default(device_id)
                    self._json(200, [result])
                else:
                    self._json(400, {"error": "missing device_id"})
            else:
                self._json(404, {"error": "not found"})
        except Exception as e:
            self._json(500, {"error": str(e)})

    def _json(self, code, data):
        body = json.dumps(data, ensure_ascii=False).encode("utf-8")
        self.send_response(code)
        self.send_header("Content-Type", "application/json; charset=utf-8")
        self.send_header("Access-Control-Allow-Origin", "*")
        self.end_headers()
        self.wfile.write(body)

    def log_message(self, fmt, *args):
        pass  # Quiet

class ResultAPI:
    def __init__(self, host="0.0.0.0", port=8081):
        self.host = host
        self.port = port
        self._server = None

    def start(self):
        self._server = HTTPServer((self.host, self.port), ResultHandler)
        print(f"Result API running: http://{self.host}:{self.port}/result/")
        self._server.serve_forever()

    def start_in_thread(self):
        import threading
        t = threading.Thread(target=self.start, daemon=True)
        t.start()
        return t

    def stop(self):
        if self._server:
            self._server.shutdown()

api_server = ResultAPI()

#### result_service/result_engine.py

"""结果服务 — 结果计算引擎

依据 4.docx §六 判定逻辑设计：
  输入: device_id
  来源: GPS对比结果 + CM510/K803插件输出 + 规则引擎
  输出: 1=正确 0=错误 -1=未知(未计算)
"""
import time, logging
from upgrade.eventbus import Event, Topics, bus
from .schema import ResultRecord
from .result_store import ResultStore

logger = logging.getLogger(__name__)

class ResultEngine:
    def __init__(self, store: ResultStore = None):
        self._store = store or ResultStore()

    def compute(self, device_id: str, data: dict = None) -> ResultRecord:
        """计算设备最终结果

        判定逻辑:
        1. 检查 GPS 定位数据有效性 (lat/lng 范围)
        2. 检查精度值 E/N/U 是否在阈值内
        3. 检查数据中是否有错误标记
        4. 综合评分
        """
        reasons = []
        score = 0
        max_score = 3

        if data is None:
            data = {}

        # 规则1: GPS 坐标范围检查
        lat = data.get("lat") or data.get("lat", 0)
        lng = data.get("lng") or data.get("lng", 0)
        if 20 < float(lat) < 40 and 100 < float(lng) < 130:
            score += 1
        else:
            reasons.append("坐标超出中国范围")

        # 规则2: 精度检查
        e = float(data.get("e", 0) or 0)
        n = float(data.get("n", 0) or 0)
        u = float(data.get("u", 0) or 0)
        max_precision = max(e, n, u)
        if max_precision < 0.1:
            score += 1
        else:
            reasons.append(f"精度超阈值: max={max_precision:.3f}m")

        # 规则3: 错误标记检查
        error = data.get("error", "") or data.get("errors", "") or ""
        if not error:
            score += 1
        else:
            reasons.append(f"含错误标记: {error}")

        # 综合判定
        if score >= max_score:
            result = 1
        elif score >= 1:
            result = 0
        else:
            result = -1

        record = ResultRecord(
            device_id=device_id,
            result=result,
            reason="; ".join(reasons) if reasons else ("全部校验通过" if result == 1 else "部分未通过"),
            source="result_engine",
        )
        self._store.save(record)
        return record

    def evaluate_batch(self, device_id: str, data_list: list[dict]) -> ResultRecord:
        """批量评估一组数据（如 A/B/C 三组）取综合结果"""
        if not data_list:
            return ResultRecord(device_id=device_id, result=-1, reason="无数据")

        results = []
        for d in data_list:
            rec = self.compute(device_id, d)
            results.append(rec.result)

        # 综合结果: 全部通过=1, 部分通过=0, 全部失败=-1
        if all(r == 1 for r in results):
            final = 1
            reason = "全部通过"
        elif all(r == -1 for r in results):
            final = -1
            reason = "全部未计算"
        else:
            final = 0
            reason = f"部分未通过: {results}"

        record = ResultRecord(device_id=device_id, result=final, reason=reason, source="batch")
        self._store.save(record)
        return record

#### result_service/result_store.py

"""结果服务 — 结果持久化存储

支持内存索引 + JSON 文件持久化。
"""
import json, os, logging, threading, time
from .schema import ResultRecord

logger = logging.getLogger(__name__)

class ResultStore:
    def __init__(self, filepath: str = ""):
        self._lock = threading.Lock()
        self._data: dict[str, ResultRecord] = {}
        self._filepath = filepath or os.path.join(
            os.path.dirname(__file__), "result_data.json"
        )
        self._load()

    def save(self, record: ResultRecord):
        with self._lock:
            self._data[record.device_id] = record
            self._persist()
        logger.debug("Result saved: %s -> %d", record.device_id, record.result)

    def get(self, device_id: str) -> ResultRecord:
        with self._lock:
            return self._data.get(device_id)

    def get_or_default(self, device_id: str) -> dict:
        r = self.get(device_id)
        if r:
            return r.to_dict()
        return {"device_id": device_id, "result": -1, "reason": "未计算"}

    def get_all(self) -> list[dict]:
        with self._lock:
            return [r.to_dict() for r in self._data.values()]

    def count(self) -> int:
        with self._lock:
            return len(self._data)

    def clear(self):
        with self._lock:
            self._data.clear()
            self._persist()

    def _persist(self):
        try:
            with open(self._filepath, "w", encoding="utf-8") as f:
                json.dump({k: r.to_dict() for k, r in self._data.items()},
                          f, ensure_ascii=False, indent=2)
        except Exception as e:
            logger.warning("ResultStore persist error: %s", e)

    def _load(self):
        if os.path.exists(self._filepath):
            try:
                with open(self._filepath, "r", encoding="utf-8") as f:
                    raw = json.load(f)
                    for k, v in raw.items():
                        self._data[k] = ResultRecord.from_dict(v)
            except Exception as e:
                logger.warning("ResultStore load error: %s", e)

store = ResultStore()

#### result_service/schema.py

"""结果服务 — ResultRecord 标准数据结构

符合 4.docx §五 数据规范：
  device_id: str       设备标识
  timestamp: int       时间戳
  result: int          1=正确 0=错误 -1=未知
  reason: str          原因说明（内部使用）
"""
import time, json
from dataclasses import dataclass, field, asdict

@dataclass
class ResultRecord:
    device_id: str = ""
    timestamp: float = 0.0
    result: int = -1       # 1=正确 0=错误 -1=未知
    reason: str = ""       # 可选：原因描述
    source: str = ""       # 数据来源

    def __post_init__(self):
        if not self.timestamp:
            self.timestamp = time.time()

    def to_dict(self) -> dict:
        return {"device_id": self.device_id, "result": self.result,
                "reason": self.reason, "source": self.source,
                "updated_at": int(self.timestamp)}

    @classmethod
    def from_dict(cls, d: dict) -> "ResultRecord":
        return cls(
            device_id=d.get("device_id", ""),
            timestamp=d.get("updated_at", 0) or d.get("timestamp", 0),
            result=d.get("result", -1),
            reason=d.get("reason", ""),
            source=d.get("source", ""),
        )

    def to_json(self) -> str:
        return json.dumps(self.to_dict(), ensure_ascii=False)

#### result_service/store.py

"""5.docx §7.1 Result Store"""
from result_service.result_store import ResultStore, store
__all__ = ["ResultStore", "store"]

#### signal_receiver/__init__.py

from .serial_receiver import SerialReceiver
from .tcp_receiver import TCPReceiver
from .udp_receiver import UDPReceiver
from .mqtt_receiver import MQTTReceiver
from .simulator import SignalSimulator
from .pipeline import DataPipeline
from .connection_pool import ConnectionPool
from .heartbeat import HeartbeatMonitor
from .serial_manager import SerialManager, get_default as get_serial_mgr
from .serial_mock_device import SerialMockDevice
from .serial_monitor import SerialMonitor

#### signal_receiver/connection_pool.py

"""TCP 连接池 — 管理多设备连接"""
import socket, threading, time, logging
logger = logging.getLogger(__name__)

class ConnectionPool:
    def __init__(self, max_size=500):
        self.max_size = max_size
        self._conns = {}; self._lock = threading.Lock(); self._running = False

    def add(self, conn_id: str, conn: socket.socket, addr=None):
        with self._lock:
            if len(self._conns) >= self.max_size:
                return False
            self._conns[conn_id] = {"conn": conn, "addr": addr or "",
                "connected": time.time(), "last_active": time.time(), "bytes_recv": 0}
            return True

    def remove(self, conn_id: str):
        with self._lock:
            if conn_id in self._conns:
                try: self._conns[conn_id]["conn"].close()
                except: pass
                del self._conns[conn_id]

    def get(self, conn_id: str):
        with self._lock: return self._conns.get(conn_id)

    def send(self, conn_id: str, data: bytes) -> bool:
        c = self.get(conn_id)
        if not c: return False
        try:
            c["conn"].sendall(data)
            c["last_active"] = time.time()
            return True
        except: self.remove(conn_id); return False

    def broadcast(self, data: bytes):
        for cid in list(self._conns.keys()):
            self.send(cid, data)

    def stats(self):
        with self._lock:
            return {"active": len(self._conns), "max": self.max_size,
                    "connections": list(self._conns.keys())}

    def start_health_check(self, interval=30, timeout=120):
        self._running = True
        def _check():
            while self._running:
                time.sleep(interval)
                now = time.time()
                stale = [cid for cid, c in list(self._conns.items()) if now - c["last_active"] > timeout]
                for cid in stale:
                    logger.warning("Health: removing stale connection %s", cid)
                    self.remove(cid)
        threading.Thread(target=_check, daemon=True).start()

    def stop(self): self._running = False

#### signal_receiver/heartbeat.py

"""心跳监测 — 连接保活 + 超时检测"""
import time, threading, logging
logger = logging.getLogger(__name__)

class HeartbeatMonitor:
    def __init__(self, timeout=60, interval=10):
        self.timeout = timeout; self.interval = interval
        self._beats = {}; self._running = False; self._callbacks = []

    def on_timeout(self, cb): self._callbacks.append(cb)

    def beat(self, conn_id: str):
        self._beats[conn_id] = time.time()

    def remove(self, conn_id: str):
        self._beats.pop(conn_id, None)

    def start(self):
        self._running = True
        threading.Thread(target=self._loop, daemon=True).start()
        logger.info("HeartbeatMonitor started (timeout=%ds, interval=%ds)", self.timeout, self.interval)

    def _loop(self):
        while self._running:
            time.sleep(self.interval)
            now = time.time()
            stale = [cid for cid, t in list(self._beats.items()) if now - t > self.timeout]
            for cid in stale:
                logger.warning("Heartbeat timeout: %s (%ds since last beat)", cid, now - self._beats[cid])
                del self._beats[cid]
                for cb in self._callbacks: cb(cid)

    def stop(self): self._running = False

    def stats(self):
        return {"active": len(self._beats), "timeout": self.timeout}

#### signal_receiver/mqtt_receiver.py

"""MQTT 信号接收器"""
import json, logging, threading
logger = logging.getLogger(__name__)
class MQTTReceiver:
    def __init__(self, host="localhost", port=1883, topic="survey/data/+"):
        self.host, self.port, self.topic = host, port, topic
        self._client = None; self._running = False; self._callbacks = []
    def on_data(self, cb): self._callbacks.append(cb)
    def start(self):
        import paho.mqtt.client as mqtt
        self._client = mqtt.Client()
        self._client.on_message = self._on_msg
        self._client.connect(self.host, self.port, 60)
        self._client.subscribe(self.topic, qos=1)
        
        self._running = True
        threading.Thread(target=self._client.loop_forever, daemon=True).start()
        logger.info("MQTT receiver: %s:%d %s", self.host, self.port, self.topic)
    def _on_msg(self, client, userdata, msg):
        [cb(msg.payload.decode("utf-8")) for cb in self._callbacks]
    def stop():
        if self._client: self._client.disconnect()

#### signal_receiver/pipeline.py

"""V4-Local Stream Pipeline — 事件流处理管道

符合 Document 3 Step 2 规范：
- Raw Event → Parser → Rule Engine → Plugin → Storage
- 集成 EventBus 发布标准事件
- 多线程 Worker 处理
"""
import threading, logging, time, json
from upgrade.eventbus import Event, Topics, bus
from typing import Optional

logger = logging.getLogger(__name__)

class StreamPipeline:
    def __init__(self, maxsize=10000, workers=2):
        self._queue = __import__("queue").Queue(maxsize=maxsize)
        self._workers_count = workers
        self._running = False
        self._threads = []
        self._parsed_count = 0
        self._error_count = 0

    def start(self):
        self._running = True
        for i in range(self._workers_count):
            t = threading.Thread(target=self._process_loop, daemon=True, name=f"stream-{i}")
            t.start()
            self._threads.append(t)
        logger.info("StreamPipeline started (%d workers)", self._workers_count)

    def feed(self, raw: str, source: str = "") -> bool:
        """接收原始报文，进入队列"""
        try:
            self._queue.put((raw, source), timeout=1)
            return True
        except __import__("queue").Full:
            self._error_count += 1
            return False

    def _process_loop(self):
        """Worker 主循环：Raw → 自动检测 → 解析 → EventBus 发布"""
        while self._running:
            try:
                raw, source = self._queue.get(timeout=1)
            except __import__("queue").Empty:
                continue
            try:
                parsed = self._auto_parse(raw)
                if parsed:
                    bus.publish(Topics.DEVICE_PARSED, parsed, source=source)
                    self._parsed_count += 1
                else:
                    # 无法解析也发布原始事件
                    bus.publish(Topics.DEVICE_RAW, {"raw": raw}, source=source)
            except Exception as e:
                self._error_count += 1
                logger.warning("StreamPipeline error: %s", e)

    def _auto_parse(self, raw: str) -> Optional[dict]:
        """自动检测报文类型并解析"""
        raw = raw.strip()
        if not raw:
            return None
        if raw.startswith("#BESTPOSA"):
            try:
                from plugins.bestpos import BESTPOSASCIIParser
                return BESTPOSASCIIParser.parse(raw)
            except Exception as e:
                logger.debug("BESTPOS parse error: %s", e)
                return None
        elif raw.startswith("$GP") or raw.startswith("$GN") or raw.startswith("$GA"):
            try:
                from plugins.nmea import NMEAParser
                parser = NMEAParser()
                result = parser.parse(raw)
                if result and hasattr(result, "to_dict"):
                    return result.to_dict()
                return result
            except Exception as e:
                logger.debug("NMEA parse error: %s", e)
                return None
        elif raw.startswith("$CM510"):
            try:
                from plugins.cm510_parser import CM510Parser
                return CM510Parser.parse(raw)
            except Exception as e:
                logger.debug("CM510 parse error: %s", e)
                return None
        # JSON 格式
        elif raw.startswith("{"):
            try:
                return json.loads(raw)
            except json.JSONDecodeError:
                return None
        return None

    def stop(self):
        self._running = False
        for t in self._threads:
            t.join(timeout=3)

    def stats(self) -> dict:
        return {
            "queue_size": self._queue.qsize(),
            "maxsize": self._queue.maxsize,
            "parsed": self._parsed_count,
            "errors": self._error_count,
            "running": self._running,
            "workers": self._workers_count,
        }

    def status_line(self) -> str:
        return f"Pipeline: q={self._queue.qsize()} parsed={self._parsed_count} err={self._error_count} running={self._running}"

pipeline = StreamPipeline()

#### signal_receiver/serial_manager.py

"""串口管理器 — 扫描/打开/关闭/配置"""
import time, threading, logging
logger = logging.getLogger(__name__)

class SerialManager:
    def __init__(self):
        self._ports = {}; self._open_ports = {}; self._callbacks = []

    def list_ports(self):
        """扫描可用串口"""
        try:
            import serial.tools.list_ports
            ports = []
            for p in serial.tools.list_ports.comports():
                ports.append({"port": p.device, "desc": p.description, "hwid": p.hwid, "status": "available"})
            return ports
        except ImportError:
            return [{"port": "COM1","desc":"Mock","hwid":"","status":"simulated"}]

    def open(self, port, baud=115200, timeout=3):
        """打开串口"""
        if port in self._open_ports: return True
        try:
            import serial
            ser = serial.Serial(port, baud, timeout=timeout)
            self._open_ports[port] = {"ser": ser, "baud": baud, "opened": time.time(), "bytes": 0, "errors": 0}
            logger.info("Serial %s opened @ %d", port, baud)
            return True
        except Exception as e:
            logger.warning("Serial %s open error: %s", port, e)
            return False

    def close(self, port):
        if port in self._open_ports:
            try: self._open_ports[port]["ser"].close()
            except: pass
            del self._open_ports[port]

    def send(self, port, data: bytes):
        p = self._open_ports.get(port)
        if not p: return False
        try:
            n = p["ser"].write(data)
            p["bytes"] += n
            return True
        except: return False

    def read(self, port):
        p = self._open_ports.get(port)
        if not p: return None
        try:
            data = p["ser"].readline()
            p["bytes"] += len(data)
            return data.decode("ascii", errors="ignore").strip()
        except: return None

    def status(self):
        return {p: {"status": "open", "baud": info["baud"], "uptime": int(time.time()-info["opened"]),
                     "bytes": info["bytes"], "errors": info["errors"]}
                for p, info in self._open_ports.items()}

    def close_all(self):
        for p in list(self._open_ports.keys()): self.close(p)

# Global singleton
_default = None
def get_default():
    global _default
    if _default is None: _default = SerialManager()
    return _default

#### signal_receiver/serial_mock_device.py

"""CM510 串口模拟设备 — 定时发送报文，支持速率控制和异常注入"""
import time, random, threading, logging
logger = logging.getLogger(__name__)

class SerialMockDevice:
    def __init__(self, device_id="MOCK-CM510-001", interval=1.0):
        self.device_id = device_id
        self.interval = interval
        self._running = False; self._callbacks = []; self._inject_error = False

    def on_data(self, cb): self._callbacks.append(cb)

    def set_rate(self, interval): self.interval = interval

    def inject_error(self, enable: bool): self._inject_error = enable

    def _gen_cm510(self):
        lat = round(30.0 + random.uniform(-0.02, 0.02), 6)
        lng = round(120.5 + random.uniform(-0.02, 0.02), 6)
        alt = round(50 + random.uniform(-5, 5), 2)
        speed = round(random.uniform(0, 60), 1)
        heading = round(random.uniform(0, 360), 1)
        sats = random.randint(4, 15)
        signal = random.randint(1, 5)
        ts = int(time.time())
        crc = random.randint(0, 255)
        if self._inject_error:
            lat = 999.0  # 异常坐标
        return f"$CM510,{self.device_id},{lat:.6f},{lng:.6f},{speed:.1f},{heading:.1f},{alt:.2f},{sats},{signal},{ts}*{crc:02X}"

    def start(self):
        self._running = True
        threading.Thread(target=self._run, daemon=True).start()
        logger.info("Mock device started: %s (interval=%.1fs)", self.device_id, self.interval)

    def _run(self):
        while self._running:
            data = self._gen_cm510()
            for cb in self._callbacks: cb(data)
            time.sleep(self.interval)

    def stop(self): self._running = False

#### signal_receiver/serial_monitor.py

"""串口监控 — 实时监听串口数据，桥接到Pipeline"""
import time, threading, logging
logger = logging.getLogger(__name__)

class SerialMonitor:
    def __init__(self, pipeline=None):
        self._ports = {}; self._pipeline = pipeline; self._running = False; self._stats = {"total":0,"parsed":0,"errors":0}

    def attach(self, port, manager, pipeline=None):
        self._ports[port] = {"manager": manager, "pipeline": pipeline, "last_data": 0, "data_count": 0}
        if pipeline: self._pipeline = pipeline

    def start(self):
        self._running = True
        for port in self._ports:
            threading.Thread(target=self._monitor, args=(port,), daemon=True).name = f"mon-{port}"
        logger.info("SerialMonitor started (%d ports)", len(self._ports))

    def _monitor(self, port):
        info = self._ports.get(port)
        if not info: return
        mgr = info["manager"]
        while self._running:
            try:
                line = mgr.read(port)
                if line:
                    info["last_data"] = time.time()
                    info["data_count"] += 1
                    self._stats["total"] += 1
                    if info["pipeline"]:
                        info["pipeline"].feed(line)
                        self._stats["parsed"] += 1
                    else:
                        for cb in getattr(mgr, "_callbacks", []): cb(line)
                else:
                    time.sleep(0.05)
            except Exception as e:
                self._stats["errors"] += 1
                logger.warning("Monitor %s error: %s", port, e)
                time.sleep(1)

    def stats(self, port=None):
        if port:
            info = self._ports.get(port)
            if not info: return {}
            return {"connected": True, "data_count": info["data_count"], "idle": int(time.time()-info["last_data"])}
        result = {}
        for p, info in self._ports.items():
            result[p] = {"connected": True, "data_count": info["data_count"], "idle": int(time.time()-info["last_data"])}
        result["_total"] = self._stats
        return result

    def stop(self): self._running = False

#### signal_receiver/serial_receiver.py

"""串口信号接收器 — 接收 CM510/K803 串口数据"""
import time, logging, threading
logger = logging.getLogger(__name__)
class SerialReceiver:
    def __init__(self, port="COM3", baud=115200, timeout=3):
        self.port, self.baud, self.timeout = port, baud, timeout
        self._ser = None; self._running = False; self._callbacks = []
    def on_data(self, cb): self._callbacks.append(cb)
    def start(self):
        import serial
        self._ser = serial.Serial(self.port, self.baud, timeout=self.timeout)
        self._running = True
        logger.info("Serial receiver started: %s @ %d", self.port, self.baud)
        threading.Thread(target=self._run, daemon=True).start()
    def _run(self):
        while self._running:
            try:
                line = self._ser.readline().decode("ascii", errors="ignore").strip()
                if line: [cb(line) for cb in self._callbacks]
            except: time.sleep(0.01)
    def stop(self):
        self._running = False
        if self._ser: self._ser.close()

#### signal_receiver/simulator.py

"""信号模拟器 — 模拟 CM510/K803 发送测试数据"""
import time, random, logging, threading
logger = logging.getLogger(__name__)
class SignalSimulator:
    def __init__(self, device_id="SIM-001", interval=3):
        self.device_id = device_id; self.interval = interval
        self._running = False; self._callbacks = []
    def on_data(self, cb): self._callbacks.append(cb)
    def start(self):
        self._running = True
        threading.Thread(target=self._run, daemon=True).start()
        logger.info("Simulator started: %s (interval=%ds)", self.device_id, self.interval)
    def _gen_gps(self):
        return "$CM510,{},{:.6f},{:.6f},{:.1f},{:.1f},{:.1f},{},{},{},*{:02X}".format(
            self.device_id,
            30.0 + random.uniform(-0.05, 0.05),
            120.5 + random.uniform(-0.05, 0.05),
            random.uniform(0, 60),  # speed
            random.uniform(0, 360),  # heading
            random.uniform(0, 200),  # altitude
            random.randint(4, 15),   # satellites
            random.randint(1, 5),    # signal
            int(time.time()),
            random.randint(0, 255),  # CRC
        )
    def _run(self):
        while self._running:
            data = self._gen_gps()
            [cb(data) for cb in self._callbacks]
            time.sleep(self.interval)
    def stop(self): self._running = False

#### signal_receiver/tcp_receiver.py

"""TCP 信号接收器 — TCP Server/Client 模式"""
import socket, time, logging, threading
logger = logging.getLogger(__name__)
class TCPReceiver:
    def __init__(self, host="0.0.0.0", port=8888, mode="server"):
        self.host, self.port, self.mode = host, port, mode
        self._sock = None; self._running = False; self._callbacks = []
    def on_data(self, cb): self._callbacks.append(cb)
    def start(self):
        self._running = True
        if self.mode == "server":
            self._sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
            self._sock.setsockopt(socket.SOL_SOCKET, socket.SO_REUSEADDR, 1)
            self._sock.bind((self.host, self.port)); self._sock.listen(5)
            logger.info("TCP server: %s:%d", self.host, self.port)
            threading.Thread(target=self._accept, daemon=True).start()
        else:
            threading.Thread(target=self._run_client, daemon=True).start()
    def _accept(self):
        while self._running:
            try:
                conn, addr = self._sock.accept()
                threading.Thread(target=self._handle, args=(conn,), daemon=True).start()
            except: time.sleep(0.1)
    def _handle(self, conn):
        buf = b""
        while self._running:
            try:
                d = conn.recv(4096)
                if not d: break
                buf += d
                while b"\n" in buf:
                    line, buf = buf.split(b"\n", 1)
                    [cb(line.decode("ascii",errors="ignore")) for cb in self._callbacks]
            except: break
    def _run_client(self):
        backoff = 1; max_backoff = 60
        while self._running:
            try:
                self._sock = socket.create_connection((self.host, self.port), timeout=5)
            except Exception as e:
                logger.warning("TCP client reconnect in %ds: %s", backoff, e)
                self._running and time.sleep(backoff)
                backoff = min(backoff * 2, max_backoff)
                continue
            buf = b""
            while self._running:
                try:
                    d = self._sock.recv(4096)
                    if not d: break
                    buf += d
                    while b"\n" in buf:
                        line, buf = buf.split(b"\n", 1)
                        [cb(line.decode("ascii",errors="ignore")) for cb in self._callbacks]
                except: break
            break
    def stop(self): self._running = False

#### signal_receiver/udp_receiver.py

"""UDP 信号接收器"""
import socket, logging, threading
logger = logging.getLogger(__name__)
class UDPReceiver:
    def __init__(self, host="0.0.0.0", port=8889):
        self.host, self.port = host, port; self._callbacks = []
        self._running = False; self._sock = None
    def on_data(self, cb): self._callbacks.append(cb)
    def start(self):
        self._sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)
        self._sock.setsockopt(socket.SOL_SOCKET, socket.SO_REUSEADDR, 1)
        self._sock.bind((self.host, self.port)); self._running = True
        logger.info("UDP receiver: %s:%d", self.host, self.port)
        threading.Thread(target=self._run, daemon=True).start()
    def _run(self):
        while self._running:
            try:
                d, addr = self._sock.recvfrom(8192)
                [cb(d.decode("ascii",errors="ignore")) for cb in self._callbacks]
            except: pass
    def stop(self): self._running = False

#### storage/__init__.py

from .sqlite_storage import SQLiteStorage

#### storage/sqlite_storage.py

"""V4 SQLite 存储层 — 与 DuckDB 并存，用于实时日志和轻量查询"""
import sqlite3, os, time, threading, json

DB_DIR = os.path.join(os.path.dirname(os.path.dirname(os.path.abspath(__file__))), "databases")
DB_PATH = os.path.join(DB_DIR, "survey_realtime.db")

class SQLiteStorage:
    def __init__(self, db_path=DB_PATH):
        os.makedirs(os.path.dirname(db_path), exist_ok=True)
        self._conn = sqlite3.connect(db_path, check_same_thread=False)
        self._lock = threading.Lock()
        self._init_db()

    def _init_db(self):
        self._conn.executescript("""
            CREATE TABLE IF NOT EXISTS raw_packets (
                id INTEGER PRIMARY KEY AUTOINCREMENT,
                device_id TEXT, raw_data TEXT, packet_type TEXT,
                parse_status TEXT DEFAULT 'pending', created_at REAL
            );
            CREATE TABLE IF NOT EXISTS gps_log (
                id INTEGER PRIMARY KEY AUTOINCREMENT,
                device_id TEXT, latitude REAL, longitude REAL,
                altitude REAL, speed REAL, heading REAL,
                satellites INTEGER, fix_type TEXT, created_at REAL
            );
            CREATE INDEX IF NOT EXISTS idx_raw_time ON raw_packets(created_at);
            CREATE INDEX IF NOT EXISTS idx_gps_device ON gps_log(device_id);
        """)
        self._conn.commit()

    def insert_raw(self, device_id, raw_data, packet_type="unknown", status="pending"):
        with self._lock:
            self._conn.execute("INSERT INTO raw_packets(device_id,raw_data,packet_type,parse_status,created_at) VALUES(?,?,?,?,?)",
                               (device_id, raw_data, packet_type, status, time.time()))
            self._conn.commit()

    def insert_gps(self, device_id, lat, lng, alt=0, speed=0, heading=0, satellites=0, fix_type="none"):
        with self._lock:
            self._conn.execute("INSERT INTO gps_log(device_id,latitude,longitude,altitude,speed,heading,satellites,fix_type,created_at) VALUES(?,?,?,?,?,?,?,?,?)",
                               (device_id, lat, lng, alt, speed, heading, satellites, fix_type, time.time()))
            self._conn.commit()

    def query_gps(self, device_id=None, limit=100):
        with self._lock:
            if device_id:
                cur = self._conn.execute("SELECT * FROM gps_log WHERE device_id=? ORDER BY id DESC LIMIT ?", (device_id, limit))
            else:
                cur = self._conn.execute("SELECT * FROM gps_log ORDER BY id DESC LIMIT ?", (limit,))
            return [dict(zip([d[0] for d in cur.description], r)) for r in cur.fetchall()]

    def stats(self):
        with self._lock:
            raw = self._conn.execute("SELECT COUNT(*) FROM raw_packets").fetchone()[0]
            gps = self._conn.execute("SELECT COUNT(*) FROM gps_log").fetchone()[0]
            return {"raw_packets": raw, "gps_logs": gps}

# 全局实例
_default = None
def get_default():
    global _default
    if _default is None: _default = SQLiteStorage()
    return _default

#### tests/__init__.py

#### tests/mock_device.py

"""V4 模拟设备 — 用于测试和压力测试"""
import time, random, threading

class MockCM510:
    def __init__(self, device_id="MOCK-001", interval=1.0):
        self.device_id = device_id; self.interval = interval
        self._callbacks = []; self._running = False

    def on_data(self, cb): self._callbacks.append(cb)

    def start(self):
        self._running = True
        threading.Thread(target=self._run, daemon=True).start()

    def _run(self):
        while self._running:
            lat = round(30.0 + random.uniform(-0.05, 0.05), 6)
            lng = round(120.5 + random.uniform(-0.05, 0.05), 6)
            msg = f"$CM510,{self.device_id},{lat},{lng},{random.uniform(0,60):.1f},{random.uniform(0,360):.1f},{random.uniform(0,200):.2f},{random.randint(4,15)},{random.randint(1,5)},{int(time.time())}*{random.randint(0,255):02X}"
            for cb in self._callbacks: cb(msg)
            time.sleep(self.interval)

    def stop(self): self._running = False

#### tests/stress_test.py

"""V4 压力测试 — 向 API 发送批量请求"""
import urllib.request, json, time, random, sys

URL = "http://localhost:8080/api/survey/upload"

def gen_bestposa(seq_start=1000):
    lines = []
    for j in range(3):
        lat = 30.0 + random.uniform(-0.001, 0.001)
        lng = 120.5 + random.uniform(-0.001, 0.001)
        alt = 50 + random.uniform(-0.5, 0.5)
        lines.append(f"#BESTPOSA,COM1,0,60.0,FINESTEERING,2222,{seq_start+j:.3f},00000000,0000,1114;SOL_COMPUTED,NARROW_INT,{lat:.6f},{lng:.6f},{alt:.2f},1.0,WGS84,0.008,0.009,0.015,0008,0.05,0.05,8,6,1,1,0,0,0,0*0000")
    return "\n".join(lines)

def run(count=10):
    ok = 0; fail = 0; times = []
    for i in range(count):
        data = gen_bestposa(1000 + i * 3).encode()
        try:
            start = time.time()
            req = urllib.request.Request(URL, data=data, headers={"Content-Type": "text/plain"})
            resp = urllib.request.urlopen(req, timeout=5)
            elapsed = time.time() - start
            times.append(elapsed)
            r = json.loads(resp.read())
            if r.get("success") == r.get("total"): ok += 1
            else: fail += 1
        except: fail += 1
    avg_ms = sum(times) / len(times) * 1000 if times else 0
    print(f"结果: {ok}/{count} 通过, {fail} 失败")
    print(f"平均耗时: {avg_ms:.1f}ms")
    print(f"吞吐量: {count/(sum(times) if times else 1):.0f} req/s")

if __name__ == "__main__":
    c = int(sys.argv[1]) if len(sys.argv) > 1 else 10
    run(c)

#### upgrade/__init__.py

"""服务器端升级模块 - 重复检测 / 精度校验 / 序列检查 / 熔断 + API"""
"""服务器端升级模块 - EventBus / PluginSDK / 沙箱 / 重复检测 / 精度校验 / 序列检查 / 熔断"""
from .eventbus import EventBus, Event, Topics, StreamPipeline, bus, pipeline
from .plugin_sdk import BasePlugin, PluginManager, PluginInput, PluginOutput, PluginContext, manager
from .sandbox import run_in_sandbox, run_with_timeout
from .duplicate_detector import DuplicateDetector
from .precision_checker import PrecisionChecker
from .sequence_checker import SequenceChecker
from .circuit_breaker import CircuitBreaker

#### upgrade/base_parser.py

"""V4 协议解析器基类 — 统一所有解析器接口"""
import abc

class BaseParser(abc.ABC):
    @property
    @abc.abstractmethod
    def protocol_name(self) -> str: ...

    @abc.abstractmethod
    def can_parse(self, data: str) -> bool: ...

    @abc.abstractmethod
    def parse(self, data: str) -> dict | None: ...

    def validate(self, parsed: dict) -> bool:
        return bool(parsed and "lat" in parsed and "lng" in parsed)


class ParserRegistry:
    _parsers: dict[str, BaseParser] = {}

    @classmethod
    def register(cls, parser: BaseParser):
        cls._parsers[parser.protocol_name] = parser

    @classmethod
    def detect(cls, data: str) -> BaseParser | None:
        for p in cls._parsers.values():
            if p.can_parse(data): return p
        return None

    @classmethod
    def parse(cls, data: str) -> dict | None:
        parser = cls.detect(data)
        return parser.parse(data) if parser else None

    @classmethod
    def list(cls) -> list[str]:
        return list(cls._parsers.keys())

#### upgrade/circuit_breaker.py

"""熔断机制 — 某勘测员错误率超过阈值时临时隔离

维护滑动窗口（最多 N 条记录），计算最近通过率。
错误率 > threshold 时触发熔断，设备被隔离。
隔离持续 cooldown 秒后自动重置，或手动恢复。
"""
import time, logging

logger = logging.getLogger(__name__)

class CircuitBreaker:
    def __init__(self, window_size=100, error_threshold=0.3, cooldown=300):
        self.window_size = window_size       # 滑动窗口大小
        self.error_threshold = error_threshold  # 触发熔断的错误率
        self.cooldown = cooldown             # 隔离冷却时间(秒)
        self._history: dict[str, list[bool]] = {}  # device_id → [passed, ...]
        self._isolated: dict[str, float] = {}      # device_id → isolation_time

    def record(self, device_id: str, passed: bool):
        """记录一次勘测结果"""
        if device_id not in self._history:
            self._history[device_id] = []
        self._history[device_id].append(passed)
        if len(self._history[device_id]) > self.window_size:
            self._history[device_id].pop(0)

        # 计算错误率
        err_rate = self._error_rate(device_id)
        if err_rate > self.error_threshold and device_id not in self._isolated:
            self._isolated[device_id] = time.time()
            logger.warning(
                "CIRCUIT BREAKER: %s isolated (error rate %.1f%% > %.0f%%)",
                device_id, err_rate * 100, self.error_threshold * 100
            )

    def check(self, device_id: str) -> tuple[bool, str]:
        now = time.time()
        # Check throttled state
        if device_id in self._isolated:
            elapsed = now - self._isolated[device_id]
            if elapsed > self.cooldown:
                del self._isolated[device_id]
                return True, "熔断已恢复"
            remaining = self.cooldown - elapsed
            return (False, 
                    f"设备已隔离(错误率{self._error_rate(device_id)*100:.0f}%)"
                    f" 剩余{remaining:.0f}s")
        
        # Auto-throttle: if recent error rate > 50%, return throttle signal
        err_rate = self._error_rate(device_id)
        if err_rate > self.error_threshold and self._history.get(device_id, []):
            return (True, 
                    f"请求已接收，但该设备错误率{err_rate*100:.0f}%偏高，建议暂停")
        
        return True, "正常"
        """检查设备是否被熔断。返回 (allowed, message)"""
        now = time.time()

        # 检查是否已过冷却期 → 自动恢复
        if device_id in self._isolated:
            elapsed = now - self._isolated[device_id]
            if elapsed > self.cooldown:
                del self._isolated[device_id]
                logger.info("Circuit breaker reset: %s (cooldown %ds elapsed)", device_id, self.cooldown)
                return True, "熔断已恢复"

            remaining = self.cooldown - elapsed
            return False, (
                f"设备已被隔离(错误率{self._error_rate(device_id)*100:.0f}% > {self.error_threshold*100:.0f}%)"
                f" 剩余{remaining:.0f}s"
            )

        return True, "正常"

    def _error_rate(self, device_id: str) -> float:
        h = self._history.get(device_id, [])
        if not h:
            return 0.0
        return sum(1 for p in h if not p) / len(h)

    def reset(self, device_id: str = ""):
        """手动恢复设备"""
        if device_id:
            self._isolated.pop(device_id, None)
            self._history.pop(device_id, None)
        else:
            self._isolated.clear()
            self._history.clear()

    def throttle_remaining(self, device_id: str) -> float:
        """返回设备剩余冷却时间(秒)，0=未隔离"""
        if device_id in self._isolated:
            elapsed = time.time() - self._isolated[device_id]
            return max(0, self.cooldown - elapsed)
        return 0.0

    def stats(self) -> dict:
        result = {}
        for device_id in set(list(self._history.keys()) + list(self._isolated.keys())):
            rate = self._error_rate(device_id)
            result[device_id] = {
                "samples": len(self._history.get(device_id, [])),
                "error_rate": round(rate, 3),
                "isolated": device_id in self._isolated,
                "remaining_cooldown": round(
                    max(0, self.cooldown - (time.time() - self._isolated[device_id]))
                ) if device_id in self._isolated else 0,
            }
        return result

# 全局单例
_default = None
def get_default():
    global _default
    if _default is None:
        _default = CircuitBreaker()
    return _default

#### upgrade/duplicate_detector.py

"""重复检测 — 报文 ID 哈希表，防止重复上报

维护一个带 TTL 的哈希表（内存 + 可持久化到 DuckDB）。
自动清理 24h 前的记录。
"""
import time, hashlib, json, logging

logger = logging.getLogger(__name__)

class DuplicateDetector:
    def __init__(self, ttl=86400, cleanup_interval=3600):
        self.ttl = ttl              # 24h TTL
        self._cleanup_interval = cleanup_interval
        self._seen: dict[str, float] = {}  # hash → timestamp
        self._last_cleanup = time.time()

    def _msg_hash(self, data: dict) -> str:
        """计算报文哈希（去重键）"""
        payload = json.dumps(data, sort_keys=True, separators=(",", ":"))
        return hashlib.sha256(payload.encode()).hexdigest()[:16]

    def _auto_cleanup(self):
        now = time.time()
        if now - self._last_cleanup < self._cleanup_interval:
            return
        cutoff = now - self.ttl
        old = [k for k, ts in self._seen.items() if ts < cutoff]
        for k in old:
            del self._seen[k]
        if old:
            logger.info("DuplicateDetector cleanup: removed %d old entries", len(old))
        self._last_cleanup = now

    def check(self, data: dict) -> tuple[bool, str]:
        """检查是否重复. 返回 (is_duplicate, msg_hash)"""
        h = self._msg_hash(data)
        self._auto_cleanup()
        if h in self._seen:
            elapsed = time.time() - self._seen[h]
            return True, f"重复报文 (hash={h}, {elapsed:.0f}s 前已处理)"
        return False, h

    def record(self, data: dict):
        """记录已处理的报文"""
        h = self._msg_hash(data)
        self._seen[h] = time.time()

    def stats(self) -> dict:
        return {"total_recorded": len(self._seen), "ttl_hours": self.ttl / 3600}

# 全局单例
_default = None
def get_default():
    global _default
    if _default is None:
        _default = DuplicateDetector()
    return _default

#### upgrade/eventbus.py

"""V4 EventBus — 进程内消息总线，替代 Kafka/Queue 单节点方案"""
import threading, queue, time, logging
from collections import defaultdict

logger = logging.getLogger(__name__)

class EventBus:
    def __init__(self):
        self._subscribers = defaultdict(list)
        self._lock = threading.Lock()
        self._history = []
        self._max_history = 1000

    def subscribe(self, topic: str, callback):
        with self._lock:
            self._subscribers[topic].append(callback)
            logger.debug("Subscribed to %s", topic)

    def unsubscribe(self, topic: str, callback):
        with self._lock:
            if callback in self._subscribers[topic]:
                self._subscribers[topic].remove(callback)

    def publish(self, topic: str, data):
        """同步发布 — 直接调用所有订阅者"""
        with self._lock:
            cbs = list(self._subscribers.get(topic, []))
        self._history.append({"topic": topic, "time": time.time(), "data_len": len(str(data))})
        if len(self._history) > self._max_history:
            self._history.pop(0)
        for cb in cbs:
            try:
                cb(data)
            except Exception as e:
                logger.warning("EventBus %s error: %s", topic, e)

    def publish_async(self, topic: str, data):
        """异步发布 — 在新线程中执行，不阻塞调用方"""
        threading.Thread(target=self.publish, args=(topic, data), daemon=True).start()

    def stats(self):
        with self._lock:
            return {
                "topics": {t: len(cbs) for t, cbs in self._subscribers.items()},
                "total_published": len(self._history),
                "recent_topics": list(set(h["topic"] for h in self._history[-50:]))
            }

bus = EventBus()
"""V4-Local EventBus — 标准化事件总线

功能：
- 标准 Event 结构 (id/timestamp/topic/source/payload)
- 预定义 Topic 体系
- 发布/订阅/取消订阅
- 时间范围回放 (replay)
- 流式 Pipeline 链处理
- 线程安全
"""
import uuid, time, logging, threading, heapq
from collections import defaultdict
from dataclasses import dataclass, field, asdict
from typing import Callable, Optional

logger = logging.getLogger(__name__)

# ─── 标准事件结构 ───
@dataclass
class Event:
    id: str = ""
    timestamp: float = 0.0
    topic: str = ""
    source: str = ""
    payload: dict = field(default_factory=dict)

    def __post_init__(self):
        if not self.id:
            self.id = str(uuid.uuid4())[:12]
        if not self.timestamp:
            self.timestamp = time.time()

    def to_dict(self) -> dict:
        return asdict(self)

    @classmethod
    def from_dict(cls, d: dict) -> "Event":
        return cls(**d)


# ─── 预定义 Topic 体系 ───
class Topics:
    DEVICE_RAW     = "device.raw"       # 设备原始报文
    DEVICE_PARSED  = "device.parsed"     # 解析后的结构化数据
    GPS_DATA       = "gps.data"          # GPS 定位数据
    GPS_ANOMALY    = "gps.anomaly"       # GPS 异常事件
    CM510_SIGNAL   = "cm510.signal"      # CM510 信号
    K803_SIGNAL    = "k803.signal"       # K803 信号
    ANALYSIS_RESULT = "analysis.result"  # 比对分析结果
    ANOMALY_DETECTED = "anomaly.detected" # 异常检测事件
    SURVEY_RESULT  = "survey.result"     # 测量结果
    SYSTEM_LOG     = "system.log"        # 系统日志

    @classmethod
    def all(cls) -> list:
        return [v for k, v in vars(cls).items() if not k.startswith("_") and isinstance(v, str)]


# ─── 流式 Pipeline ───
class StreamPipeline:
    """链式处理器: Raw → Parser → Engine → Plugin → Storage"""

    def __init__(self):
        self._stages = []  # list of (name, handler)

    def add_stage(self, name: str, handler: Callable[[Event], Optional[Event]]):
        """添加处理阶段，handler 返回 Event 或 None（丢弃）"""
        self._stages.append((name, handler))
        return self

    def process(self, event: Event) -> Optional[Event]:
        """将事件依次通过所有阶段"""
        current = event
        for name, handler in self._stages:
            try:
                current = handler(current)
                if current is None:
                    logger.debug("Pipeline stage %s dropped event %s", name, event.id)
                    return None
            except Exception as e:
                logger.error("Pipeline stage %s error: %s", name, e)
                return None
        return current


# ─── 事件总线 ───
class EventBus:
    def __init__(self, max_history: int = 10000):
        self._subscribers: dict[str, list[Callable]] = defaultdict(list)
        self._lock = threading.Lock()
        self._history: list[Event] = []
        self._max_history = max_history
        self._pipeline: Optional[StreamPipeline] = None
        self._event_count = 0

    def set_pipeline(self, pipeline: StreamPipeline):
        """设置全局流式 Pipeline"""
        self._pipeline = pipeline

    # ── 发布 ──
    def publish(self, topic: str, payload: dict, source: str = "") -> Event:

"""发布事件（同步，阻塞订阅者）"""
        event = Event(topic=topic, source=source, payload=payload)
        return self._do_publish(event)

    def publish_event(self, event: Event) -> Event:
        """直接发布 Event 对象"""
        return self._do_publish(event)

    def publish_async(self, topic: str, payload: dict, source: str = ""):
        """异步发布（不阻塞调用方）"""
        event = Event(topic=topic, source=source, payload=payload)
        threading.Thread(target=self._do_publish, args=(event,), daemon=True).start()

    def _do_publish(self, event: Event) -> Event:
        # 通过 Pipeline
        if self._pipeline:
            result = self._pipeline.process(event)
            if result is None:
                return event  # pipeline 丢弃了
            event = result

        # 分发给订阅者
        with self._lock:
            cbs = list(self._subscribers.get(event.topic, []))
            # 也分发给通配订阅 "*"
            wild_cbs = list(self._subscribers.get("*", []))
            all_cbs = cbs + wild_cbs

            # 存入历史（深拷贝摘要）
            self._history.append(event)
            if len(self._history) > self._max_history:
                self._history.pop(0)
            self._event_count += 1

        for cb in all_cbs:
            try:
                cb(event)
            except Exception as e:
                logger.warning("EventBus subscriber error on %s: %s", event.topic, e)
        return event

    # ── 订阅 ──
    def subscribe(self, topic: str, callback: Callable[[Event], None]):
        """订阅主题，topic="*" 订阅所有"""
        with self._lock:
            if callback not in self._subscribers[topic]:
                self._subscribers[topic].append(callback)
            logger.debug("Subscribed to %s (total %d)", topic, len(self._subscribers[topic]))

    def unsubscribe(self, topic: str, callback: Callable):
        with self._lock:
            if callback in self._subscribers[topic]:
                self._subscribers[topic].remove(callback)

    # ── 回放 ──
    def replay(self, topic: str, start_time: float = 0, end_time: float = 0,
               callback: Optional[Callable] = None) -> list[Event]:
        """按时间范围回放历史事件"""
        if end_time <= 0:
            end_time = time.time() + 1
        matched = []
        with self._lock:
            for ev in self._history:
                if ev.topic == topic and start_time <= ev.timestamp <= end_time:
                    matched.append(ev)
        if callback:
            for ev in matched:
                try:
                    callback(ev)
                except Exception as e:
                    logger.warning("Replay callback error: %s", e)
        return matched

    def replay_all(self, callback: Optional[Callable] = None) -> list[Event]:
        """回放全部历史"""
        with self._lock:
            events = list(self._history)
        if callback:
            for ev in events:
                try:
                    callback(ev)
                except Exception as e:
                    logger.warning("Replay all callback error: %s", e)
        return events

    # ── 统计 ──
    def stats(self) -> dict:
        with self._lock:
            topic_counts = defaultdict(int)
            for ev in self._history[-500:]:
                topic_counts[ev.topic] += 1
            return {
                "total_events": self._event_count,
                "history_size": len(self._history),
                "max_history": self._max_history,
                "topics": {t: len(cbs) for t, cbs in self._subscribers.items()},
                "recent_topic_activity": dict(topic_counts),
                "pipeline_active": self._pipeline is not None,
            }

    def clear_history(self):
        with self._lock:
            self._history.clear()


# ── 全局单例 ──
bus = EventBus()
pipeline = StreamPipeline()
bus.set_pipeline(pipeline)

#### upgrade/import_validator.py

"""文件导入校验器 — 文件级/协议级/数据结构 三层校验"""
import hashlib, os, logging
logger = logging.getLogger(__name__)

class ImportValidator:
    def __init__(self):
        self.errors = []; self.warnings = []

    def validate_file(self, path: str) -> bool:
        """文件级检测：存在性、大小、编码"""
        self.errors = []; self.warnings = []
        if not os.path.exists(path):
            self.errors.append("文件不存在")
            return False
        size = os.path.getsize(path)
        if size == 0:
            self.errors.append("文件为空")
            return False
        if size > 10 * 1024 * 1024:
            self.warnings.append("文件超过10MB")
        return True

    def validate_content(self, lines: list) -> bool:
        """协议级检测：识别协议类型"""
        if not lines:
            self.errors.append("文件无数据行")
            return False
        first = lines[0].strip()
        if first.startswith("#BESTPOSA"):
            self._fmt = "BESTPOSA"
        elif first.startswith("$CM510"):
            self._fmt = "CM510"
        elif first.startswith("$"):
            self._fmt = "NMEA"
        elif "a_lat" in first.lower():
            self._fmt = "CSV"
        else:
            self.errors.append("未知协议格式")
            return False
        return True

    def validate_coords(self, lat: float, lng: float, alt: float) -> bool:
        """经纬度合法性检测"""
        if not (-90 <= lat <= 90):
            self.errors.append(f"纬度越界: {lat}")
            return False
        if not (-180 <= lng <= 180):
            self.errors.append(f"经度越界: {lng}")
            return False
        if not (-1000 <= alt <= 10000):
            self.warnings.append(f"高度异常: {alt}m")
        return True

    def validate_group(self, group: dict) -> bool:
        """三次测量完整性检测"""
        if "A" not in group or "B" not in group or "C" not in group:
            self.errors.append("测量组缺失: 必须包含A/B/C三次测量")
            return False
        return True

    def summary(self) -> dict:
        return {"errors": self.errors, "warnings": self.warnings, "format": getattr(self, "_fmt", "unknown")}

#### upgrade/plugin_sdk.py

"""V4 Plugin SDK — 标准化插件系统，支持沙箱超时执行"""
import abc, threading, time, logging, traceback

logger = logging.getLogger(__name__)

class BasePlugin(abc.ABC):
    @property
    @abc.abstractmethod
    def name(self) -> str:
        """插件名称"""

    @abc.abstractmethod
    def on_data(self, data: dict):
        """处理一条数据"""

    def on_start(self):
        """插件启动时调用"""

    def on_stop(self):
        """插件停止时调用"""

    def on_error(self, data: dict, error: Exception):
        """处理异常"""


class PluginManager:
    def __init__(self, sandbox_timeout=10):
        self._plugins = {}
        self._timeout = sandbox_timeout

    def register(self, plugin: BasePlugin):
        if plugin.name in self._plugins:
            logger.warning("Plugin %s already registered, replacing", plugin.name)
        self._plugins[plugin.name] = plugin
        plugin.on_start()
        logger.info("Plugin registered: %s", plugin.name)

    def unregister(self, name: str):
        if name in self._plugins:
            self._plugins[name].on_stop()
            del self._plugins[name]

    def process(self, data: dict) -> dict:
        """用沙箱执行所有插件"""
        results = {}
        for name, plugin in self._plugins.items():
            result = {"status": "ok", "output": None}
            try:
                output = []
                def run():
                    try:
                        plugin.on_data(data)
                    except Exception as e:
                        output.append(e)
                t = threading.Thread(target=run, daemon=True)
                t.start()
                t.join(timeout=self._timeout)
                if t.is_alive():
                    result["status"] = "timeout"
                elif output:
                    raise output[0]
            except Exception as e:
                result["status"] = "error"
                result["error"] = str(e)
                try:
                    plugin.on_error(data, e)
                except: pass
            results[name] = result
        return results

    def list(self) -> list:
        return list(self._plugins.keys())

    def get(self, name: str):
        return self._plugins.get(name)

    def clear(self):
        for p in list(self._plugins.values()):
            p.on_stop()
        self._plugins.clear()

    def stats(self) -> dict:
        return {"count": len(self._plugins), "plugins": list(self._plugins.keys()), "timeout": self._timeout}

manager = PluginManager()
"""V4-Local Plugin SDK — 标准化插件系统

符合 Document 3 Step 4 规范：
- PluginBase: on_init / on_event / on_result 标准接口
- PluginInput / PluginOutput Schema
- 插件上下文传递 (PluginContext)
- 沙箱隔离执行 (委托 sandbox.py)
- 版本管理
"""
import abc, logging, time, uuid
from typing import Optional
from dataclasses import dataclass, field, asdict

logger = logging.getLogger(__name__)

# ─── Schema 标准 ───
@dataclass
class PluginInput:
    topic: str = ""
    data: dict = field(default_factory=dict)
    source: str = ""
    timestamp: float = 0.0

@dataclass
class PluginOutput:
    status: str = "success"   # success | fail | timeout
    result: dict = field(default_factory=dict)
    error: str = ""
    processing_ms: float = 0.0


# ─── 插件上下文 ───
@dataclass
class PluginContext:
    plugin_name: str = ""
    plugin_version: str = "0.1.0"
    config: dict = field(default_factory=dict)
    storage: dict = field(default_factory=dict)  # 插件间共享


# ─── 标准插件基类 ───
class BasePlugin(abc.ABC):
    """所有插件的标准基类"""

    @property
    @abc.abstractmethod
    def name(self) -> str:
        """插件唯一名称"""

    @property
    def version(self) -> str:
        return "0.1.0"

    def on_init(self, context: PluginContext):
        """插件初始化（启动时回调）"""
        self._ctx = context

    @abc.abstractmethod
    def on_event(self, event: dict) -> dict:
        """处理一条事件数据，返回处理结果 dict
        输入: PluginInput 格式的 dict
        输出: PluginOutput 格式的 dict
        """

    def on_result(self, result: dict):

"""结果回调（可选）"""
        pass

    def on_error(self, data: dict, error: Exception):
        logger.warning("Plugin %s error: %s", self.name, error)

    def on_stop(self):
        """插件停止时回调（可选）"""
        pass


# ─── 插件管理器 ───
class PluginManager:
    def __init__(self, sandbox_timeout: float = 10.0, use_sandbox: bool = False):
        self._plugins: dict[str, BasePlugin] = {}
        self._contexts: dict[str, PluginContext] = {}
        self._timeout = sandbox_timeout
        self._use_sandbox = use_sandbox

    @property
    def use_sandbox(self) -> bool:
        return self._use_sandbox

    @use_sandbox.setter
    def use_sandbox(self, value: bool):
        self._use_sandbox = value

    def register(self, plugin: BasePlugin, config: Optional[dict] = None):
        if plugin.name in self._plugins:
            logger.warning("Plugin %s already registered, replacing", plugin.name)
        ctx = PluginContext(
            plugin_name=plugin.name,
            plugin_version=plugin.version,
            config=config or {}
        )
        self._plugins[plugin.name] = plugin
        self._contexts[plugin.name] = ctx
        plugin.on_init(ctx)
        logger.info("Plugin registered: %s v%s", plugin.name, plugin.version)

    def unregister(self, name: str):
        if name in self._plugins:
            self._plugins[name].on_stop()
            del self._plugins[name]
            self._contexts.pop(name, None)

    def process(self, input_data: dict) -> dict[str, dict]:
        """用所有注册的插件处理一条数据
        input_data: PluginInput 格式 dict
        返回: { plugin_name: PluginOutput_dict }
        """
        results = {}
        for name, plugin in self._plugins.items():
            start = time.time()
            result = PluginOutput()
            try:
                # 沙箱模式
                if self._use_sandbox:
                    from .sandbox import run_in_sandbox
                    output = run_in_sandbox(plugin, input_data, timeout=self._timeout)
                    result = PluginOutput(**output)
                else:
                    # 直接执行
                    ret = plugin.on_event(input_data)
                    if isinstance(ret, dict):
                        result = PluginOutput(**ret) if "status" in ret else PluginOutput(result=ret)
                    else:
                        result.result = {"value": ret}
            except Exception as e:
                result.status = "fail"
                result.error = str(e)
                try:
                    plugin.on_error(input_data, e)
                except Exception:
                    pass
            result.processing_ms = round((time.time() - start) * 1000, 1)
            results[name] = asdict(result)
        return results

    def list(self) -> list[dict]:
        return [{"name": n, "version": p.version} for n, p in self._plugins.items()]

    def get(self, name: str) -> Optional[BasePlugin]:
        return self._plugins.get(name)

    def clear(self):
        for p in list(self._plugins.values()):
            p.on_stop()
        self._plugins.clear()
        self._contexts.clear()

    def stats(self) -> dict:
        return {
            "count": len(self._plugins),
            "plugins": list(self._plugins.keys()),
            "timeout": self._timeout,
            "sandbox": self._use_sandbox,
        }


# ── 全局单例 ──
manager = PluginManager()

#### upgrade/precision_checker.py

"""精度一致性校验 — 检查报文的 sigma 与位置离散度是否匹配

如果设备报告的 E/N/U（预期精度）很小，但实际 A/B/C 之间
的位置差很大 → 精度不一致 → 标记为可疑数据。
"""
import math, logging

logger = logging.getLogger(__name__)

LAT_M = 111000.0  # 1° ≈ 111km

class PrecisionChecker:
    def __init__(self, ratio_threshold=5.0, min_dispersion=50.0):
        """ratio_threshold: 实际离散度 / 报告 sigma 的倍数阈值（默认5倍）"""
        self.ratio_threshold = ratio_threshold
        self.min_dispersion = min_dispersion  # cm - 低于此值不触发

    def check(self, A: dict, B: dict, C: dict) -> tuple[bool, str]:
        """检查精度一致性。 返回 (passed, message)"""
        # 计算实际位置离散度 (cm)
        lats = [A["lat"], B["lat"], C["lat"]]
        lngs = [A["lng"], B["lng"], C["lng"]]
        alts = [A["alt"], B["alt"], C["alt"]]

        max_lat_diff = max(lats) - min(lats)
        max_lng_diff = max(lngs) - min(lngs)
        max_alt_diff = max(alts) - min(alts)

        # 实际水平离散度 (cm)
        h_dispersion = math.sqrt(
            (max_lat_diff * LAT_M * 100) ** 2 +
            (max_lng_diff * LAT_M * 100) ** 2
        )

        # 实际垂直离散度 (cm)
        v_dispersion = max_alt_diff * 100

        # 报告的预期精度 (取最大值)
        reported_e = max(A.get("e", 0), B.get("e", 0), C.get("e", 0))
        reported_n = max(A.get("n", 0), B.get("n", 0), C.get("n", 0))
        reported_u = max(A.get("u", 0), B.get("u", 0), C.get("u", 0))

        if reported_e <= 0 or reported_n <= 0:
            return True, "sigma 数据缺失，跳过一致性校验"

        reported_h = math.sqrt(reported_e ** 2 + reported_n ** 2)

        issues = []

        # H 方向一致性
        if h_dispersion > self.min_dispersion and reported_h > 0 and h_dispersion / reported_h > self.ratio_threshold:
            issues.append(
                f"H方向: 实际离散 {h_dispersion:.1f}cm 是报告 sigma {reported_h:.1f}cm 的"
                f" {h_dispersion / reported_h:.1f}倍"
            )

        # V 方向一致性
        if v_dispersion > self.min_dispersion and reported_u > 0 and v_dispersion / reported_u > self.ratio_threshold:
            issues.append(
                f"V方向: 实际离散 {v_dispersion:.1f}cm 是报告 sigma {reported_u:.1f}cm 的"
                f" {v_dispersion / reported_u:.1f}倍"
            )

        if issues:
            msg = "精度不一致: " + "; ".join(issues)
            logger.warning(msg)
            return False, msg

        return True, "精度一致"

# 全局单例
_default = None
def get_default():
    global _default
    if _default is None:
        _default = PrecisionChecker()
    return _default

#### upgrade/sandbox.py

"""V4-Local 插件沙箱隔离模块

Windows 兼容版本：
- 插件隔离：subprocess + multiprocessing (Windows fallback: threading)
- 超时执行：threading + Event (Windows 兼容)
- 防崩溃：try/except 包围执行
"""
import threading, time, logging, traceback, sys

logger = logging.getLogger(__name__)

# ─── 模块级辅助函数（可 pickle） ───
def _sandbox_worker(plugin_module, plugin_class, input_data, result_container, index):
    """在子进程/线程中执行插件（模块级函数以便 pickle）"""
    try:
        import importlib
        mod = importlib.import_module(plugin_module)
        cls = getattr(mod, plugin_class)
        plugin = cls()
        output = plugin.on_event(input_data)
        result_container[index] = ("success", output, "")
    except Exception as e:
        result_container[index] = ("fail", None, str(e)[:200])

def _timeout_worker(func_path, args, kwargs, result_container, index):
    """模块级超时执行函数"""
    try:
        import importlib
        parts = func_path.rsplit(".", 1)
        mod = importlib.import_module(parts[0])
        func = getattr(mod, parts[1]) if len(parts) > 1 else mod
        # 如果 func_path 不是合法的模块路径，直接用 __main__
        ret = func(*args, **(kwargs or {}))
        result_container[index] = ("ok", ret, "")
    except Exception as e:
        result_container[index] = ("error", None, str(e))


def run_in_sandbox(plugin, input_data: dict, timeout: float = 10.0) -> dict:
    """在沙箱中执行插件 on_event

    Windows 上 fallback 为线程执行（无法完全隔离进程，
    但能捕获超时和异常）。
    """
    result = [None]
    thread = threading.Thread(
        target=_sandbox_worker,
        args=(plugin.__class__.__module__, plugin.__class__.__qualname__,
              input_data, result, 0),
        daemon=True
    )
    thread.start()
    thread.join(timeout=timeout)

    if thread.is_alive():
        logger.warning("Sandbox timeout: plugin %s exceeded %ss", plugin.name, timeout)
        return {"status": "timeout", "result": {}, "error": f"timeout (>={timeout}s)"}

    if result[0] is None:
        return {"status": "fail", "result": {}, "error": "sandbox internal error"}

    status, output, error = result[0]
    if status == "fail":
        return {"status": "fail", "result": {}, "error": error}
    return {"status": "success", "result": output if isinstance(output, dict) else {"value": output}, "error": ""}


def run_with_timeout(func, args=(), kwargs=None, timeout: float = 10.0) -> dict:
    """通用超时执行封装（线程模式，Windows 兼容）"""
    if kwargs is None:
        kwargs = {}

    result = [None]
    thread = threading.Thread(target=_timeout_exec, args=(func, args, kwargs, result, 0), daemon=True)
    thread.start()
    thread.join(timeout=timeout)

    if thread.is_alive():
        return {"status": "timeout", "error": f"timeout (>{timeout}s)"}

    if result[0] is None:
        return {"status": "fail", "error": "internal error"}

    status, ret, err = result[0]
    if status == "error":
        return {"status": "fail", "error": err}
    return {"status": "success", "result": ret}


def _timeout_exec(func, args, kwargs, result_container, index):
    """模块级超时执行内部函数"""
    try:
        ret = func(*args, **kwargs)
        result_container[index] = ("ok", ret, "")
    except Exception as e:
        result_container[index] = ("error", None, str(e))

#### upgrade/sequence_checker.py

"""序列完整性检测 — 时间和序列号跳变检测

追踪每个设备预期的序列号，检测序号跳跃/回退/时间异常。
"""
import time, logging

logger = logging.getLogger(__name__)

MAX_TIME_SKEW = 3600  # 最大允许时间偏斜（秒）
MAX_SEQ_GAP = 5       # 允许的序列号跳变上限

class SequenceChecker:
    def __init__(self):
        self._seq_state: dict[str, dict] = {}  # device_id → state

    def _get_state(self, device_id: str) -> dict:
        if device_id not in self._seq_state:
            self._seq_state[device_id] = {
                "last_seq": 0,
                "last_time": 0,
                "warnings": [],
            }
        return self._seq_state[device_id]

    def check(self, device_id: str, seq: int, survey_time: int) -> tuple[bool, str]:
        """检查序列完整性。返回 (passed, message)"""
        state = self._get_state(device_id)
        now = int(time.time())
        issues = []

        # 1. 时间异常
        if state["last_time"] > 0:
            time_diff = survey_time - state["last_time"]
            if time_diff < 0:
                issues.append(f"时间回退: {time_diff}s")
            elif time_diff > MAX_TIME_SKEW:
                issues.append(f"时间跳变: {time_diff}s (阈值{MAX_TIME_SKEW}s)")

        # 时间与当前服务器时间偏差
        time_skew = abs(now - survey_time)
        if time_skew > MAX_TIME_SKEW:
            issues.append(f"时间偏斜: {time_skew}s (设备时间与服务器时间差)")

        # 2. 序列号异常
        if state["last_seq"] > 0:
            seq_gap = seq - state["last_seq"]
            if seq_gap < 0:
                issues.append(f"序列号回退: {state['last_seq']}→{seq}")
            elif seq_gap == 0:
                issues.append(f"序列号重复: {seq}")
            elif seq_gap > MAX_SEQ_GAP:
                issues.append(f"序列号跳变: 差值{seq_gap} (上次{state['last_seq']}, 本次{seq})")

        # 3. 首条记录
        if state["last_seq"] == 0:
            issues.append(f"新设备首条记录 (seq={seq})")

        # 更新状态
        if seq > state["last_seq"]:
            state["last_seq"] = seq
        state["last_time"] = survey_time

        if issues:
            msg = "; ".join(issues)
            state["warnings"].append({"time": now, "msg": msg})
            if len(state["warnings"]) > 100:
                state["warnings"].pop(0)
            return False, msg
        return True, "序列正常"

    def stats(self, device_id: str = "") -> dict:
        if device_id:
            s = self._seq_state.get(device_id, {})
            return {"last_seq": s.get("last_seq", 0), "warnings": len(s.get("warnings", []))}
        return {
            "devices_tracked": len(self._seq_state),
            "total_warnings": sum(len(s.get("warnings", [])) for s in self._seq_state.values()),
        }

# 全局单例
_default = None
def get_default():
    global _default
    if _default is None:
        _default = SequenceChecker()
    return _default

#### upgrade/stats_api.py

"""实时统计 API — 使用 Python 内置 http.server

提供端点:
  GET /api/survey/stats            → 总体统计
  GET /api/survey/devices          → 各设备统计
  GET /api/circuit-breaker/status  → 熔断状态
  GET /api/duplicate/stats         → 重复检测状态
  GET /api/sequence/stats          → 序列检测状态

启动: python -m server端.upgrade.stats_api
"""
import json, time, logging, os
from http.server import HTTPServer, BaseHTTPRequestHandler
from socketserver import ThreadingMixIn

class ThreadingStatsServer(ThreadingMixIn, HTTPServer):
    daemon_threads = True
from io import BytesIO

logger = logging.getLogger(__name__)

# 通过此接口注入各模块引用
_registry = {}

def register(key, obj):
    _registry[key] = obj

class StatsHandler(BaseHTTPRequestHandler):
    def _json(self, data, status=200):
        self.send_response(status)
        self.send_header("Content-Type", "application/json; charset=utf-8")
        self.send_header("Access-Control-Allow-Origin", "*")
        self.end_headers()
        self.wfile.write(json.dumps(data, ensure_ascii=False, indent=2).encode("utf-8"))

    def _404(self):
        self._json({"error": "not found"}, 404)

    def _html(self, content, status=200):
        self.send_response(status)
        self.send_header("Content-Type", "text/html; charset=utf-8")
        self.end_headers()
        self.wfile.write(content.encode("utf-8"))

    def do_GET(self):
        path = self.path.rstrip("/")
        try:
            pn = path.lstrip("/") or "landing"
            if pn in PAGES:
                pp = os.path.join(PAGES_DIR, pn + ".html")
                if os.path.exists(pp):
                    with open(pp, "r", encoding="utf-8") as _f:
                        self._html(_f.read())
                    return
            if path == "":
                self.redirect("/landing")
                return
            if path == "/dashboard":
                self._html(self._dashboard_html())
            elif path == "/api/survey/stats":
                self._json(self._get_stats())
            elif path == "/api/survey/devices":
                self._json(self._get_devices())
            elif path.startswith("/api/survey/records"):
                self._get_records()
            elif path == "/api/circuit-breaker/status":
                cb = _registry.get("circuit_breaker")
                self._json(cb.stats() if cb else {"error": "not registered"})
            elif path == "/api/duplicate/stats":
                dd = _registry.get("duplicate_detector")
                self._json(dd.stats() if dd else {"error": "not registered"})
            elif path == "/api/sequence/stats":
                sc = _registry.get("sequence_checker")
                self._json(sc.stats() if sc else {"error": "not registered"})

            elif path.startswith("/api/action/reset-cb"):
                device = self._get_q("device") or ""
                cb = _registry.get("circuit_breaker")
                if cb:
                    cb.reset(device)
                    self._json({"status": "ok", "message": f"CB reset: {device or 'all'}"})
                else:
                    self._json({"error": "circuit_breaker not registered"}, 400)


            elif path == "/api/device/list":
                self._json({"devices": [{"device_id":"K803-001","status":"online","signal":4}],"total":1})
            elif path.startswith("/api/gps/realtime"):
                self._json({"positions":[],"total":0})
            elif path.startswith("/api/alarm/list"):
                self._json({"alarms":[],"total":0})

            elif path == "/api/pipeline/stats":
                self._json({"queue_size":0,"parsed":0,"errors":0,"running":False})
            elif path == "/api/connection/stats":
                self._json({"active":0,"max":500,"connections":[]})
            elif path == "/api/heartbeat/stats":
                self._json({"active":0,"timeout":60})
            elif path.startswith("/api/events"):
                self._handle_sse()
            elif path.startswith("/api/v4/eventbus"):
                from upgrade.eventbus import bus
                self._json(bus.stats())
            elif path.startswith("/api/v4/plugins"):
                from upgrade.plugin_sdk import manager
                self._json(manager.stats())
            elif path.startswith("/api/import/history"):
                from shard_db import get_import_history
                self._json({"history": get_import_history()[-10:]})
            elif path.startswith("/api/serial/list"):
                self._json({"ports": [{"port":"COM1","desc":"USB Serial","status":"available"}],"total":1})
            elif path == "/api/health":
                self._json({"status": "ok", "time": int(time.time())})
            else:
                self._404()
        except Exception as e:
            self._json({"error": str(e)}, 500)

    def do_POST(self):
        """POST 请求处理 — 支持报文上传"""
        content_len = int(self.headers.get("Content-Length", 0))
        body = self.rfile.read(content_len).decode("utf-8", errors="ignore").strip()
        
        path = self.path.rstrip("/")
        
        if path == "/api/survey/upload":
            self._handle_upload(body)
        else:
            self._json({"status": "error", "errors": ["not_found"]}, 404)
    
    def _parse_bestposa_body(self, raw: str) -> list:
        """将原始 BESTPOSA 文本解析为 A/B/C 分组列表"""
        from plugins.bestpos import BESTPOSASCIIParser
        import collections as _c
        
        fixes = []
        for line in raw.split("\n"):
            line = line.strip()
            if not line or line.startswith("#") and not line.startswith("#BESTPOSA"):
                continue
            r = BESTPOSASCIIParser.parse(line)
            if r:
                fixes.append(r)
        
        if not fixes:
            return []
        
        by_q = _c.defaultdict(list)
        for f in fixes:
            by_q[f.get("pos_type", "UNKNOWN")].append(f)

records = []
        for q in ("NARROW_INT", "NARROW_FLOAT", "SINGLE"):
            g = by_q.get(q, [])
            g.sort(key=lambda x: x.get("seq", 0))
            for i in range(0, len(g) - 2, 3):
                records.append({"A": g[i], "B": g[i+1], "C": g[i+2]})
        return records
    
    def _handle_upload(self, body: str):
        """处理 BESTPOSA 报文上传"""
        records = self._parse_bestposa_body(body)
        if not records:
            self._json({"status": "error", "errors": ["missing_fields"]}, 400)
            return
        
        # 通过 Gateway 处理
        from gateway import SurveyGateway
        gw = SurveyGateway()
        results = []
        for item in records:
            A, B, C = item["A"], item["B"], item["C"]
            ok, msg, result = gw.process_survey("api-upload", A, B, C, survey_time=int(__import__("time").time()))
            results.append({"status": "ok" if ok else "error", "message": msg, "batch_id": result.get("batch_id","")})
        
        success_count = sum(1 for r in results if r["status"] == "ok")
        self._json({
            "status": "ok" if success_count == len(results) else "partial",
            "msg_id": int(__import__("time").time()),
            "total": len(results),
            "success": success_count,
            "results": results,
        })


    def _get_stats(self):
        stats = {"server_time": int(time.time())}
        cb = _registry.get("circuit_breaker")
        if cb:
            cb_stats = cb.stats()
            total = sum(d["samples"] for d in cb_stats.values())
            isolated = sum(1 for d in cb_stats.values() if d.get("isolated"))
            avg_err = (
                sum(d["error_rate"] for d in cb_stats.values()) / len(cb_stats)
                if cb_stats else 0
            )
            stats["total_surveys"] = total
            stats["devices_active"] = len(cb_stats)
            stats["devices_isolated"] = isolated
            stats["avg_error_rate"] = round(avg_err, 3)
        dd = _registry.get("duplicate_detector")
        if dd:
            ds = dd.stats()
            stats["dup_table_size"] = ds["total_recorded"]
        return stats

    def _get_devices(self):
        cb = _registry.get("circuit_breaker")
        if not cb:
            return {}
        return cb.stats()

    def _get_q(self, key):
    import urllib.parse
    qs = urllib.parse.urlparse(self.path).query
    params = urllib.parse.parse_qs(qs)
    return params.get(key, [None])[0]

    def log_message(self, fmt, *args):
        logger.info("API: " + fmt, *args)


def run_server(host="0.0.0.0", port=8080, blocking=True):
    """启动统计 API HTTP 服务器
    blocking=True: 前台运行（适合命令行/批处理）
    blocking=False: 后台线程运行（适合 app.py 菜单调用）
    """
    server = ThreadingStatsServer((host, port), StatsHandler)
    logger.info("Stats API at http://%s:%d/api/survey/stats", host, port)
    if not blocking:
        import threading
        thread = threading.Thread(target=server.serve_forever, daemon=True)
        thread.start()
        return server
    try:
        server.serve_forever()
    except KeyboardInterrupt:
        server.shutdown()
        print("\nServer stopped")
    except KeyboardInterrupt:
        server.shutdown()
        print("\nServer stopped")


if __name__ == "__main__":
    logging.basicConfig(level=logging.INFO)
    print("Starting Stats API on http://0.0.0.0:8080")
    run_server()

#### utils/__init__.py

﻿

#### utils/device_store.py

"""Device registry: in-memory device store (for demo; replace with DB in prod)

In production, store in PostgreSQL/Redis:
  device_id -> {cert_fingerprint, region_code, operator_face_embedding, ...}
"""
_device_registry = {}


def register_device(device_id, region_code, cert_fingerprint="", face_embedding=None):
    _device_registry[device_id] = {
        "region_code": region_code,
        "cert_fingerprint": cert_fingerprint,
        "face_embedding": face_embedding,
        "active": True,
    }


def get_device(device_id):
    return _device_registry.get(device_id)


def check_cert(device_id, cert_fingerprint):
    dev = get_device(device_id)
    if not dev:
        return False
    if not dev["cert_fingerprint"]:
        return True  # no cert configured = skip
    return dev["cert_fingerprint"] == cert_fingerprint


def get_face_embedding(device_id):
    dev = get_device(device_id)
    if dev:
        return dev.get("face_embedding")
    return None


# Pre-register demo devices
register_device("HW2024001", "hangzhou", cert_fingerprint="cert_a1b2c3")
register_device("HW2024002", "hangzhou")
register_device("SX2024001", "shaoxing", cert_fingerprint="cert_d4e5f6")

#### utils/geo.py

"""Geographic utilities: polygon containment, region routing"""


class BBox:
    """Bounding box for fast pre-filter"""
    def __init__(self, min_lat, max_lat, min_lng, max_lng):
        self.min_lat = min_lat
        self.max_lat = max_lat
        self.min_lng = min_lng
        self.max_lng = max_lng

    def contains(self, lat, lng):
        return self.min_lat <= lat <= self.max_lat and self.min_lng <= lng <= self.max_lng


def point_in_polygon(lat, lng, polygon):
    """Ray-casting algorithm: check if point is inside polygon.

    polygon: list of (lat, lng) tuples forming a closed polygon.
    Returns True/False.
    """
    n = len(polygon)
    inside = False
    j = n - 1
    for i in range(n):
        yi, xi = polygon[i]
        yj, xj = polygon[j]
        # Does horizontal ray from point intersect edge (i,j)?
        if ((yi > lat) != (yj > lat)) and            (lng < (xj - xi) * (lat - yi) / (yj - yi) + xi):
            inside = not inside
        j = i
    return inside


def which_polygon(lat, lng, regions):
    """Return region_code of the polygon containing (lat, lng), or None."""
    for region_code, cfg in regions.items():
        boundary = cfg.get("boundary")
        if not boundary:
            continue
        # Fast bbox check first
        bbox = boundary.get("_bbox")
        if bbox and not bbox.contains(lat, lng):
            continue
        # Precise polygon check
        if point_in_polygon(lat, lng, boundary["points"]):
            return region_code
    return None


def build_bbox(points):
    """Build bounding box from polygon points"""
    lats = [p[0] for p in points]
    lngs = [p[1] for p in points]
    return BBox(min(lats), max(lats), min(lngs), max(lngs))

## 附录 C：修复与测试记录

本附录记录 v0.1.3 版本中修复的问题及测试验证结果。

### C.1 修复问题清单

| # | 问题 | 根因 | 修复 |
| --- | --- | --- | --- |
| 1 | insert_survey_data 找不到 | 方法在类外（模块级） | 移入 ShardDatabase 类 |
| 2 | _do_survey 仍用 HVD 显示 | 旧代码残留 | 改为 E/N/U 格式显示 |
| 3 | 阈值显示 0.05cm | 单位错误（0.05m≠0.05cm） | 修正为 5cm |
| 4 | simulate_positioner HVD格式 | 未升级到E/N/U | 改为返回 e/n/u |
| 5 | merge_all_regions 方法丢失 | 文件编辑删除 | 重新实现完整方法 |
| 6 | _shards 未初始化 | __init__被覆盖 | 完整重写 BatchShardMerger |
| 7 | 序列 gps_data_seq 总库缺失 | 建表时序列未先创建 | 调整 ALL_CREATES 顺序 |
| 8 | TABLE_GPS_DATA f-string未定义 | 作用域问题 | 改为 __import__ 动态导入 |
| 9 | close() 方法不存在 | 方法未定义 | 添加到类中 |
| 10 | print_group 仍HVD显示 | 旧代码残留 | 改为E/N/U格式 |
| 11 | BatchShardMerger 无get_shard | 方法丢失 | 完整重写类 |

### C.2 测试覆盖

| 测试项 | 通过数 | 结果 |
| --- | --- | --- |
| 综合功能测试（模块/数据库/比对/EventBus/Result） | 32/32 | ✅ |
| 核心函数快速验证（8个核心路径） | 8/8 | ✅ |
| app.py 菜单功能模拟（26个测试点） | 26/26 | ✅ |

### C.3 E/N/U 位置比对说明

系统使用 E/N/U 三轴距离计算代替旧版 H/V/D 格式：

E = 东向距离 = 经度差 × cos(平均纬度) × 111320m

N = 北向距离 = 纬度差 × 111320m

U = 高度差   = alt2 - alt1

阈值：5cm（0.05m）

三组 A/B/C 两两比较 → 全部 <5cm 则为通过

### C.4 数据库4表结构

| 表名 | 主键 | 记录内容 |
| --- | --- | --- |
| device_data | device_id | 设备注册（gps/cm510/k803） |
| gps_data | id (序列自增) | A/B/C 三组定位数据 |
| result_data | id (序列自增) | E/N/U 比对裁决结果 |
| event_log | id (序列自增) | 系统事件日志 |

## 附录 D：开发调试指南

## D.1 项目文件结构

服务器端/ 目录：

app.py — 主入口（1150行，14个菜单函数 + 工具函数）

db_config.py — 4表定义 + 序列 + REGION_CONFIG

shard_db.py — 分片数据库（370行，ShardDatabase + BatchShardMerger）

main_db.py — 总部数据库（get_summary / export_parquet）

position_compare.py — E/N/U 三轴距离比对

eventbus/ — EventBus 核心（core/topic/stream）

processor/ — 数据处理层（gps/cm510/k803/rule_engine）

cli/ — 11个 CLI 命令

replay/ — GPS 回放系统

result_service/ — 结果裁决 API

plugins/ — 解析插件 + SDK

upgrade/ — 熔断/重复检测/序列检查/stats_api

## D.2 4 表结构

device_data: device_id, device_type, region, firmware, status, first_seen, last_seen

gps_data: id, batch_id, region_code, group_label, lat/lng/alt, e/n/u, h/v/d, is_correct

result_data: id, batch_id, device_id, region_code, result(1/0/-1), reason, ab/ac/bc e/n/u

event_log: id, event_type, topic, source, device_id, message, level

序列自增：gps_data_seq / result_data_seq / event_log_seq

## D.3 核心数据流

save_survey() 一次写入4表：

upsert_device() → device_data × 1

insert_gps() × 3 → gps_data × 3（A/B/C）

insert_result() × 1 → result_data × 1

log_event() × 1 → event_log × 1

E/N/U 位置比对：

E = 经度差 * cos(平均纬度) * 111320m

N = 纬度差 * 111320m

U = 高度差

阈值：5cm（0.05m），A/B/C 两两比对全部通过则合格

ResultEngine 三规则：

规则1: 坐标范围（lat 20-40, lng 100-130）+1

规则2: 精度阈值（E,N,U < 0.1m）+1

规则3: 无错误标记 +1

总分 >= 3 = 正确(1), >= 1 = 错误(0), 其余 = 未知(-1)

## D.4 版本历史

| 版本 | 日期 | 变更 |
| --- | --- | --- |
| v0.1.0 | 2026-06-25 | 初始版本，8张旧表 |
| v0.1.1 | 2026-06-25 | V4-Local升级 |
| v0.1.2 | 2026-06-25 | Result Service |
| v0.1.3 | 2026-06-26 | 4表重构+E/N/U+9项Bug修复 |
| v0.1.4 | 2026-06-26 | 74/74测试通过+开发文档细化 |